3GPP TSG-SA WG6 Meeting #63	S6-244211

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244xxx)

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on Application calling service API

Spec:	3GPP TS 23.392 v0.1.0

Agenda item:	9.12

Document for:	Approval

Contact:	wanghan28@huawei.com

1. Introduction

This pCR proposes to add the Application calling service exposure procedure.

2. Reason for Change

With the conclusion of FS_eMMTelAPP for KI#1, the corresponding normative work need to be implementd.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.392 v0.1.0.

* * * First Change * * * *

### 8.4.1	Application calling service API

#### 8.4.2.1	General

The MMTel Enabler Server exposes the Application calling service API to the Applications in order to support establishing a call between Application and DCMTSI client. The Application calling service API exposed by the MMTel Enabler Server relies on the IMS core network capabilities.

#### 8.4.2.2	Procedure

##### 8.4.2.2.1	Establishing an Application Call

Figure 8.4.2.2.1-1 illustrates the operation to establish an Application Call between the Application and the MMTel Enabler Server.

Pre-conditions:

1.	The Application is authorized to use Application calling service API provided by the MMTel Enabler Server.

2.	The MMTel Enabler Server is authorized to use the Session Management related API and the IMS Event exposure related API of IMS AS.

3.	The MMTel Enabler Server is authorized to use OMA AudioCall API.

Figure 8.4.2.2.1-1: Establishing a Application Call

1.	The Application sends an Application Call establishment request to the MMTel Enabler Server. The Application Call establishment request shall include the Originating ID, Terminating ID and Notification address. It may also include Media information, DC media information and Application Profile requested.

2.	Upon receiving the request, the MMTel Enabler Server can utilize the OMA OneAPI to establish a Call session with audio or video media as specified in OMA-TS-REST_NetAPI_AudioCall or request to establish an Application Call session with audio or video media via IMSAS directly. If DC media information is also included in step 1, the MMTel Enabler Server utilizes the DC update capability of the IMS core network to add DC media to the existing Application Call Session as specified in 3GPP TS 23.228 [x]. If only DC media information is included in step 1, the MMTel Enabler Server utilizes the standalone DC capability of the IMS core network as specified in 3GPP TS 23.228 [x] to establish an Application Call Session only include DC media.

3.	The MMTel Enabler Server sends an Application Call establishment response to the Application. The response includes the Result and Session ID, it may include a Failure Reason if the Result is failure.

##### 8.4.2.2.2	Updating media information to an Existing Application Call

Figure 8.4.2.2.2-1 illustrates the operation to update an existing Application Call between the Application and the MMTel Enabler Server.

Pre-conditions:

1.	The Application is authorized to use Application Calling service API provided by the MMTel Enabler Server.

2.	The MMTel Enabler Server is authorized to use the Session Management related API and the IMS Event exposure related API of IMS AS.

3.	An Application Call session between Application and DCMTSI client has established.

Figure 8.4.2.2.3-1: Update an Application Call

1.	The Application sends an Application Call update request to the MMTel Enabler Server. The Application Call update request shall include the Session ID and media information.

2.	Upon receiving the request, the MMTel Enabler Server utilizes the OMA OneAPI to add/remove audio/video media from the existing Application Call Session only includes data channel media, and can utilize the DC update capability of the IMS core network to add/remove data channel media from the existing Application Call Session only include audio/video media as specified in 3GPP TS 23.228 [x].

3.	The MMTel Enabler Server sends an Application Call update response to the Application. The response includes the Result and Session ID, it may include a Failure Reason if the Result is failure.

#### 8.4.2.3	Information flows

##### 8.4.2.3.1	Application Call establishment request

Table 8.4.2.3.1-1: Application Call establishment request

| Information element | Status | Description |
| --- | --- | --- |
| Originating ID | M | The identifier of the caller. |
| Terminating ID | M | The identifier of the callee. |
| Media information | O | Identifier of one or more media type(s) which is expected to be used in the call, i.e. the media type(s) to be applied to the participants in the call session. In this IE, the media type(s) includes a) audio; b) video; c) audio and video. The detailed media information, e.g. the information included in the SDP of mmtel call session is negotiated by the underlying network. If the parameter is absent, the media type(s) and detailed media information are all negotiated by the underlying network. |
| DC media information | O | Identifier of whether DC media is expected to be used in this call |
| Application Profile requested | O | The DC application profile expected to be used in this call |
| Notification address | M | The address where the call related notification, e.g. whether the call between UE A and UE B is established successfully or not, is sent to. |

##### 8.4.2.3.2	Application Call establishment response

Table 8.4.2.3.2-1: Application Call establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates the Application Call establishment request was successful |
| > Session ID | M | The identifier of the call session. This ID is as same as the Session ID in IMS. |
| Failure response | O | Indicates the Application Call establishment request was failed |
| > Cause | O | Indicates the cause of request failure |

##### 8.4.2.3.3	Application Call update request

Table 8.4.2.3.3-1: Application Call update request

| Information element | Status | Description |
| --- | --- | --- |
| Session ID | M | The identifier of the call session. This ID is as same as the Session ID in IMS. |
| Media information | O | Identifier of one or more media type(s) which is expected to be used in the call, i.e. the media type(s) to be applied to the participants in the call session. In this IE, the media type(s) includes a) audio; b) video; c) audio and video. The detailed media information, e.g. the information included in the SDP of mmtel call session is negotiated by the underlying network. If the parameter is absent, the media type(s) and detailed media information are all negotiated by the underlying network. |
| DC media information | O | Identifier of whether DC media is expected to be used in this call |
| Application Profile requested | O | The DC application profile expected to be used in this call |

##### 8.4.2.3.4	Application Call update response

Table 8.4.2.3.4-1: Application Call update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates the Application Call update request was successful |
| > Session ID | M | The identifier of the call session. This ID is as same as the Session ID in IMS. |
| Failure response | O | Indicates the Application Call update request was failed |
| > Cause | O | Indicates the cause of request failure |

* * * End of Changes * * * *