3GPP TSG-SA6 Meeting #63	S6-244237

# Hyderabad, India, 14th Oct 2024 - 18th Oct 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.436 | CR | 0043 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Architecture for supporting interactions with AIMLE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2024-10-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | One of the use cases related to AIMLE, is the utilization of AIMLE services by ADAES for supporting ML-enabled analytics. This contribution provides the architecture enhancements in ADAES to capture this use case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause is added based on TR 23.700-82 on the interaction between ADAES and AIMLE server, and additions in clause 5.4.3 are introduced for clarifying the support of AIMLE in ADAE analytics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The relation between ADAES and AIMLE server and the dependency between these servers will not be clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 5.2.4 (new), clause 5.2.9 (new), clause 2, clause 5.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 5.2.4	Functional Architecture for supporting interactions with SEAL AIMLE

Figure 5.2.4-1 illustrates the architecture representation including AIMLE (as specified in 3GPP TS 23.482 [19]) for supporting ML-enabled analytics in ADAES. In this representation, the AIML support capabilities serve ADAES to enhance its analytics services. Based on the VAL request to provide ML-enabled analytics, ADAES may consume AIMLE services (e.g., for ML model training for a given analytics ID) to derive application layer data analytics.

Figure 5.2.4-1: Architecture representation for supporting AIML-enabled ADAE analytics.

For the interaction between AIMLE server and ADAES, AIML-X is introduced to support consuming AIMLE services for deriving ADAE analytics (e.g. VAL server performance analytics).

The ML repository is specified in 3GPP TS 23.436 [19] as a repository for the ML model and the ML-related information (such as ML participants). This repository may be utilized by ADAES via AIMLE server (via AIML-R) for fetching ML-related information (trained ML model, ML training entity list) which is used for a given ADAE analytics event.

Further details on the AIMLE capabilities and architecture, where the ADAES is a consumer of the AIMLE services, are specified in 3GPP TS 23.436 [19].

* * * Next Change * * * *

### 5.5.9	AIML-X

The interactions between the SEAL AIMLE server and ADAES for supporting ML-enabled analytics is generically referred to as AIML-X reference point. AIML-X is an instance of SEAL-X as described 3GPP TS 23.434 [2].

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[3]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture"

[4]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[5]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[6]	3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service".

[7]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[8]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[9]	3GPP TS 23.501: “System architecture for the 5G System”.

[10]	GSMA NG.116 - Generic Network Slice Template.

[11]	3GPP TS 22.261: “Service requirements for the 5G system”.

[12]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS)".

[13]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications".

[14]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[15]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[16]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[17]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[18]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[19]	3GPP TS 23.482: “Functional architecture and information flows for AIML Enablement Service”.

* * * Next Change * * * *

### 5.4.3	Application Data Analytics Enablement server

The application data analytics enablement server functional entity provides application layer analytics to support the VAL applications. The application data analytics enablement server acts as CAPIF's API exposing function as specified in 3GPP TS 23.222 [8]. The application data analytics enablement server also supports interactions with the corresponding application data analytics enablement server in distributed SEAL deployments. The ADAE server also interacts with 3GPP core network over N33 or N6 interface to subscribe to changes in configuration or other application server specific events. The ADAE server also acts as a co-ordinating entity to collect data from different sources and perform necessary actions to provide required analytics.

The ADAE server provides following server side functionalities:

-	monitoring performance of an application (VAL server or EAS, application session) and providing support for application performance analytics;

-	monitoring performance of a given network slice (from a list of subscribed slices for the VAL customer) and also usage pattern, and providing support for slice-specific application performance analytics and slice usage pattern analytics;

-	monitoring performance of an application session among two or more VAL UEs within a service or group, and providing support for UE-to-UE application performance analytics;

-	monitoring accuracy of a location and providing support for location accuracy analytics;

-	monitoring availability and service level for service APIs and providing support for service API analytics;

-	monitoring edge load parameters and providing support for edge load analytics;

-	monitoring ranging/SL positioning data and location information, and providing support for collision detection analytics.

-	monitoring location information and providing support for location-related UE group analytics.

-	monitoring application layer AI/ML member capability data and providing support for Application Layer AI/ML Member Capability Analytics.

To support ML-enabled analytics services, the ADAE server may provide the above server-side functionalities by consuming SEAL AIMLE services (as specified in 3GPP TS 23.436 [19]).