3GPP TSG-SA WG6 Meeting #63	S6-244263

Hyderabad, India, 14th – 18th October 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0106 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SEALDD enabled reliable guarantee data transmission connection establishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the data transmission connection establishment, in order to provide friendly API to the VAL server, the paper propose to split the data transmission connection establishment procedure into Bandwidth guarantee data transmission connection establishment, Reliability guarantee data transmission connection establishment, Critical frame guarantee data transmission connection establishment. In this paper, reliable guarantee data transmission connection establishment procedure is added in this paper. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added reliable guarantee data transmission connection establishment procedure - Make adaptively adjustment on message and information flow |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is difficult for VAL server to invoke SEALDD functionality |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.2.2, 8.2.11, 8.8.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.2.2.b	SEALDD enabled reliable guarantee data transmission connection establishment procedure

Figure 9.2.2.a-1 illustrate the procedure for establishing reliable guarantee data transmission connection. The reliable guarantee data transmission connection establishment feature can provide VAL server with low latency and high reliability data transmission.

This procedure is similar to the SEALDD enabled Bandwidth guarantee data transmission connection establishment procedure, the differences are as follows:

Figure 9.2.2.2-1: SEALDD enabled reliable guarantee data transmission connection establishment procedure

1.	The VAL server decides to use SEALDD service for reliable guarantee application data traffic transmission, then the VAL server send SEALDD enabled Reliable guarantee data transmission connection establishment request to the SEALDD server. The request includes UE ID/address, VAL server ID, VAL service ID, SEALDD-S connection information of the VAL server side, and optionally, the Reliable guarantee data transmission requirement for the application data traffic.

2.	Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, SEALDD server allocates address of the SEALDD server to receive the application data traffic from the VAL server for application data transfer. The SEALDD server responds VAL server with a SEALDD enabled Bandwidth guarantee response.

The SEALDD server determines the QoS requirement based on the application data transmission requirement provided by VAL server. Then SEALDD server may invoke 5GC function as described in clause 9.2.2.2 step2 in order to establish reliable guarantee data transmission connection.

Step 3-9 are same as the step3-9 in clause 9.2.2.2, the only difference is that athe SEALDD server and SEALDD client establish reliable guarantee data transmission connection to transmit application data based on the Data transmission requirement provided by the VAL server.

9.2.3.Z1	SEALDD enabled reliable guarantee data transmission connection establishment request

Table 9.2.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the reliable guarantee data transmission service.

Table 9.2.3.1-1: SEALDD enabled Reliable guarantee data transmission connection establishment request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |
| UEID/ VAL user ID | O | Identifier of specific UE or VAL user |
| SEALDD-S connection information | M | Address information (e.g. IP address and/or port, URL) of VAL server to receive the signalling traffic from the VAL server |
| Data transmission requirement | M | The data transmission requirement |
| > VAL server’s total bandwidth limit | M (See NOTE) | The total bandwidth limit of VAL server, including UL/DL |
| > VAL users’ bandwidth limit | M (See NOTE) | The bandwidth limits (i.e. minimum bandwidth requirement and maximum bandwidth limit) for VAL users, including UL/DL |
| > Requested Priority | O | The priorigy requested by the VAL server |
| > Maximum Burst Size | O | The Maximum Burst Size provided by the VAL server |
| > Requested 5GS Delay | M | The Requested 5GS Delay provided by the VAL server |
| > Requested Maximum Bitrate | O | The Requested Maximum Bitrate provided by the VAL server |
| > Requested Guaranteed Bitrate | M | The Requested Guaranteed Bitrate provided by the VAL server |
| > Requested Packet Error Rate | O | The Requested Packet Error Rate provided by the VAL server |
| NOTE: These IEs are used for the SEALDD enabled bandwidth control for different VAL users. |  |  |

9.2.3.Z2	SEALDD enabled Reliable guarantee data transmission connection establishment response

Table 9.2.3.2-1 describes the information flow from the SEALDD server to the VAL server for responding to reliable guarantee application data traffic transmission.

Table 9.2.3.2-1: SEALDD enabled reliable guarantee data transmission connection establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicate the bandwidth guarantee data transmission connection establish success or failure. |
| SEALDD-S connection information | O | Address information (e.g. IP address and/or port, URL) of SEALDD server to receive the signalling traffic from the SEALDD server |
| Cause | O See NOTE | Indicates the reason for the failure |
| NOTE: The IE is only present if the Result is failure. |  |  |

* * * END of Change * * * *