3GPP TSG-SA WG6 Meeting #63	S6-244285

Hyderabad, India, 14th – 18th October 2024	(revision of S6-24xxxx)

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on new solution of IOPS network deployment aspect to KI#1

Spec:	3GPP TR 23.700-09 v0.2.0

Agenda item:	8.8

Document for:	Approval

Contact:	gecuili@huawei.com

- Introduction

This pCR is proposed to add solution to KI#1.

2. Reason for Change

In key issue #1, the following bullet are to be studied.

-	Review and evaluate existing work in the current IOPS technical specification 3GPP TS 23.180 [7] and determine if the existing baseline in TS 23.180 [7] can be used in the generic IOPS solution.

-	Provide a summary of the solutions from 3GPP TS 23.180 [7] that are suitable for a generic IOPS model and can be used with the required changes and which cannot be re-used for generic IOPS and must be re-written.

The existing 23.280 based on the IOPS network architecture defined in 23.401 annex K, which is actually an informative deployment option. However, in order to make the mechanism defined in 23.180 generic, the 5G IOPS network deployment is required.

Introduce a Annex to have similar IOPS network deployment as 4G IOPS that a local 5GC is used due a backhual failure.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR  23.700-09 v0.2.0.

* * * First Change * * * *

6.a	Solution #a: Converting the functional model and deployment section of TS 23.180

6.aa.1 General

This clause is related to KI#1 addresses the Scope section of 3GPP TS 23.180. It describes which changes are needed to make the application of functional model to deployments of 3GPP TS 23.180 applicable for all supported 3GPP access methods.

NOTE:	in clause 6.aa.2 and 6.aa.3, the original clause has been copied from TS 23.180. The change marks highlight the required modifications for an access generic IOPS solution.

6.aa.2		Addressing the functional model and deployment section of 3GPP TS 23.180

9	Application of functional model to deployments

9.1	Overview

The IOPS architectural model includes application functions at the IOPS MC system and UEs to support MC services in the IOPS mode of operation during a backhaul failure. The IOPS MC system provides MC services support to the MC service UE via a single PLMN dedicated to the IOPS mode of operation.

The architectural model to support MC services in the IOPS mode of operation consists of a signalling control plane and an application plane. The signalling control plane provides the necessary signalling support for the related IOPS application layer transactions, e.g. the registration and discovery of UEs on the IOPS MC system. The IOPS application plane provides the necessary support for the transport of the IOPS operation related application data as well as the IP packets containing the MC service application data to be distributed via the IOPS MC system.

The MC service application data includes all signalling control data and application data (control and media) required to provide MC services between MC service clients. The IP related transmissions are established over the IOPS MC system via IP unicast and multicast transmissions.

9.2	IOPS architectural model diagram

Figure 9.2-1 shows the IOPS architectural model for the IOPS MC system solution in case of a backhaul failure.

Figure 9.2-1 Architectural model in the IOPS mode of operation

9.2.1	IOPS application services support layer

The IOPS MC system is composed of the following functional entities for the support of MC services in the IOPS mode of operation:

-	an IOPS MC connectivity function as described in clause 7.4.1.3, an IOPS distribution function as described in clause 7.4.1.4 and a signalling application server as described in clause 7.4.2.2.

9.2.2	IOPS network

An IOPS network can comprise either:

-	a Local core network and a single isolated IOPS-capable xG-RAN node, which may be co-located or have connectivity to the Local core network; or

-	a Local core network and two or more IOPS-capable xG-RAN node, which have connectivity to a single Local core network.

NOTE:	For 4G IOPS, the implementation and deployment guidelines using a Local EPC approach is described in 3GPP TS 23.401 [2]. For 5G IOPS, the implementation and deployment guidelines using a Local 5GC approach is described in Annex aa.

The IOPS network provides point-to-point and/or point-to-multipoint transmission services with QoS in the IOPS mode of operation.

9.2.3	UE 1

The UE 1 is an MC service UE supporting the IOPS mode of operation. It supports transmission services and applications to provide MC services over the IOPS MC system. The UE 1 is composed of the following functional entities:

-	for MC services over the IOPS MC system, MC service client(s) as described in clause 7.4.1.5;

-	for IOPS application plane, an IOPS connectivity client as described in clause 7.4.1.2; and

-	for IOPS signalling control, a signalling user agent as described in subclause 7.4.2.1.

9.2.4	UE 2

The UE 2 represents one or more UEs with the same functionality as UE 1.

6.aa.3	Other affected clauses

6.aa.3.1	definitions update

The generic definitions are required.

3	Definitions and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

IOPS MC system: The collection of application functions and enabling capabilities required to support mission critical services in the IOPS mode of operation.

IOPS mode of operation: Refer to the Isolated E-UTRAN operation definition in 3GPP TS 22.346 [9].

IOPS-capable xG-RAN: a xG-RAN node that has the capability of IOPS mode operation, which provides local connectivity (e.g. for IP or Ethernet) and public safety services to IOPS-enabled UEs via a Local core network when the xG-RAN node has lost backhaul to the Macro core network or it has no backhaul to the Macro core network.

IOPS-enabled UE: is an UE that is configured to use networks operating in IOPS mode.

IOPS network: a IOPS network consists of one or more xG-RAN nodes operating in IOPS mode and connected to a Local core network.

For the purposes of the present document, the following terms given in 3GPP TS 23.280 [3] apply

MC service

MC service client

MC service group

MC service ID

MC service user

MC service UE

MC user

6.aa.3.1	References update

New references are introduced.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[3]	3GPP TS 23.280: "Common functional architecture to support mission critical services ".

[4]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[5]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[6]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[7]	3GPP TS 23.002: "Network Architecture".

[8]	3GPP TS 23.203: "Policy and charging control architecture".

[9]	3GPP TS 22.346: "Isolated Evolved Universal Terrestrial Radio Access Network (E-UTRAN) operation for public safety; Stage 1".

[10]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[11]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1"

[36304]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[38413]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[38304]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode".

6.aa.3.1	Introduction of new Annex of 5G IOPS aspect

The 4G IOPS defined in 23.401 annex K is referred in this annex. Also, the 5G IOPS aspects are required to be captured in this Annex.

Annex aa (informative):
Isolated Operation for Public Safety in EPS

4G IOPS aspects is defined in Annex K of 3GPP TS 23.401 [2].

Annex bb (informative):
Isolated Operation for Public Safety in 5GS (5IOPS)

bb.1	General Description of the 5IOPS concept

Isolated Operation for Public Safety in 5GS (5IOPS) provides the ability to maintain a level of communications for public safety users, via a 5IOPS-capable NG-RAN (or set of connected 5IOPS-capable NG-RANs), following the loss of backhaul communications.

The Isolated mode of operation in 5GS is also applicable to the formation of a Nomadic 5GS deployment, i.e. a deployment of one or more standalone 5IOPS-capable NG-RANs, creating a serving radio access network without backhaul communications and also providing local connections and services to Public Safety users in the absence of normal 5GS infrastructure availability.

This annex provides implementation and deployment guidelines for the operation of Public Safety networks in the no backhaul (to Macro 5GC) scenario using a Local 5GC approach.

5IOPS-capable NG-RAN: a NG-RAN node that has the capability of 5IOPS mode operation, which provides local connectivity (e.g. for IP or Ethernet) and public safety services to 5IOPS-enabled UEs via a Local 5GC when the NG-RAN node has lost backhaul to the Macro 5GC or it has no backhaul to the Macro 5GC.

5IOPS-enabled UE: is an UE that is configured to use networks operating in 5IOPS mode.

5IOPS network: a 5IOPS network consists of one or more NG-RAN nodes operating in 5IOPS mode and connected to a Local 5GC.

bb.2	Operation of isolated public safety networks using a Local 5GC

bb.2.1	General Description

This approach to the provision of isolated operation (e.g. when there is no N2 connectivity to the macro 5GC) assumes that the 5IOPS-capable NG-RAN can reach a Local 5GC instance which is used in 5IOPS mode. The Local 5GC instance includes at least AMF, SMF, UDM, PCF and UDM functionalities.

A PLMN ID is dedicated to 5IOPS mode of operation and is broadcast in System Information by the NG-RAN when 5IOPS mode is in operation. Only authorized 5IOPS-enabled UEs can access a PLMN indicated as a 5IOPS PLMN.

The 5IOPS network will provide local connectivity services, i.e. IP address assignment and routing in the 5IOPS network. During the PDU session establishment procedure to the local 5GC a local IP address is assigned to the UE. The Local 5GC acts as a router among the UEs locally registred to the same 5IOPS network.

bb.2.2	UE configuration

A 5IOPS-enabled UE has the dedicated 5IOPS PLMN ID configured in a USIM as an HPLMN along with the Access Class status of 11 or 15, subject to regional/national regulatory requirements and operator policy.

NOTE: Access Class 15 can be reserved for use by network operator personnel who are responsible for critical recovery operations of the network.

When an authorised 5IOPS-enabled UE selects a 5IOPS-mode cell, it registers to the 5IOPS PLMN and performs the authentication procedures.

bb.2.3	5IOPS network configuration

A 5IOPS network can comprise either:

-	A Local 5GC and a single isolated 5IOPS-capable NG-RAN, which has connection with the Local 5GC; or

-	A Local 5GC and two or more 5IOPS-capable NG-RANs, which have connectivity to the single Local 5GC.

Existing procedures described in TS 38.413 [38413] can be used to configure of the N2 interface to AMF. A 5IOPS-capable NG-RAN can be pre-provisioned with IP endpoint information, relating to the AMFs of one or more candidate Local 5GC instances. For each AMF of Local 5GC in turn the NG-RAN can try to initialise a SCTP association. Once SCTP connectivity has been established, the NG-RAN and AMF exchange application level configuration data over the N2 with the NGAP Setup Procedure (see TS 38.413 [38413]). In line with local operator policies the NG-RAN can be provisioned with the IP endpoint of a preferred AMF instance and the IP endpoints of one or more alternative AMF instances. The alternative Local 5GC instances will be used if an N2 path cannot be established with the local AMF of the preferred Local 5GC instance.

All Local 5GCs deployed by a Public Safety authority/operator assume the same PLMN ID. In order to achieve the broadcast of different TAIs on separate 5IOPS networks the TACs broadcast by the cells of NG-RANs connected to different Local 5GCs are distinct to ensure the required UE mobility behaviour (see clause aa.2.5). Therefore, the TAC broadcast by the cells of a NG-RAN operating in 5IOPS mode will be dependent upon the Local 5GC to which the NG-RAN has established an N2 connection to AMF of Local 5GC.

If the scope of service of a Local 5GC is a single NG-RAN, then all cells served by the NG-RAN share the same TAC (assigned for use in 5IOPS mode) and neighbouring NG-RANs that are also operating in 5IOPS mode with the same dedicated PLMN ID are assigned different TACs (resulting in different TAIs) so a mobility registration update attempt is triggered upon mobility.

If multiple NG-RANs are configured to be served by a single Local 5GC, configuration of TAIs for 5IOPS can be done according to local operator policies in such a way that a reselection to a cell operating a PLMN in non-5IOPS mode always triggers an initial registration request.

If sharing the same PLMN ID, it is assumed the TAC assigned to cells in a Nomadic 5GS would be different from the TACs assigned to infrastructure NG-RANs operating in 5IOPS mode, so as to trigger a mobility registration update between these systems.

bb.2.4	5IOPS network establishment/termination

The decision by a 5IOPS-capable NG-RAN to enter 5IOPS mode of operation is made in accordance with the local policies of the operator. In situations when the backhaul to the Macro 5GC is lost and a NG-RAN can start 5IOPS mode of operation based on local policies, or a NG-RAN is deployed as part of a Nomadic 5GS, the following NG-RAN behaviour is expected:

-	If the NG-RAN can reach a Local 5GC for 5IOPS mode of operation, the NG-RAN uses the Local 5GC.

-	If the NG-RAN cannot reach a Local 5GC, then the NG-RAN enters a state where UEs do not attempt to select the cells under its control.

The 5IOPS networks will be established by the independent actions of each NG-RAN entering 5IOPS mode of operation. A 5IOPS network comprising two or more NG-RANs will be established as a result of multiple NG-RANs entering 5IOPS mode of operation and establishing N2 backhaul paths to the local AMF of the same Local 5GC.

A NG-RAN in 5IOPS mode of operation, indicates/broadcasts the 5IOPS PLMN cell(s) as "Not Barred" & "Reserved for Operator Use", for the 5IOPS PLMN identity, as defined in TS 36.304 [36304] and TS 38.304 [38304]. This "Cell Reserved for Operator Use" feature will allow the 5IOPS-enabled UEs to get access to the 5IOPS network while barring other non-5IOPS-enabled UEs in the same area.

When an N2 backhaul to the Macro 5GC is re-established, the N2 connections to the Local 5GC are released according to the local 5IOPS network policies, to move the UEs to Idle mode, and stops the 5IOPS mode of operation. The PLMN ID of the Macro 5GC is announced by the NG-RAN so that UEs reselect the PLMN of Macro 5GC. Figure aa.2.4-1 provides an example of the procedures of 5IOPS network establishment, access and termination.

Figure bb.2.4-1: Example of Local 5GC based 5IOPS operation

1)	The UE registers to the Macro 5GC accessing normal application (e.g. MCPTT) services.

2)	The NG-RAN detects loss of the backhaul to the Macro 5GC and in accordance with local operator policies decides to activate 5IOPS mode of operation.

NOTE 1:	The above steps are not applicable for the Nomadic 5GS case.

3)	Local 5GC is activated. The NG-RAN establishes the backhaul link to Local 5GC.

4)	The NG-RAN broadcasts the PLMN ID for 5IOPS operation with the Local 5GC and indicates the 5IOPS PLMN cell(s) as "Not Barred" & "reserved" for operator use.

5)	The UE detects the 5IOPS PLMN ID and selects the 5IOPS PLMN ID.

6)	The UE registers to the 5IOPS network, including the authentication procedure with the 5IOPS network. After a success registration to the 5IOPS network, the UE establishes PDU session to get access to the Public Safety services and to obtain local IP address.

7)	Public Safety services supported by the 5IOPS network can be accessed at this time.

NOTE 2:	The following steps are not applicable for the Nomadic 5GS case.

8)	At some point in time the NG-RAN detects that the backhaul to the Macro 5GC has been restored.

9)	The local backhaul connections to the Local 5GC are released according to the 5IOPS network policies. The NG-RAN stops its 5IOPS mode of operation and the Local 5GC is de-activated. The UE moves to idle mode. The Local 5GC is deactivated.

10)	The NG-RAN establishes a backhaul link to the Macro 5GC.

11)	The PLMN ID of the Macro 5GC is announced by the NG-RAN.

12)	The UE detects the normal PLMN ID and selects the PLMN ID.

13)	The UE joins the macro network.

bb.2.5	UE mobility

A number of distinct UE mobility scenarios can be identified given the following assumptions as specified in clause aa.2.3:

-	Multiple NG-RANs can be configured to be served by a single Local 5GC;

-	A single dedicated PLMN ID will be advertised by all NG-RANs operating in 5IOPS mode (of a given public safety authority/operator);

-	The TACs broadcast by cells (NG-RANs) served by different Local 5GCs will be different.

The mobility scenarios that can be distinguished are:

1.	UE transitions from a cell controlled by the macro 5GC to a cell operating in 5IOPS mode;

2.	UE transitions from a cell operating in 5IOPS mode to a cell controlled by the normal macro 5GC;

3.	UE transitions from a cell operating in 5IOPS mode whose NG-RAN is served by one Local 5GC to a cell also operating in 5IOPS mode whose NG-RAN is served by a different Local 5GC (Inter-5IOPS network cell transition);

4.	UE transitions between cells operating in 5IOPS mode whose NG-RAN(s) are served by the same Local 5GC (Intra-5IOPS network cell transition).

The expected mobility behaviour in each of these scenarios is summarised in Table bb.2.5-1.

Table bb.2.5-1: UE mobility behaviour

| MOBILITY TRANSITION | IDLE MODE | CONNECTED MODE |
| --- | --- | --- |
| Normal mode cell to 5IOPS mode cell | Cell re-selection: - UE performs cell re-selection based upon radio measurements and no suitable cell is found. - UE performs cell selection and a suitable cell is found. - UE initiates registration and PDU session establishment procedures towards Local/Normal 5GC. | Radio link failure followed by cell re-selection: - UE performs radio measurements but source and target cells are on different networks. - Radio link failure occurs and UE returns to Idle Mode. - UE proceeds as per behaviour for Idle Mode. |
| 5IOPS mode cell to Normal mode cell |  |  |
| Intra-5IOPS network cell transition | Idle Mode mobility as per normal 5GC mobility. | As per normal 5GC Connected mode mobility procedures. |
| Inter-5IOPS network cell transition | Idle Mode mobility as per normal 5GC mobility. | As per normal 5GC Connected mode mobility procedures. |

6.aa.4	Solution evaluation

The above propopsal convertes the current scope section of 3GPP TS 23.180 into an access generic IOPS solution.