3GPP TSG-SA WG6 Meeting #63	S6-244296

Hyderabad, India, 14th – 18th October 2024	(revision of S6-24xxxx)

Source:	Ericsson

Title:	Procedure for Assisting ML Model Training

Spec:	3GPP TS 23.482 V0.2.0

Agenda item:	9.10

Document for:	Approval

Contact:	Jing Yue (jing.yue@ericsson.com)

1. Introduction

Added procedure for assisting ML model training to TS 23.482.

2. Reason for Change

Based on the requset from the consumer (e.g. VAL server), AIMLE server can assist the ML model training at VAL server and among VAL client(s) for HFL and VFL, etc. This paper proposes to add procedure for assisting ML model training to TS 23.482.

3. Conclusions

This paper proposes to add procedure for assisting ML model training to TS 23.482.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.2.0.

* * * First Change * * * *

## 6.A	Support for ML Model Training

This functionality enables a consumer (who can be a VAL server) to request AIMLE server for training an ML model at enablement layer or assisting the ML model training at VAL server and VAL client(s).

* * * Next Change * * * *

## 8.3	ML model training

### 8.3.1	General

This clause describes the procedure for supporting the ML model training by the AIMLE server. The VAL server requests the AIMLE server for the ML model training or assisting ML model training. VAL server requests the AIMLE server to train an ML model by specifying the type of learning, details about the ML model to be trained or requirements to select a ML model for training and criteria for selecting the members who can participate in the ML model training. VAL server can also request the AIMLE server to assist the training of an ML model at the VAL server and VAL client(s).

### 8.3.2	Procedures

#### 8.3.2.1	Procedure for ML model training

Figure 8.3.2.1-1 illustrates the procedure for AIMLE server to support ML model training based on the request from the VAL server.

Figure 8.3.2.1-1: ML model training

1.	The VAL server sends an ML model training request to AIMLE server, requesting to assist in its ML model training. This request consists of ML model information or ML model requirement information, etc.

2.	The AIMLE server checks whether the VAL server is authorized to perform the ML model training request. If no model information is provided but only the model requirement information is provided in step 1, the AIMLE server identifies and selects the appropriate ML model for training based on the ML model requirement information.

Editor’s Note: Adding a reference to the procedure for ML model discovery and selection is FFS.

3.	If the VAL server is authorized, AIME server returns the success response, otherwise a failure response indication the reason for failure.

* * * Next Change * * * *

#### 8.3.2.X	Procedure for assisting ML model training

Figure 8.3.2.X-1 illustrates the procedure for AIMLE server to assisting ML model training based on the request from the VAL server.

Figure 8.3.2.X-1: Assist ML model training

1.	The VAL server sends an ML model training assist request to AIMLE server, requesting to assist the ML model training at the VAL server and may also at VAL client(s). This request consists of parameters as described in clause 8.3.3.Y1.

2.	The AIMLE server checks whether the VAL server is authorized for the ML model training assist request. If the VAL server is authorized, the AIMLE server generates assistance information for the ML model training at the VAL server and may also at VAL client(s).

3.	The AIME server sends ML model training assist response to the VAL server. The response contains the information as described in clause 8.3.3.Y2.

* * * Next Change * * * *

#### 8.3.3.2	ML model training response

Table 8.3.3.2-1 describes the information flow of ML model training response from the AIMLE server to the VAL server.

Table 8.3.3.2-1: ML model training response

| Information element | Status | Description |
| --- | --- | --- |
| Success response | O (NOTE) | Indicates that the ML model training request was successful. |
| > ML model identifier | O | Identifies the ML model selected by AIMLE server for training. |
| > Indication of ML model training complete | O | Indication of the ML model training (e.g. VFL) is completed, and the trained ML model(s) can be used for inference, but the ML model(s) cannot be shared. |
| Failure response | O (NOTE) | Indicates that the ML model training request was failure. |
| > Cause | M | Reason for the failure. |
| NOTE: At least one of these information elements shall be present |  |  |

* * * Next Change * * * *

#### 8.3.3.Y1	ML model training assist request

Table 8.3.3.Y1-1 describes the information flow of ML model training assist request from the VAL server to the AIMLE server.

Table 8.3.3.Y1-1: ML model training assist request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | Identity of the VAL server performing the request. |
| Training type | M | Identifies the type of ML model training to be performed, e.g. traditional ML model training at single entity, FL (e.g. VFL or HFL). |
| Training member(s) | O | The member or list of members, e.g. VAL server, VAL clients, to be utilized for training the ML model. The IE shall be provided when the assit type is member status monitoring, may be provided when the assist type is training status evaluation. |
| Assist type | M | The type of assist required for the ML model training, e.g. member selection, member status monitoring, training status evaluation, etc. |
| > Information for member selection | O (NOTE) | Indicates the information for member selection when the assist type is member selection. |
| >> Number of member(s) | O | Indicates the number of member(s) required for the ML model training. |
| > Information for member status monitoring | O (NOTE) | Indicates the information for member status monitoring when the assist type is member status monitoring. |
| >> Monitor status | O | Indicates the status of member(s) to be monitored, e.g. available time change, capability change (e.g. communication capability, computation capability). |
| > Information for training status evaluation | O (NOTE) | Indicates the information for training status evaluation when the assist type is training status evaluation. |
| NOTE: At least one of these information elements shall be present. |  |  |

Editor's Note: The IEs under each of the assist type are FFS.

#### 8.3.3.Y2	ML model training assist response

Table 8.3.3.Y2-1 describes the information flow of ML model training assist response from the AIMLE server to the VAL server.

Table 8.3.3.Y2-1: ML model training assist response

| Information element | Status | Description |
| --- | --- | --- |
| Success response | O (NOTE) | Indicates that the ML model training assist request is successful. |
| > Assist information | M | The information for assisting the ML model training. |
| Failure response | O (NOTE) | Indicates that the ML model training assist request is failure. |
| > Cause | M | Reason for the failure. |
| NOTE: At least one of these information elements shall be present |  |  |

Editor's Note: Whether more sub-IEs need to be added under the assist information are FFS.

# *** End of Changes ***