3GPP TSG-SA WG6 Meeting #63	S6-244313

Hyderabad, India, 14th – 18th October 2024

Source:	Ericsson

Title:	Spatial Anchor Create Group Procedure

Spec:	3GPP TS 23.437 v0.2.0

Agenda item:	9.11

Document for:	Approval

Contact:	ashish.sanjay.sharma@ericsson.com

1. Introduction

This CR brings normative text related to the spatial anchor group creation procedure.

2. Reason for Change

The create group procedure was concluded in the study to consider for the normative work.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.437 v0.2.0.

* * * First Change * * * *

#### 8.3.x	Creating spatial anchor group

##### 8.3.x.1	General

Spatial anchor group creation enables a San client or VAL server to create a group of spatial anchor(s) with the SAn server.

##### 8.3.x.2	Procedure

Figure 8.3.x.2-1 depicts the procedure for creating the group of spatial anchor services linked to each other. The service is provided by Spatial Anchor server and consumed by VAL server or SAn client.

Figure 8.3.x.2-1: Create spatial anchor group

1)	The VAL server (or SAn) sends a request message to the Spatial Anchor server to create a group of spatial anchors. The request includes VAL server identity, spatial anchor identity list and spatial anchor service area, as per the Table 8.3.x.3.1-1.

NOTE: The group spatial anchor provides an abstraction for the underlying linked spatial anchor(s), which enables to discovery of the linked spatial anchors as per the request of the user or based on the location, etc.

2)	The MMES server authorizes VAL server (or SAn client). If the requestor is authorized, then the SAn server creates a group spatial anchor (along with its identity) for the linked spatial anchors for the given spatial anchor service area. The SAn server sends the response back to the VAL server (or SAn client) including the spatial anchor (along with its identity) of the spatial anchor for a successful case.

* * * End of Changes * * * *

#### 8.3.x.3	Information flows

##### 8.3.x.3.1	Create spatial anchor group request

Table 8.3.x.3.1-1 describes information elements for the spatial anchor create request from the SAn client or VAL server to the SAn server.

Table 8.3.x.3.1-1: Create Spatial anchor group request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| VAL service information | M | Information about the application service associated with the spatial anchor. |
| > Service ID | M | The identifier of the application service. |
| > Service endpoint | M | Endpoint information (e.g., URI, FQDN, IP address) used to communicate with the application service. |
| > Service connection information | O | The connection information (e.g., DNN, DNAI(s)) for establishing connectivity to the application service. |
| List of spatial anchors | M | List of spatial anchors identities to be linked in the group. |
| Spatial anchor service area | O | Indicates the service area for the grouped spatial anchor |
| Spatial anchor specific information | O | A container to contain application specific information. |

##### 8.3.x.3.2	Create Spatial anchor group response

Table 8.3.x.3.2-1 describes information elements for the Create spatial anchor group response from the SAn service.

Table 8.3.x.3.2-1: Create Spatial anchor group response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the spatial anchor create request was successful. |
| Grouped Spatial anchor anchor ID | O | Identity for the spatial anchor group |
| Failure response | O | Indicates that the create spatial anchor group request has failed. |
| > Cause | O | The cause for the request failure. |

* * * End of Changes * * * *