3GPP TSG-SA WG6 Meeting #63	S6-244460

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244413)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0569 | rev | 3 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Location reporting information obtained from the PLMN operator (LTE) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TS 22.280, there is the location related requirement: [R-6.12-002] The MCX Service shall support conveyance of Location information provided by 3GPP location services. Location information obtained from the network comes with several advantages to the MC operators, as discussed in S6-244077, as: - The MC service server is always aware of the location information of MC users irrespective of possible unforeseen events, such as GPS deactivation intentionally or unintentionally, GPS not working properly, UE is off during a time a trigger should have been sent. - The MC service server is independent from the LMC (including its configuration) for its operations and decision making. - Maintain network resources and offload the radio interface especially during network congestion cases. There are two main mechanisms to obtain the location information from the core network: LCS services and via PCRF/PCF notification as part of the PCC rule mechanism. The LCS network can provide location information either on demand based (immediately) or event-triggered based, according to the procedures defined in 3GPP TS 23.271, e.g., mobile terminating location request related procedures. The type of events related to the event triggered based request are the UE availability (after temporarily being unavailable due to e.g., inaction of the user, IMSI detach, radio connectivity loss), periodic reporting, changing of a predefined geographical area or UE motion once moving more than a predefined distance from a certain location. The location information provided by the LCS report includes, among others, the geographical location of the MC service UE, the time stamp of the location information, and related event based on the requested triggers as mentioned earlier. Furthermore, the MC service server can obtain the Access Network Information Reporting parameters upon subscribing to the PCRF notification. The Access Network Information Reporting includes the user location report (e.g., the cell ID, SAI, PLMN ID) and the UE time zone, as described in 3GPP TS 23.203. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - adding the reference point Le in clause 7.3.1 - adding a note in clause 7.3.1 related to the Le reference point - remove the note in clause 7.4.2.2.10 - adding a description for Le reference point - update clause 10.9 to refer to the two mechanisms described earlier - update clause 10.6.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 1 requirement indicated above is not met, and the mechanisms how the MC service server obtains location information from the PLMN operator is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 7.3.1, 7.4.2.2.10, 7.5.2.x (new), 10.6.1, and 10.9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 23.280 CR 0597, CR 0584 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Figure 7.3.1-1 is updated by adding the reference point Le between the location management server and EPS. After implementing this CR, the same figure will be updated as per CR0584. Regarding the updates done in clause 10.6.1, CR 0597 needs to be implemented first, then this CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 7.3.1	On-network functional model

Each MC service can be represented by an application plane functional model. The functional model across MC services may be similar but is described by the individual functional entities and reference points that belong to that MC service. Within the application plane for an MC service there is a common set of functions and reference points. The common set is shared across services. This common set of functions and reference points is known as the common services core.

Figure 7.3.1-1 shows the functional model for the application plane for an MC system.

Figure 7.3.1-1: Functional model for application plane for an MC system

The common services core functions and reference points shown in figure 7.3.1-1 are shared across each MC service. The description of the functions and reference points specific to an MC service is contained in the corresponding MC service TS.

NOTE 1:	The ACM client and ACM server are not essential entities to enable MC services. The ACM client and ACM server are optional and only required if the administrative configuration management feature is deployed. In administrative configuration management case, only the UE of an authorized MC service user requires such an ACM client.

NOTE 2:	The recording server, replay client and mass storage(s) are optional entities. They are not essential to enable MC services.

NOTE 3:	MC replay UE can connect to the CSC and recording server using EPS or other communication networks.

NOTE 4:	The Le reference point is not an essential reference point to enable MC services. It is only required if location information of MC service UE(s) is obtained from the PLMN operator via the LCS.

In the model shown in figure 7.3.1-1, the following apply:

-	A specific MC service server is an instantiation of a GCS AS in accordance with 3GPP TS 23.468 [18].

-	The functional alias management client is an integrated functional entity of the configuration management client. The functional alias management client is described in subclause 7.4.2.2.12.

-	The functional alias management server is an integrated functional entity of the configuration management server. The functional alias management server is described in subclause 7.4.2.2.13.

Figure 7.3.1-2 shows the functional model for the signalling control plane.

Figure 7.3.1-2: Functional model for signalling control plane

Figure 7.3.1-3 shows the relationships between the reference points of the application plane of an MC service server and the signalling plane.

Figure 7.3.1-3: Relationships between reference points of MC service application plane and signalling control planes

NOTE 5:	Application plane reference point CSC-7 makes use of SIP-2 reference point when the group management servers are connected by a single SIP core. Where they are joined by more than one SIP core, CSC-7 also makes use of the SIP-3 reference point.

NOTE 6:	For simplicity, the HTTP proxy, which provides the interconnection between HTTP-1, HTTP-2 and HTTP-3 reference points, is not shown in figure 7.3.1-3.

NOTE 7:	CSC-5, CSC-9, and CSC-15 make use of SIP-1 and SIP-2 reference points. For simplicity, this mapping relationship is not shown in figure 7.3.1-3.

# * * * * Second change * * * *

##### 7.4.2.2.10	Location management server (LMS)

The location management server is a functional entity that receives and stores user location information, and provides user location information to the MC service server. The location management server may also acquire location information provided by PLMN operator (e.g. LCS).

# * * * * Third change * * * *

#### 7.5.2.x	Reference point Le

The Le reference point, which exists between the location management server and the 3GPP core network, is used to obtain location information of MC service UEs using the LCS. The Le reference point is defined in 3GPP TS 23.271 [29].

# * * * * Fourth change * * * *

### 10.6.1	Primary MC system

NOTE:	Figure 10.6.1-1 is a high level user authentication and authorization flow. 3GPP TS 33.180 [25] defines the specific user authentication and authorization architecture required by the MC services in order to realize the MC service user authentication and authorization requirements as defined in 3GPP TS 22.280 [3].

The user authentication process shown in figure 10.6.1-1 may take place in some scenarios as a separate step independently from a SIP registration phase, for example if the SIP core is outside the domain of the MC service server.

A procedure for user authentication is illustrated in figure 10.6.1-1. Other alternatives may be possible, such as authenticating the user within the SIP registration phase.

Figure 10.6.1-1: MC service user authentication and registration with Primary MC system, single domain

1.	In this step the identity management client begins the user authorization procedure. The MC service user supplies the user credentials (e.g. biometrics, secureID, username/password) for verification with the identity management server. This step may occur before or after step 3. In a MC system with multiple MC services, a single user authentication as in step 1 can be used for multiple MC service authorizations for the user.

2.	The signalling user agent establishes a secure connection to the SIP core for the purpose of SIP level authentication and registration.

3.	The signalling user agent completes the SIP level registration with the SIP core (and an optional third-party registration with the MC service server(s)). If location information is obtained from the PLMN operator via the LCS, the MC service client shall share the MC service UE’s IMSI to the MC service server during the third party registration.

NOTE:	The MC service client(s) perform the corresponding MC service authorization for the user by utilizing the result of this procedure.

# * * * * Fifth change * * * *

### 10.9.1	General

Location information of MC service user shall be provided by the location management client to the location management server. The location information may also be provided by the 3GPP core network (i.e., via the PLMN operator). The location information reporting triggers are based on the location reporting configuration. Different type of location information can be provided.

NOTE 1:	It is up to implementation whether location information reporting from the core network is considered as an optional or primary source of location information to the MC system.

The location information provided by the 3GPP core network can be obtained from the location services feature in EPS (i.e., LCS) as described in 3GPP TS 23.271 [29], or from the PCRF as part of the PCC mechanism once the MC service server subscribes to related notifications from the PCRF as described in 3GPP TS 23.203 [8].

The location management client can be configured to store location information while not reporting location information to the location management server. The location management client may report stored location information to the location management server.

NOTE 2:	Configuration and location information transmission may occur while there is a communication link between the location management client and location management server.

In the information flows of clause 10.9.2, and procedures of clause 10.9.3:

-	the term "requesting MC service user" is used to refer to the MC service user which sends a request to subscribe to location information reports, to trigger a location information report or to modify existing location information triggers from another MC service user;

-	the term "requested MC service user" is used to refer to the MC service user whose location information is requested or existing location information triggers are to be modified; and

-	the term "reporting MC service user" is used to refer to the MC service user which provides location related information based on on-demand request or trigger criteria.

# * * * * End of changes * * * *