3GPP TSG-SA6 Meeting #63	S6-244514

(Revision of S6-244253)

# Hyderabad, India, 14th Oct 2024 - 18th Oct 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0694 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service Provisioning and EAS discovery for satellite EDN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN N.V., Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | 3GPP TSG SA WG6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_App |  |  |  |  |  | Date: |  |  | 2024-10-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There has been an interim conclusion which captured in the Annex A.2.5 on service provisioning for satellite EDN. The satellite edge computing solution, as concluded in TR 23.700-01 (KI#2, sol#1), facilitates UE application to connect to satellite EAS. This contribution brings in both the service provisioning and discovery aspect and also brings in the Trajectory ID along with Satellite ID. Satellite ID can be used to find the satellite EDN matching the UE location and the Trajectory ID is used to optimize the selection of satellite EDN for planned route of the AC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added Service Provisioning procedure for satellite EDN and modified the respective IEs in the respective Tables. Added Discovery procedure for satellite EDN and modified the respective IEs in the respective Tables. The Trajectory ID is used together with Satellite ID in case of Dynamic Service area to optimize the selection of satellite EDN matching the planned route of the AC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Service Provisiong and the Discovery of EAS is still not addressed for Satellite EDN and selection of the satellite EDN while cosidering planned route of the AC is still not optmized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.5, 11 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 11	Edge Enabler Using Satellite Access

## 11.1	General

This clause provides clarifications on the procedures and information flows necessary for enabling edge application using satellite access.

* * * Second Change * * * *

## 11.x	Service provisioning for satellite EDN

### 11.x.x	Procedure

This procedure is applicable for EDN deployment with gNB on satellite.

A satellite EDN may be deployed as described in Annex A.2.x. The service provisioning procedure for satellite EDN is   same as clause 8.3.3.2.2 and clause 8.3.3.2.3 with the following clarifications.

During service provisioning request/subscription, the EEC provides necessary satellite capability information (e.g., satellite frequency bands) to the ECS. The ECS monitors UE location and movement prediction, and queries external servers to get a list of Satellite IDs that match the UE location and Trajectory IDs that match the UE planned route. The external server provides satellite availability information based on real-time satellite orbit data (e.g., eccentricity, inclination, true anomaly).

For EES on board a MEO or LEO satellite the service area will be dynamic due to EES mobility. The Dynamic EDN service area indication is used in the EES profile and EDN Configuration Information to indicate wheather the service area is mobile. When the service area is mobile, the trajectory of the EES is considered for efficient service provisioning, ensuring the selected EDN matches the movement of the AC hosting UE during the AC’s operation schedule.

The ECS uses the list of Satelllite IDs and Trajectory IDs for a UE location and UE planned route to provision an EES by matching them with the Satellite IDs and Trajecotry IDs in EES profiles. This enables the ECS to determine a suitable EES with both a matching Satellite IDs and Trajectory ID to serve the UE for the longest service time, considering the satellite’s trajectory and selecting an EDN that aligns with the AC’s operation schedule. Multiple satellites can use the same Trajectory ID, and the UE can also use the corresponding trajectory.

The “Expected AC Geographical Service Area” in the AC profile, as specified in clause 8.2.2, can be used by the ECS to request the external server (managed by the satellite service provider or operator) for a list of Trajectory IDs that match the UE’s planned route and for a list of Satellite IDs that match the UE’s location.

### 11.x.y	Information flows

#### 11.x.y.z1 EES profile

The EES profile is same as clause 8.2.6 with the following additions.

Table 11.x.y.z1-1: EES Profile

| Information element | Status | Description |
| --- | --- | --- |
| IEs from Table 8.2.6-1 |  |  |
| Dynamic service area indication | O | Indicates if the service area is dynamic or not. The service area is dynamic in case of EES on board a MEO or LEO satellite and not dynamic in case of a GEO satellite. |
| > Satellite ID | M | The satellite ID which identifies a satellite EDN. |
| >Trajectory ID (NOTE x) | O | For mobile EES, it is linked with the trajectory of the satellite and therefore identifies the trajectory of the EES on board a MEO and LEO satellites. |
| NOTE x: The assignment of the trajectory ID for the EES and linking it to an actual trajectory of the satellite can be done by the operator and/or the satellite service provider. |  |  |

#### 11.x.y.z2 Service provision request

Table 11.x.y.z2 is same as clause 8.3.3.3.2 Table 11.3.3.3.2-1 with the following additions.

Table 11.x.y.z2-1: Service provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| IEs from Table 8.3.3.3.2-1 |  |  |
| Satellite Connectivity Information | O | List of satellite connectivity information for the UE, e.g. satellite RAT Types, satellite frequency bands, Type of Array (e.g. rectangular or hexagonal), Minimum elevation angle. |
| UE location (NOTE x) | O | The location information of the UE. The UE location is described in clause 7.3.2, or Present UE location (e.g. latitude and longitude) for a reference grid point. |
| NOTE x: This IE is not an additional IE but update an existing IE in Table 8.3.3.3.2-1. |  |  |

#### 11.x.y.z3 Service provisioning subscription request

Table 11.x.y.z4 is same as clause 8.3.3.3.4 Table 8.3.3.3.4-1. with the following additions.

Table 11.x.y.z3-1: Service provisioning subscription request

| Information element | Status | Description |
| --- | --- | --- |
| IEs from Table 8.3.3.3.4-1 |  |  |
| Satellite Connectivity Information | O | List of satellite connectivity information for the UE, e.g. satellite RAT Types, satellite frequency bands, Type of Array (e.g. rectangular or hexagonal), Minimum elevation angle. |
| UE location (NOTE x) | O | The location information of the UE. The UE location is described in clause 7.3.2, or Present UE location (e.g. latitude and longitude) for a reference grid point. |
| NOTE x: PLMN ID, satellite RAT types, frequency bands, Type of Array, Minimum elevation angle and present UE location for a reference grid point are applicable for UE connecting with satellite. |  |  |

#### 11.x.y.z4 EDN configuration information

Table 11.x.y.z3 is same as clause 8.3.3.3.3 Table 8.3.3.3.3-2 with the following additions.

Table 11.x.y.z4-1: EDN configuration information

| Information element | Status | Description |
| --- | --- | --- |
| IEs from Table 8.3.3.3.3-2 |  |  |
| > Dynamic EDN service area indication | O | Indicates if the service area is dynamic or not. The service area is dynamic in case of EES and EAS on board a MEO or LEO satellite and not dynamic in case of a GEO satellite. |
| >> Satellite ID | M | The satellite ID which identifies a satellite EDN. |
| >> Trajectory ID (NOTE x) | O | For mobile EDN, it is linked with the trajectory of the satellite and therefore identifies the trajectory of the EES on board a MEO and LEO satellites. |
| NOTE x: The assignment of the trajectory ID for EDN (EES and EAS) and linking it to an actual trajectory of the satellite can be done by the operator and/or the satellite service provider. |  |  |

* * * Next Change * * * *

## A.2.5	Option 4. EDN in GEO/MEO/LEO satellite

Figure A.2.5-1: EDN deployment with gNB on satellite

Figure A.2.x-2: EDN deployment with gNB on ground

In this deployment option, ECS is deployed on ground. The UE can be on the ground/sea or in the air (e.g. drone). The  UPF to access satellite EDN is deployed on satellite, EAS and EES are deployed in one or more satellites. RAN (e.g. gNB) can either be deployed on ground/sea (e.g. in a ship) and connected to satellite UPF or be deployed on regenerative satellite as depicted in Figure A.2.5-1 and Figure A.2.5-2, respectively. The 5GS control plane functions (e.g. AMF, SMF) are deployed on the ground, which is not depicted in the figure for simplicity. EASs and their registered EES are deployed on one or more satellites which constitutes an EDN and these satellites’ coverage areas can correspond to an EDN service area. UPF can be deployed on ground to access the ECS, the EEC (in UE) can reach the ECS by EDGE-4 (on ground or via space). The EES and EAS on board the satellite can be mobile depending on the satellite they are deployed on: GEO, MEO, or LEO. For GEO satellites, since the satellites are stationary relative to the Earth's surface, the EDN is static. However, for MEO and LEO satellites, the EDN is mobile as the satellites are moving relative to the Earth's surface.

NOTE 1:	If there are multiple EESs in a satellite EDN, EEC can trigger EAS discovery towards each EES which increases delay due to EDGE-1 interactions. It is recommended to deploy a single EES per EDN to reduce complexity in satellite. The UPF and RAN deployment are described above for the sake of clarity which is related to EDN on-board satellite (e.g. EDGE-4 path), see also 3GPP TS 23.501 [2] clause 5.43.2).

During initial service discovery, the EEC (in UE) contacts the ECS via EDGE-4 (on ground or via space) to find an appropriate EES, then an appropriate EAS instance is selected during EAS discovery via EDGE-1 interaction. Finally, AC (in UE) communicates with the selected EAS.

* * * END of Change * * * *