3GPP TSG-SA WG6 Meeting #63	S6-244583

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244214)

Source:	NTT DOCOMO

Title:	Pseudo-CR on Editorial corrections in TS 23.482 V0.2.0

Spec:	3GPP TS 23.482 V0.2.0

Agenda item:	9.10

Document for:	Approval

Contact:	yuushin.hayashi.ew@nttdocomo.com

1. Introduction

This contribution corrects incorrect text in section 8.12.2

2. Reason for Change

This addition corrects the inclusion of the"fail" to "failure" in 8.12.2.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.2.0

* * * First Change * * * *

-8.12.2	Procedure

Pre-conditions:

- AIMLE client discovery and selection have been performed.

- Datasets are available and prepared for AI/ML training at the AIMLE clients.

Figure 8.12.2-1: HFL training

  - An AIMLE server receives an ML model training request from a VAL server as described in clause 8.3.2. If the AIMLE server determines to use HFL training, the procedure continues to step 2.

  - The AIMLE server retrieves the indicated ML model using the ML model retrieval procedure as described in clause 8.1.

  - At the beginning of a training round, the AIMLE server performs AIMLE client selection using the AIMLE client selection criteria or a list of AIMLE clients provided in step 1. If AIMLE client selection criteria are provided in step 1, the AIMLE server continuously monitors and selects AIMLE clients for the HFL training. If a list of AIMLE clients is provided in step 1, the AIMLE server selects the provided AIMLE clients for the HFL training.

  - The AIMLE server configures training schedule for each of the AIMLE client and sends a HFL training subscription request with information as described in Table 8.12.3.1-1.

  - Each AIMLE client sends a HFL training subscription response with information as described in Table 8.12.3.2-2. If the AIMLE client is not able to grant the subscription (e.g., is not able to perform the training), the AIMLE client sends a response with a failure status and the procedure skips to step 7.

  - Each AIMLE client performs local training using the configured AI/ML model, model parameters, and the prepared local data associated with the dataset identifier for the specified number of samples according to the operational schedule.

  - Upon completion or due to errors in the training, each AIMLE client sends a HFL training notification to the AIMLE server. The notification includes information as described in Table 8.12.3.1-3. The AIMLE client provides an AIMLE service ID for the HFL training operation.

  - If errors were encountered, the procedure skips to step 8.

If training was successful, the AIMLE server aggregates (e.g. averages) the model parameters received from the AIMLE clients.

If the training schedule is not complete (e.g., there are remaining training rounds), the AIMLE server configures the next set of training schedules and steps 3 to 7 are repeated for the next training round.

When the training schedule has been exhausted (e.g. there are no remaining training round) and if configured, the AIMLE server makes a ML model information storage request as described in clause 8.11.2 to store the AI/ML model in the ML repository.

  - The AIMLE server sends a ML model training notification to the VAL server as described in clause 8.3.2.

Editor’s Note: The description of the procedure will be further discussed and possibly reformatted.

* * * End of Changes * * * *