3GPP TSG-SA WG6 Meeting #63	S6-244593

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244051, S6-243639)

Source:	Convida Wireless LLC

Title:	AIMLE data management

Spec:	3GPP TS 23.482 V0.2.0

Agenda item:	9.10

Document for:	Approval

Contact:	Ly.Quang@convidawireless.com, liu.lu@convidawireless.com

1. Introduction

The contribution introduces a procedure for AIMLE data management with necessary modifications for inclusion into TS 23.482.

2. Reason for Change

The conclusion for key issue #3 in TR 23.700-82 stated that solution #21 with modifications can be considered for normative work. The discussion paper S6-244050 describes what AIMLE data management is and why it is needed in AIML_App. It is well known that having quality data is crucial to the successful training of machine learning models. In addition, training data in AIML_App are application centric and therefore requires support for data management operations to ensure quality data are used to train ML models. Data management operations are futher required with Federated Learning due to data privacy concerns since data belongs to the UEs. AIMLE data management includes data collection, data preparation, and exploratory data analysis.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.2.0.

* * *  First Change  * * * *

## 8.x	AIMLE data management assistance

### 8.x.1	General

AIMLE data management assistance is the process of the AIMLE server assisting AIMLE service consumers with managing data operations performed by VAL clients. The data operations include data preparation and data analysis. The AIMLE server offloads the AIMLE service consumer from interacting with AIMLE clients to manage the data operations. The VAL clients perform the actual data preparation and data analysis operations and send the outputs to the AIMLE server for aggregation.

The following clauses specify procedures, information flows, and APIs for AIMLE data management assistance.

### 8.x.2	Procedure

Pre-conditions:

- AIMLE clients have registered with AIMLE server.

- Application data have been collected and dataset identifier have been assigned to each dataset.

Figure 8.x.2-1: AIMLE data management assistance

- An AIMLE service consumer (e.g., VAL server) makes a request for AIMLE data management assistance. The request includes information as described in Table 8.x.3.1-1.

- The AIMLE server authenticates and authorizes the request. If authorized, the AMILE server assigns an identifier for the subscription.

- The AIMLE server sends an AIMLE data management assistance subscription response to the AIMLE service consumer. The response includes information as described in Table 8.x.3.2-1.

- The AIMLE server sends Client data management requests to AIMLE clients. The request can be for data preparation or data analysis. The request includes information as described in Table 8.x.3.4-1.

- Each AIMLE client performs the data operation necessary to trigger data management at the UE, e.g. performed by VAL clients. The VAL client performs the operation locally.

NOTE: The data preparation and data analysis functions operate in a similar manner as ML models. The AIMLE layer is able to transport the functions to the VAL clients and the VAL clients performs the associated function on the application data as part of data preparation or data analysis in a similar manner as ML training.

- After completing the data operation, each AIMLE client sends a response to the AIMLE server with information as described in Table 8.x.3.5-1.

- The AIMLE server aggregates the output of each AIMLE client. Aggregation includes combining or performing a statistical operation on the received data. Data received from the AIMLE clients can be numerical or categorical, which allows the AIMLE server to aggregate the output data from the AIMLE clients. If necessary, steps 4–7 is repeated to complete all the required data operations.

- The AIMLE server sends an AIMLE data management assistance notification to the AIMLE service consumer with information as described in Table 8.x.3.3-1.

### 8.x.3	Information flows

#### 8.x.3.1	AIMLE data management assistance subscription request

Table 8.x.3.1-1 shows the request sent by an AIMLE service consumer to an AIMLE server for the AIMLE data management assistance subscription procedure.

Table 8.x.3.1-1: AIMLE data management assistance subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Data management operations | M | An indicator showing what data management type is being requested: data preparation, data analysis. |
| Data management requirements | M | Requirements for the data management request: |
| >Data preparation requirements | O (NOTE1) | Data preparation requirements as detailed in Table 8.x.3.1-2. |
| >Data analysis requirements | O (NOTE1) | Data analysis requirements as detailed in Table 8.x.3.1-3. |
| AIMLE clients list | O (NOTE2) | A list of AIMLE clients for which data management should be performed. The list may be specified by AIMLE client set identifier. |
| AIMLE client selection criteria | O (NOTE2) | Selection criteria for finding suitable AIMLE clients for AI/ML operations as detailed in Table 8.8.3.1-2. |
| NOTE1: At least one of the information elements shall be provided. NOTE2: At least one of the information elements shall be provided. |  |  |

Table 8.x.3.1-2: Data preparation requirements

| Information element | Status | Description |
| --- | --- | --- |
| Dataset identifier | M | An identifier for the dataset. |
| Data preparation requirements | M | Requirements for data preparation. |
| > Dataset feature ID | M | Identifier or name of dataset feature to process. |
| > Data preparation function | M | Data preparation function to process the data. The function can be an identifier of a function or an executable. |

Table 8.x.3.1-3: Data analysis requirements

| Information element | Status | Description |
| --- | --- | --- |
| Dataset identifier | M | An identifier for the dataset. |
| Dataset analysis requirements | M | Requirements for data analysis. |
| > Dataset feature ID | M | Identifier or name of dataset feature. |
| > Data analysis function | M | Statistical function for which data analysis is performed. The function can be an identifier of a function or an executable. |

#### 8.x.3.2	AIMLE data management assistance subscription response

Table 8.x.3.2-1 shows the response sent by the AIMLE server to the AIMLE service consumer for the AIMLE data management assistance subscription procedure.

Table 8.x.3.2-1: AIMLE data management assistance subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the data management operation |
| Subscription identifier | M | An identifier for the subscription. |
| Expiration time | O | Expiration time for the subscription |

#### 8.x.3.3	AIMLE data management assistance notify

Table 8.x.3.3-1 shows the notification sent by the AIMLE server to the AIMLE service consumer for the AIMLE data management assistance subscription procedure.

Table 8.x.3.3-1: AIMLE data management assistance notify

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the data management operation |
| Aggregated data preparation outputs | O (NOTE1) | Provides outputs for data preparation: dataset identifier, dataset features, and prepared data output. |
| Aggregated data analysis outputs | O (NOTE2) | Provides outputs for data analysis: dataset identifier, statistical outputs for each feature, list of outlier and anomal values, and feature correlation information. |
| Timestamp | O | Timestamp of the data management notification |
| NOTE1: At least one of the information elements shall be provided in the output. NOTE2: The output format can be numerical or categorical. If categorical, the format can be nominal or ordinal. |  |  |

#### 8.x.3.4	Client data management request

Table 8.x.3.4-1 shows the request sent by the AIMLE server to AIMLE clients for the AIMLE client data management procedure.

Table 8.x.3.4-1: Client data management request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Data management type | M | An indicator showing what data management type is being requested: data preparation, data analysis. |
| Data management requirements | M | Requirements for the data management request: |
| >Data preparation requirements | O (NOTE) | Data collection requirements as detailed in Table 8.x.3.1-2. |
| >Data analysis requirements | O (NOTE) | Data collection requirements as detailed in Table 8.x.3.1-3. |
| Operational schedule | O | A schedule to perform the requested data management operation. |
| NOTE: At least one of the information elements shall be provided. |  |  |

#### 8.x.3.5	Client data management response

Table 8.x.3.5-1 shows the response sent by AIMLE clients to the AIMLE server for the Client data management procedure.

Table 8.x.3.5-1: Client data management response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the data management operation |
| Data preparation outputs | O (NOTE1) | The output data after performing data preparation. One output data is generated for each requirement. (NOTE2) |
| Data analysis outputs | O (NOTE1) | The output data generated by data analysis. One output data is generated for each requirement. (NOTE2) |
| Timestamp | O | Timestamp of the data management operation |
| NOTE1: At least one of the information elements shall be provided in the output. NOTE2: The output format can be numerical or categorical. If categorical, the format can be nominal or ordinal. The AIMLE server is able to process either type of data. |  |  |

* * * End of Changes * * * *