3GPP TSG-SA WG6 Meeting #63	S6-244604

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244316)

Source:	Ericsson

Title:	Enhancements to add task-related capabilities in registration

Spec:	3GPP TS 23.482 v0.2.0

Agenda item:	9.10

Document for:	Approval

Contact:	ashish.sanjay.sharma@ericsson.com

1. Introduction

This CR brings normative text on the addition of task-related capabilities in the registration procedure.

2. Reason for Change

The clients will not be able to register their AIML task capabilities to the AIML server. Because of the unavailability of such information, the AIML server cannot discover AIML clients who can perform certain AIML tasks like during split operation, model training etc.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 v0.2.0.

* * * First Change * * * *

## 8.7	AIMLE client registration

### 8.7.1	General

Prior to participation in AI/ML operations, AI/ML capable UEs register to an AIMLE server and provide an AIMLE client profile and optionally a list of supported services. The AIMLE server uses information provided by the AIMLE client to discover and select suitable AIMLE clients for requested AI/ML operations.

The following clauses specify procedures, information flows, and APIs for AIMLE client registration.

### 8.7.2	Procedure

Pre-conditions:

1.	The AIMLE client has been pre-configured or has discovered the address (e.g., URI) of the AIMLE server.

2.	The AIMLE client has been pre-configured with an AIMLE client profile.

Figure 8.7.2-1: AIMLE client registration

1.	The AIMLE client sends an AIMLE client registration request to the AIMLE server, the registration request includes information as described in Table 8.7.3.1-1. The AIMLE client indicates in the registration request its AI/ML capabilities such as supported ML model types and supported AI/ML operations, supported AI/ML client task capabilities to assist with performing AIMLE client discovery and AIMLE client selection.

2.	The AIMLE server validates the registration request and performs an authentication and authorization check to determine if the AIMLE client is permitted to register to the AIMLE server and participate in AI/ML operations. The AIMLE server saves the context of the AIMLE client registration in the ML repository.

3.	The AIMLE server returns an AIMLE client registration response to the AIMLE client with the status of the request.

### 8.7.3	Information flows

#### 8.7.3.1	AIMLE client registration request

Table 8.7.3.1-1 shows the request sent by an AIMLE client to an AIMLE server for the AIMLE client registration procedure.

Table 8.7.3.1-1: AIMLE client registration request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| AIMLE client profile | M | Information about the capability of the AIMLE client to support AI/ML operations as detailed in Table 8.7.3.1-2. |
| List of supported services | O | Supported service information. |
| > List of service ID(s) | O | List of service ID(s) that the AIMLE client supports |
| > Corresponding service permission level(s) | O | Service permission level(s) (e.g., premium resource usage, standard resource usage, limited resource usage). |

Editor’s Note: The addition of AIMLE client profile ID is FFS.

Table 8.7.3.1-2: AIMLE client profile

| Information element | Status | Description |
| --- | --- | --- |
| Supported AI/ML model types | O | AI/ML model types supported by the AIMLE client (e.g., decision trees, linear regression, neural networks). |
| Supported AI/ML operations | M | AI/ML operations supported by the AIMLE client such as training. |
| AIML client schedule | O | Indicates the Availability schedule of the AIMLE client for the AIML service |
| Application layer AIMLE client capabilities | M | Application layer AIMLE client capability information (e.g. ML application type, allowed resource usage level). |
| Dataset availability | O | Dataset availability such as dataset size, age, list of dataset features, and dataset identifiers. |
| Data capabilities | O | A list of data capabilities such as the type of data that can be collected (e.g. raw data), supported data processing capabilities (e.g. processed data), and supported exploratory data analysis functions. |
| AIML client task capability | O | Indicates the AIML task performing capabilities. It includes AIML task type(e.g. model transfer, model training, model inference, model offload, model split), AIML task compute requirements( e.g. high compute, low compute), AIML task performance criteria preference includes Green task performance(e.g. usage of renewable energy for AIML task like solar powered), Energy-efficient task performance like for reduced energy-consumption of AIML task (e.g. low carbon footprint) , low computational costs task performance for cost efficient operation of AIML tasks) |

#### 8.7.3.2	AIMLE client registration response

Table 8.7.3.2-1 shows the response sent by the AIMLE server to the AIMLE client for the AIMLE client registration procedure.

Table 8.7.3.2-1: AIMLE client registration response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |

* * * Next Change * * * *

## 8.8	AIMLE client discovery

### 8.8.1	General

Discovery of AIMLE clients is an important step in the AI/ML process for distributed, federated, split AI/ML, and transfer learning. Due to the nature of such learning, VAL servers need to discover suitable AIMLE clients to fulfil the requirements for the AI/ML application. The VAL server can then use the discovered AIMLE clients to select a set of AIMLE clients to perform AI/ML operations.

The following clauses specify procedures, information flows, and APIs for AIMLE client discovery.

### 8.8.2	Procedure

#### 8.8.2.1	AIMLE client discovery

Pre-conditions:

1.	AIMLE clients that support AI/ML operations have registered with the AIMLE server and included their AIMLE client profiles and optionally a list of supported services.

2.	The AIMLE server can access a ML repository to obtain AIMLE client profiles and supported services associated with AIMLE clients.

Figure 8.8.2.1-1: AIMLE client discovery

1.	A VAL server sends an AIMLE client discovery request to an AIMLE server to discover a list of AIMLE clients that are available to participate in AI/ML operations (e.g., is available and have required data to train an ML model). The request may also include AIML task container requirements to discover clients who can perform the AIML tasks like AIML model training/offload/split. The AIMLE client discovery request includes information as described in Table 8.8.3.1-1.

2.	The AIMLE server performs authentication and authorization checks to determine if the requestor is able to discover AIMLE clients. If the requestor is authorized, the AIMLE server performs the following to discover AIMLE clients fulfilling the provided AIMLE client discovery criteria.

The AIMLE server also obtains candidate UEs from the ML repository.

Editor’s Note: Whether Member UE filtering criteria is included in the request is FFS.

From the list of candidate UEs, the AIMLE server discovers a list of UEs (i.e., AIMLE clients) that fulfils the discovery criteria based on AIMLE client profiles and AIMLE client discovery criteria.

Editor’s Note: Whether AI/ML Policy IDs is included in the request is FFS.

3.	The AIMLE server sends an AIMLE client discovery response that includes information in Table 8.8.3.2-1. The response also includes the information related to supported tasks with guaranteed task KPIs, available AIML tasks service duration, AIML task expected KPI like latency.

### 8.8.3	Information flows

#### 8.8.3.1	AIMLE client discovery request

Table 8.8.3.1-1 shows the request sent by a VAL server to an AIMLE server for the AIMLE client discovery procedure.

Table 8.8.3.1-1: AIMLE client discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor (e.g., VAL server). |
| Service identifier | O | An identifier for the service associated with the requestor. |
| AIMLE client discovery criteria | M | Discovery criteria for finding suitable AIMLE clients for AI/ML operations as detailed in Table 8.8.3.1-2. |

Table 8.8.3.1-2: AIMLE client discovery criteria

| Information element | Status | Description |
| --- | --- | --- |
| Requested ML model types | O | Requested ML model types (decision trees, linear regression, neural networks). |
| Requested AIML operations | M | Requested role for AI/ML operations such as training. |
| Application layer AIMLE client capabilities | M | Application layer AIMLE client capability information (e.g., ML application type like FL/TL/SL, client availability to support AIML operations at the UE, AIMLE drop off rate). |
| Dataset requirements | O | Information about dataset. |
| > Dataset availability | O | Dataset availability, including dataset identifiers, dataset size, age, list of dataset features. |
| > Data capabilities | O | A list of data capabilities such as the type of data that can be collected (e.g., raw data), supported data processing capabilities (e.g., processed data) and supported exploratory data analysis functions. |
| Required service | O | Information about the service. |
| > Required service ID | O | Required service ID that the client supports. |
| > Required corresponding service permission level | O | Required corresponding service permission level (e.g., premium resource usage standard resource usage, limited resource usage). |
| AIML task container requirements | O | Indicates the AIML task requirements to discover the AIML clients for performing AIML tasks. It includes AIML task type(e.g. model transfer, model training, model inference, model offload, model split), AIML task compute requirements( e.g. high compute, low compute), AIML task performance criteria preference includes Green task performance(e.g. usage of renewable energy for AIML task like solar powered), Energy-efficient task performance like for reduced energy-consumption of AIML task (e.g. low carbon footprint) , low computational costs task performance for cost efficient operation of AIML tasks) |
| AIMLE client velocity | O | Indicates the AIMLE client velocity. E.g. high, low, stationary |
| Location information | O | Indicates the location information to discover the AIMLE clients |
| AIMLE client QoS requirements | O | Indicates the AIMLE client QoS information (like PLR, bandwidth, latency jitter) to discover the AIMLE clients |

Editor’s Note: Whether target area for data collection is included in the AIMLE client discovery criteria is FFS.

#### 8.8.3.2	AIMLE client discovery response

Table 8.8.3.2-1 shows the response sent by the AIMLE server to the VAL server for the AIMLE client discovery procedure.

Table 8.8.3.2-1: AIMLE client discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |
| List of AIMLE client IDs | M | A list of AIMLE client IDs that matches the AIML discovery criteria. |
| AIML supported tasks | O | Indicates the discovered AIML task providers fulfilling the AIML task and AIML task requirements. It includes expected AIML task KPIs like latency, available AIML tasks service duration, list of AIML task providers like IDs or URI, and supported tasks. |

* * * End of Changes * * * *