3GPP TSG-SA WG6 Meeting #63	S6-244652

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244140, 4544)

Source:	Nokia, ZTE, Samsung

Title:	Solution on UE-deployed API invoker accessing resources not owned by that UE

Spec:	3GPP TR 23.700-22 v1.0.0

Agenda item:	8.7

Document for:	Approval

Contact:	Diego Preciado (diego.preciado_rojas@nokia.com)

1. Introduction

This contribution provides a proposal for solution for KI#6 on UE-deployed API invoker accessing resources not owned by that UE.

2. Reason for Change

According to clause 7.5 in 3GPP TS 23.222, an API invoker may be deployed in any of the following ways:

a)	API invoker may be deployed as AF on the UE (i.e. 3rd party application).

b)	API invoker may be deployed as AF on the UE supporting several other 3rd party applications deployed on the UE.

c)	API invoker may be deployed on the network as AF.

As from clause 4.17.1 in 3GPP TS 23.222, (for Release 18) the scope of an API invoker on a UE (i.e., options a and b) in Resource owner-aware northbound API access (RNAA) is limited to accessing its own resources only, i.e., resource owner is a user of the UE hosting the API invoker that can authorize the API access. This is also acknowledged by SA3 in clause 6.5.3 in 3GPP TS 33.122, in which “only a UE accessing its own resources is considered if the API invoker is on a UE.”

However, there are cases in which it is needed to support for API invoker(s) which are deployed on a UE accessing resources of other UEs, e.g., consider the clause 6.2.2 in 5GAA - C-V2X Use Cases and Service Level Requirements Volume I, dealing with Vehicle Health Monitoring in fleet management, in which an Application Client on UE 1 could request access to fetch location and/or vehicle health issues to another user (UE 2).

3. Proposal

It is proposed to agree the new key issue for 3GPP TR 23.700-22 V1.0.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[3]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[4]	3GPP TS 22.261: "Service requirements for the 5G system".

[5]	IETF draft-ietf-oauth-browser-based-apps-17: "OAuth 2.0 for Browser-Based Apps".

NOTE:	The IETF work in "draft-ietf-oauth-browser-based-apps-17 is in draft stage.

[6]	IETF 6749: "The OAuth 2.0 Authorization Framework".

[7]	3GPP TR 28.849: "Study on charging aspects of Common API Framework for Northbound APIs (CAPIF) phase 2".

[8]	3GPP TS 29.122: "T8 reference point for northbound Application Programming Interfaces (APIs)".

[refSEAL]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

* * * Next Change * * * *

## 6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  | X |  |  |  |  |
| Sol #2 | X |  |  |  |  |  |  |
| Sol #3 | X |  |  |  |  |  |  |
| Sol #4 |  |  |  | X |  |  |  |
| Sol #5 |  |  |  |  | X |  |  |
| Sol #6 |  |  |  |  |  |  | X |
| Sol #7 |  |  |  |  |  |  | X |
| Sol #8 |  |  |  |  |  |  | X |
| Sol #9 | X |  |  |  |  |  |  |
| Sol #10 | X |  |  |  |  |  |  |
| Sol #11 | X |  |  |  |  |  |  |
| Sol #12 | X |  |  |  |  |  |  |
| Sol #13 | X |  |  |  |  |  |  |
| Sol #14 | X |  |  |  |  |  |  |
| Sol #15 | X |  |  |  |  |  |  |
| Sol #16 | X |  |  |  |  |  |  |
| Sol #17 |  |  |  |  | X |  |  |
| Sol #Y |  |  |  |  |  | X |  |

* * * Next Change * * * *

## 6.z	Solution #Y: UE-deployed API invoker accessing resources not owned by that UE

### 6.z.1	Solution description

#### 6.z.1.1	General

As from clause 4.17.1 in 3GPP TS 23.222 [2], the scope of an API invoker on a UE in Resource owner-aware northbound API access (RNAA) is limited to accessing its own resources only, i.e., resource owner is a user of the UE hosting the API invoker that can authorize the API access. This is also acknowledged by SA3 in clause 6.5.3 in 3GPP TS 33.122 [3], in which “only a UE accessing its own resources is considered if the API invoker is on a UE.”

However, there are cases as per key issue#6 in which it is needed to support for API invoker(s) which are deployed on a UE accessing resources of other UEs, e.g., consider the clause 6.2.2 in 5GAA - C-V2X Use Cases and Service Level Requirements Volume I, dealing with Vehicle Health Monitoring in fleet management, in which an Application Client on UE 1 could request access to fetch location and/or vehicle health issues to another user (UE 2).

#### 6.z.1.2	UE-deployed API invoker accessing other UEs’ resources

Figure 6.z.1.2-1 presents the procedure for enabling a UE-hosted API invoker accessing network-hosted resources owned by other UEs.

Pre-condition:

- The network hosts personal resources for a group of UEs which can be accessed via AEF provided the right security conditions are met.

- UE 1 and UE 2 belong to a same group, and both are aware of the group identifier.

- CCF has connectivity to UDM/UDR and a SEAL-GMS instance.

- ROF at UE1 has been authorized (e.g. by group policy) by group members to be the GRO for the group.

Figure 6.z.1.2-1: UE-deployed API invoker accessing other UEs’ resources

- A group is created on SEAL-GMS as indicated in clause 10.3.3 in 3GPP TS 23.434 [refSEAL]. The Group creation request message shall carry information about the identity of a Group Resource Owner (GRO).

- The API invoker (e.g., in UE2) sends an Obtain service API authorization request to the CCF for obtaining permission to access the service API (and other UEs’ resources hosted in the network – e.g., location) by including the API invoker identity information and any information required for authentication of the API invoker. The request may include an identifier of the group of UEs whose resources are trying to be accessed. Additionally, the API invoker may include in the request information about the Purpose and operations (scope) to be performed on the targeted resources.

- CCF validates the authentication of the API invoker (using authentication information) as specified in 3GPP TS 33.122.

- CCF will retrieve the identity of the GRO using the Group Information query procedure (as in clause 10.3.4 in 3GPP TS 23.434 [refSEAL]). The UE hosting the API invoker (i.e., UE 2) must be a member of the group, if that is not the case, the authorization request is rejected.

- If Consent needs to be captured (e.g., due to the service API invocation will imply processing Personal Identifiable Information), the CCF triggers the capture of the Consent with each group member via CAPIF-8 (e.g. from GRO).

NOTE:	The authorization details for step 5 will be specified by SA3.

- If the GRO granted the Consent for accessing personal information resources on behalf of the group members, the authorization information to access the service APIs is sent to the API invoker in the obtain service API authorization response.

- The service API invocation takes place considering the authorization information received in the previous step.

### 6.z.2	Architecture Impacts

Communication between CCF and SEAL-GMS is expected to take place, so an instance of GM-S reference point shall be present.

### 6.z.3	Corresponding APIs

NOTE:	Whether new or enhanced APIs are required in support of this solution is in scope of SA3.

### 6.z.4	Solution evaluation

This solution addresses the following open issue in KI#6:

1.	Whether (and how) RNAA can support the scenario where API invoker(s) which are deployed on the UE can access resources (hosted in the network) of other resource owners (users) (e.g., application client on UE is fetching location of another UE or setting QoS for PDU sessions of another UE)

This solution provides a feasible solution for capturing user consent from a delegated RO for a group of users (the so-called Group Resource Owner - GRO). The GRO may authorize access to protected resources on behalf of the group members, depending on the Purpose and scope. A group need to be created on the SEAL-GMS containing information about the identity of the GRO. This procedure can be applied to API invokers deployed in the network.

* * * End of Changes * * *