3GPP TSG-SA WG6 Meeting #63	S6-244687

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244655, S6-244565,

S6-244184)

Source:	Samsung, InterDigital

Title:	Pseudo-CR on enhancements to spatial anchor subscription procedure

Spec:	3GPP TS 23.437 (v0.2.0)

Agenda item:	9.11

Document for:	Approval

Contact:	Sapan Shah (sapan.shah@samsung.com)

1. Introduction

This pCR enhances the spatial anchor subscription procedure.

2. Reason for Change

1) Clause 8.3.5.2.1 - For "spatial anchor in range" event notification, server needs to monitor location and pose along with the direction. Added clarification for the same.

2) Clause 8.3.5.2.1 - The event "spatial anchor in range" is about notifying spatial anchors coming in-range or out of range of the UE. The text was reverse. Corrected it.

2) Clause 8.3.5.3.1 - For "spatial anchor in range" event, indication to enable persistent search is missing in the IE table. So added it.

3) Clause 8.3.5.3.1 - Added discovery filters for "spatial anchor in range" event.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.437 (v0.2.0).

* * * First Change * * * *

##### 8.3.5.2.1	Subscribe

Figure 8.3.5.2.1-1 illustrates the spatial anchor subscription procedure.

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server; and

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

Figure 8.3.5.2.1-1: Spatial anchor subscription

1.	The requestor (e.g., SAn client or VAL server) sends a spatial anchor subscribe request to the SAn server. The request includes a requestor identifier, security credentials, a notification endpoint, and event information for which the requestor is subscribing. The request may include an expiration time.

a.	The requestor may include the "discovery" event type to indicate the SAn server to notify the requestor when the SAn server detects events related to the management of spatial anchors (e.g., creation, update, deletion) according to discovery event filters and spatial anchor filters provided in the request. If the requestor is a SAn client, the request includes a VAL client identifier.

b.	The VAL server may include the "UE with spatial anchor interest" event type to indicate the SAn server to notify the VAL server when the SAn server detects UE(s) with interest in specific spatial anchors according to the spatial anchor of interest event filters and spatial anchor filters provided in the request.

c.	The requestor may include the "spatial anchor in range" event type to indicate the SAn server to notify the requestor when the SAn server detects that spatial anchors become in range or out of range of the subscriber according to range event filters provided in the request. Range event filters can include a range for detecting spatial anchors, an indication to enable persistent search, the pose of the subscriber. If the requestor is a SAn client, the spatial anchor subscribe request includes a VAL client identifier.

2.	Upon receiving the request from the requestor, the SAn server validates if the requestor is authorized for the request. If the requestor is authorized, the SAn server creates the subscription and stores the subscription information.

a.	If the subscribed event is for "discovery", the SAn server starts monitoring spatial anchors management events according to the discovery event filters and spatial anchor filters.

b.	If the subscribed event is for "UE with spatial anchor interest", the SAn server starts monitoring spatial anchors of interest according to the UE with spatial anchor interest event filters and spatial anchor filters. If the spatial anchors of interest have been discovered by a UE, the SAn server can obtain the UE location information from the 5GS or the SEAL Location Management service. Additionally, the SAn server can consider data analytics, such as UE mobility prediction. The SAn server may monitor the distance between UEs and the spatial anchor of interest to determine whether the UEs enters, exits or is within range of the spatial anchors of interest.

c.	If the subscribed event is for "spatial anchor in range" and the requestor is a SAn client, the SAn server starts monitoring the corresponding UE location and direction corresponding to the pose information according to the range event filters and spatial anchor filters. The UE location information can be obtained by using capabilities of the 5GS or the SEAL Location Management service. Additionally, the SAn server can consider data analytics, such as UE mobility prediction. The SAn server monitors the distance between UEs and the spatial anchor of interest to determine whether the spatial anchors becomes in-range or out-of-range of the UE. If persistent search is enabled, the SAn server monitors location of the user (or UE) and pose of the user, along with a direction of movement, and discovers a list of spatial anchors which becomes in-range or out of range of the user device based on the location of the user device, the pose of the user and in the user's field of view.

3.	The SAn server sends a spatial anchor subscribe response to the requestor. If the SAn server has created the subscription, the response includes an indication of success, the subscription identifier and may include an expiration time; to maintain the subscription, the requestor shall send a subscription update request before the expiration time, otherwise the spatial anchor subscription expires. If the SAn server has not created the subscription, the response includes an indication of failure and may include a reason for failure.

##### 8.3.5.2.2	Notify

Figure 8.3.5.2.2-1 illustrates the spatial anchor notify operation between the SAn server and a SAn client or VAL server.

Pre-conditions:

1.	The SAn client or VAL server has subscribed for spatial anchor notifications with the SAn server;

Figure 8.3.5.2.2-1: Spatial anchor notification

1.	An event occurs at the SAn server that satisfies trigger conditions for notifying a subscriber (e.g., SAn client or VAL server) according to subscribed events and discovery filters provided for the events.

2.	The SAn server sends a spatial anchor notification to the requestor indicating the event. The notification includes a subscription identity and includes event-specific information.

a.	If the subscribed event is for "discovery", the notification includes the list of determined spatial anchors

b.	If the subscribed event is for "UE with spatial anchor interest", the notification includes information about UE with spatial anchor interest (e.g., UE identifier, spatial anchor identifier(s), range from the spatial anchor(s), entering or exiting the range, etc.).

c.	If the subscribed event is for "spatial anchor in range", the notification includes the list of determined spatial anchors that become in range, out of range of the subscriber.

* * * Next Change * * * *

##### 8.3.5.3.1	Spatial anchor subscribe request

Table 8.3.5.3.1-1 describes information elements for the spatial anchor subscribe request from the SAn client or VAL server to the SAn server.

Table 8.3.5.3.1-1: Spatial anchor subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| VAL client ID | O | The identifier of the VAL client when the requestor is a SAn client. |
| Notification Target Address | M | Notification target address (e.g. URL) where the notifications should be sent. |
| Proposed expiration time | O | Proposed expiration time for the subscription. |
| Event type | M | The type of event for which the requestor is subscribing. Supported event types include: - "discovery" - "UE with spatial anchor interest" - "spatial anchor in range" |
| Persistent Search (NOTE) | O | Indicates to enable persistent search. |
| Event detection filters | O | Event type specific detection filters. |
| > Discovery event filters | O | Event filters applicable to the "discovery" event type. The event filter is considered by the SAn server for triggering the notification and can include management events (e.g., creation, update, deletion). |
| > UE with spatial anchor interest event filters | O | Event filters applicable to “UE with spatial anchor interest” event type. The event filter is considered by the SAn server for triggering the notification and can include an indication for detecting UEs that have discovered spatial anchors, that are within range or become out of range of spatial anchors, or that enter or leave the range of a spatial anchor, and a value for the range. |
| > Range event filters | O | Event filters applicable to “spatial anchor in range” event type. The event filter is considered by the SAn server for triggering the notification and can include an indication for spatial anchors becoming in range of the UE, out of range of the UE, or both and a value for the range. |
| >> current location | O | Current location of the user and direction. |
| >> pose | O | Current pose of the user |
| >> application specific range | M | Specifies range within which the spatial anchors are monitored to be in range or out of range of the subscriber's field of view. |
| >> Spatial anchor specific information | O | A container to contain application specific information. |
| Spatial anchor filters | O | The characteristics of spatial anchors that the SAn server considers for triggering a notification. |
| > Spatial anchor ID list | O | Identifiers of the spatial anchors of interest. |
| > Service ID list | O | Identifiers of the associated application services of interest. |
| > Area of interest | O | The area of interest (e.g., topological and/or geographical). |
| NOTE: This IE can be included with "spatial anchor in range" event. |  |  |

* * * Next Change * * * *

<Proposed change in revision marks>