3GPP TSG-SA WG6 Meeting #64	S6-245037

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245xxx)

Source:	NTT DOCOMO

Title:	Pseudo-CR on addition of AIMLE client deregistration procedure in TS23.482

Spec:	3GPP TS 23.482 v0.3.0

Agenda item:	9.10

Document for:	Approval

Contact:	yuushin.hayashi.ew@nttdocomo.com

1. Introduction

This addition outlines a new procedure about AIMLE client deregistration.

2. Reason for Change

The current method does not describe AIMLE client deregistration procedure. Therefore, the new approach should be proposed.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 v0.3.0.

* * * First Change * * * *

8.X	AIMLE client deregistration

8.X.1	General

After an AIMLE client is registered with the AIMLE server, there may be situations where it is necessary to deregister for various reasons. The AIMLE client presents the reason for deregistration and the schedule to the AIMLE server and sends it.

The following clauses specify procedures, information flows, and APIs for AIMLE client deregistration.

8.X.2	Procedure

Pre-conditions:

1.	The AIMLE client has completed the AIMLE server discovery procedure.

Figure 8.X.2-1: AIMLE client deregistration

1.	The AIMLE client sends an AIMLE client deregistration request to the AIMLE server. This deregistration request includes information as described in Table 8.X.3.1-1. The AIMLE client specifies in the deregistration request the registered ML model types and the schedule for when the deregistration will occur.

2.	The AIMLE server validates the deregistration request and performs an authentication and authorization check to determine whether the AIMLE client can deregister from AI/ML operations. The AIMLE server saves the context of the AIMLE client deregistration in the ML repository.

3.	The AIMLE server returns an AIMLE client deregistration response to the AIMLE client with the status of the request.

8.X.3	Information flows

8.X.3.1	AIMLE client deregistration request

Table 8.X.3.1-1 shows the request sent by an AIMLE client to an AIMLE server for the AIMLE client deregistration procedure.

Table 8.X.3.1-1: AIMLE client deregistration request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| List of supported services | O | Supported service information. |
| > List of service ID(s) | O | List of service ID(s) that the AIMLE client supports |
| > Corresponding service permission level(s) | O | Service permission level(s) (e.g., premium resource usage, standard resource usage, limited resource usage). |
| Deregistration schedule | M | Information regarding when deregistration is desired. |

8.X.3.2	AIMLE client deregistration response

Table 8.X.3.2-1 shows the response sent by the AIMLE server to the AIMLE client for the AIMLE client deregistration procedure.

Table 8.X.3.2-1: AIMLE client deregistration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the ML model retrieval request was successful. |
| Failure response | O | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |

* * * End of Change * * * *