3GPP TSG-SA WG6 Meeting #64	S6-245181

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245xxx)

Source:	Samsung

Title:	Pseudo-CR on AIMLApp_enhancements to ML model management

Spec:	3GPP TS 23.482 (v0.3.0)

Agenda item:	9.10

Document for:	Approval

Contact:	Arunprasath R, arun.prasath@samsung.com

1. Introduction

This pCR resolves the EN related to model discoverability.

2. Reason for Change

Regarding follwing EN:

Editor’s Note 2: How to consider whether model is discoverable or not is FFS.

- The VAL server may requried services related to storage and management of the model from the AIMLE layer, however, the VAL server may not want the model to be discoverabel for the transfer of learning to other servers (due to various reasons e.g. security related policy). When VAL server request to store the model, it may indicate the AIMLE server to not to inlcude this model in the model discovery response for transfer of learning.

Regarding following EN:

Editor’s Note:	More parameters for ML model phase is FFS.

In previous meeting, SA6 agreed to the assistance infomration where delivery time indicating time to deliver the task is agreed. We need to add similar time as observed delivery time in ML model information.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 (v0.3.0).

* * * First Change * * * *

### 8.11.3	ML model information discovery

Figure 8.11.3-1 illustrates the procedure of ML model information discovery.

Pre-condition:

1.	Either AIMLE consumer has requested to discover ML model or AIMLE server decides to discover model for training.

Figure 8.11.3-1: ML model information discovery

1.	The AIMLE server sends an ML model information discovery request to the ML repository. The request contains information elements as described in Table 8.11.4.3-1.

2.	Upon receiving the ML model information discovery request, the ML repository verifies if the requestor is authorized to discover the ML model(s) identified by the ML model ID. Further, it verifies whether the requestor is present in the list of allowed vendors or not. If the ML repository consumer is authorized, the ML repository processes the request and discovers the model based on ML model ID, Analytics ID, Base Model ID, and/or ML model interoperability information.

If the discovery request is for transfer of learning, the ML repository identifies all candidate models matching the domain and required accuracy level.

The ML repository sends the response message to the ML repository consumer. The response includes information of the discovered ML model. Optionally, the ML repository can initiate a model transfer process to the consumer.

NOTE 1:	When the ML model is within training or re-training phase, the ML model is not available for discovery. The details is indicated within the Result IE in Table 8.11.4.4-1. The training and re-training phase is used during federated learning so that AIML enabler consumer can rely on model repository to get the model details.

NOTE 2:	The model transfer process mentioned in step 2 can use the AI/ML data transfer policies.

Editor’s Note 1: If List of allowed vendors is not present then how to get consent from model owner to share the model to the requestor is FFS.

* * * Next Change * * * *

#### 8.11.4.1	ML model information storage request

Table 8.11.4.1-1 describes the information flow from the AIMLE server to the ML repository as a request for the ML model information storage.

Table 8.11.4.1-1: ML model information storage request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the ML repository consumer performing the request. |
| ML model | O (NOTE) | Provides the ML model. |
| ML model address | O (NOTE) | Represents the ML model address that can be used by Model repository to download the ML model. |
| ML model information | M | Provides information of the ML model, as described in Table 8.11.4.1-2. |
| NOTE: At least one of these information elements shall be provided. |  |  |

Table 8.11.4.1-2: ML model information

| Information element | Status | Description |
| --- | --- | --- |
| ML model identifier | M | An identifier for the ML model |
| Analytics ID | O | Represents analytics ID for which the model can be used. |
| VAL service ID(s) | O | Identify the VAL service ID(s). |
| Domain | O | Specifies domain for which the model can be used (e.g., for speech recognition, image recognition, video processing, location predication, etc.). |
| Discoverable | O | Indicates whether model is allowed to be discovered by other applications/vendors. |
| List of allowed vendors | O (NOTE 1) | Indicates which vendors that are allowed to use the ML model and thereby also are interoperable to the model. |
| ML model interoperability information | O (NOTE 1) | Represents the vendor-specific information that conveys, e.g., requested model file format, model execution environment, etc. The encoding, format, and value of ML Model Interoperable Information is not specified since it is vendor specific information, and is agreed between vendors, if necessary for sharing purposes. |
| ML Model phase | O (NOTE 1) | Represents the ML model phase, e.g., in training, trained, re-training, deployed. |
| > Observed accuracy | O (NOTE 2) | Provides accuracy of the model (if ML model is in trained phase). |
| > Observed delivery time | O | Provides time to execute model and deliver the task |
| ML model storage and discovery requirements | O (NOTE 1) | Represents the requirements for the ML repository for the ML model storage and discovery. |
| > Storage duration | O | Represents the ML model storage duration time. When the storage duration time is expired, the stored ML model and the related information shall be deleted. |
| > Security and access requirements | O | Represents the information on security requirements for storing the ML model information and the ML model access requirements (e.g., publicly available, private use only, or available for the list of VAL server IDs). |
| NOTE 1: At least one of these information elements shall be provided. NOTE 2: This IE is included only if trained ML model is available. |  |  |

* * * End of Change * * * *

<Proposed change in revision marks>