3GPP TSG-SA WG6 Meeting #64	S6-245212

Orlando, USA, 18th – 122nd November 2024	(revision of S6-245xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.180 | CR | 0013 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update MBMS transmissions aspect |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Generic_IOPS |  |  |  |  |  | Date: |  |  | 2024-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Base on the conclusion in TR 23.700-09, the solution #9 is agreed for the normative work. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the clause 10.4 based on solution#9. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The IOPS application is not applicable for any access, e.g., 5GS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4, 10.4.1, 10.4.2, 10.4.2.1, 10.4.2.2, 10.4.3, 10.4.4, 10.4.5, 10.4.5.1, 10.4.5.2, 10.4.5.2.1, 10.4.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /*********************************** First changes ******************************/

## 10.4	Use of broadcast/multicast transmissions

### 10.4.1	General

An IOPS MC connectivity function can provide support for group communications based on broadcast/multicast transmissions. Broadcast/multicast transmissions can be used for the downlink delivery of MC service application signalling and media to the MC service UEs over broadcast/multicast session (e.g., MBMS bearers in EPS, MBS session in 5GS).

The use of broadcast/multicast transmissions in the IOPS mode of operation are based on the related information flows and procedures specified in 3GPP TS 23.280 [3], 3GPP TS 23.379 [5], 3GPP TS 23.282 [6], and 3GPP TS 23.289 [23289].

### 10.4.2	Information flows

#### 10.4.2.1	MapIOPSGroupToNetworkSession (IP connectivity functionality)

Table 10.4.2.1-1 describes the information flow to associate a group session based on the IP connectivity functionality to a broadcast/multicast session. It is sent from the IOPS MC connectivity function to the MC service client.

Table 10.4.2.1-1: MapIOPSGroupToNetworkSession (IP connectivity functionality)

| Information element | Status | Description |
| --- | --- | --- |
| IOPS MC service group ID | M | This element identifies the IOPS MC service group, in which the group session is started. |
| Media stream identifier | M | This element identifies the media stream of the SDP used for the group session (e.g. MBMS subchannel). |
| Identifier of broadcast/multicast session (NOTE) | O | The identifier of the network broadcast/multicast session (e.g., TMGI) if the media of the group session is not sent on the same broadcast/multicast session as this MapIOPSGroupToNetwork message. |
| Session acknowledgement indicator | O | Indication that the group session requires acknowledgement from receiving MC service clients. |
| NOTE: “Identifier of broadcast/multicast session” IE shall be present if this message is sent over a different broadcast/multicast session than the media of the group session; This IE may be present if this message is sent over the same broadcast/multicast session as the media of the group session. |  |  |

#### 10.4.2.2	UnmapIOPSGroupFromNetworkSession (IP connectivity functionality)

Table 10.4.2.2-1 describes the information flow to disconnect a group session based on the IP connectivity functionality from a broadcast/multicast session. It is sent from the IOPS MC connectivity function to the MC service client.

Table 10.4.2.2-1: UnmapIOPSGroupFromNetworkSession

| Information element | Status | Description |
| --- | --- | --- |
| IOPS MC service group ID | M | This element identifies the IOPS MC service group, in which the group session is started. |

### 10.4.3	Use of pre-established broadcast/multicast session

The MC services in the IOPS mode of operation shall support the procedure for using pre-established broadcast/multicast session as specified 3GPP TS 23.280 [3] or 3GPP TS 23.289 [23289] with the following clarification:

-	The MC service server is the IOPS MC connectivity function.

The MCPTT service shall use the MCPTT-IOPS-2, MCPTT-IOPS-4, and MC-IOPS-6 reference points for this procedure.

The MCData service shall use the MCData-IOPS-2, MCData-IOPS-4, and MC-IOPS-6 reference points for this procedure.

The IOPS MC system may use pre-established broadcast/multicast sessions for distributing the media associated to group sessions. Depending on the capacity of the broadcast/multicast session, the broadcast/multicast sessioncan be used to broadcast one or more group sessions in parallel.

### 10.4.4	Use of dynamic broadcast/multicast session establishment

The MC services in the IOPS mode of operation shall support the procedure for using dynamic broadcast/multicast session as specified 3GPP TS 23.280 [3] or 3GPP TS 23.289 [23289] with the following clarification:

-	The MC service server is the IOPS MC connectivity function.

The MCPTT service shall use the MCPTT-IOPS-2, MCPTT-IOPS-4, and MC-IOPS-6 reference points for this procedure.

The MCData service shall use the MCData-IOPS-2, MCData-IOPS-4, and MC-IOPS-6 reference points for this procedure.

The IOPS MC system may use dynamic MBMS bearer establishment for distributing the media associated to group sessions.

### 10.4.5	Group session connect and disconnect over broadcast/multicast session (IP connectivity functionality)

#### 10.4.5.1	General

MBMS bearers can be used for group sessions based on the IP connectivity functionality. One broadcast/multicast session can be associated to one or more specific groups or group sessions. Before sending packets related to a group session, i.e. signalling and media, over an broadcast/multicast session, the IOPS MC connectivity function transmits the association information between the group session and the broadcast/multicast session. When the group session over the broadcast/multicast session is finished, this temporary association information of a group session to specific resources on a broadcast/multicast session is undone. Prior to the association of a group session to a broadcast/multicast session the broadcast/multicast session is activated and announced to the MC service clients.

#### 10.4.5.2	Procedure

##### 10.4.5.2.1	Group Session connect over broadcast/multicast session

In figure 10.4.5.2.1-1 the MC service client 1 is the client that initiates a group session, e.g. an MCPTT client initiating a group call. The MC service client 1 may be within the broadcast/multicast  service area. The MC service clients 2 … n represent MC service clients, e.g. MCPTT clients, receiving the group session packets over an MBMS bearer. There may be other receiving MC service clients both over unicast bearers and over this or other broadcast/multicast session(s), however, they are not illustrated in this figure.

Pre-conditions:

-	The MC service user profile used for the IOPS mode of operation is pre-provisioned in the MC service UEs

-	The IOPS MC service group ID and its associated IOPS group IP multicast address are pre-configured in the MC service clients

-	The MC service users have an active PDN connection to the IOPS MC connectivity function for the communication based on the IP connectivity functionality

-	The MC service users are discovered by the IOPS MC connectivity function to support MC services based on the IP connectivity functionality

Figure 10.4.5.2.1-1: Group session connect on broadcast/multicast session (IP connectivity functionality)

1.	Activation and announcement of broadcast/multicast session availability.

2.	MC user at the MC service client 1 wants to initiate an MC service group session with a specific IOPS MC service group based on the IP connectivity functionality.

3.	The MC service client 1 transmits the related group session packets via the IOPS MC connectivity function targeting the IOPS MC service group (i.e. using the corresponding IOPS group IP multicast address)

4.	The IOPS MC connectivity function determines that the received packets correspond to a group session. Therefore, the IOPS MC connectivity function transmits a MapIOPSGroupToNetwork message over a previously established broadcast/multicast session to all users monitoring the broadcast/multicast session. The MapIOPSGroupToNetwork message includes association information between the group session and the broadcast/multicast session. It includes the corresponding IOPS MC service group ID associated to the IOPS group IP multicast address related to the group session.

5.	The IOPS MC connectivity function distributes the received group session packets from the MC service client 1 to other MC service clients (e.g. MC service clients 2 … n) over the broadcast/multicast session.

NOTE:	MC service clients receiving the group session packets over the broadcast/multicast session only decode the packets that are addressed to their pre-configured IOPS group IP multicast address(es). Otherwise, the MC service clients discard the packets.

##### 10.4.5.2.2	Group session disconnect over broadcast/multicast session

Figure 10.4.5.2.2-1 shows the high-level procedure where an UnmapIOPSGroupFromNetworkSession message is transmitted from the IOPS MC connectivity function to MC service clients to indicate that a group session is being disconnected or dissociated from the broadcast/multicast session.

Figure 10.4.5.2.2-1: Group session disconnect on broadcast/multicast session (IP connectivity functionality)

1.	A group session based on the IP connectivity functionality is ongoing; the group session packets received from the MC service client 1 are distributed over broadcast/multicast session to other MC service clients within the system.

2.	The IOPS MC connectivity function has determined to disconnect the group session from the broadcast/multicast session for the MC service clients within the system.

NOTE:	The IOPS MC connectivity function may decide to disconnect a group session from a broadcast/multicast session if group session packets are not anymore received addressing the corresponding IOPS MC service group (or IOPS group IP multicast address).

3.	An UnmapGroupFromBearer message is transmitted from the IOPS MC connectivity function to the MC service clients and possibly to the MC service client 1 (if it is within the broadcast/multicast coverage area) on broadcast/multicast session(s).