3GPP TSG-SA WG6 Meeting #64	S6-245239

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245xxx)

Source:	Huawei, Hisilicon

Title:	AIML model storage enhancement

Spec:	3GPP TS 23.482 v0.3.0

Agenda item:	8.9

Document for:	Approval

Contact:	Yajie Hu (huyajie1@huawei.com)

1. Introduction

This paper is introduced for enhance the AIML model storage and discovery

2. Reason for Change

Currently, the ML model storage and discovery does not take continuously training into consideration, as for some kind of ML model which cannot satisify the ML model requirment of the consumer, and the ML model stored with necessary parameter, then the ML repository or the consumer can based on such information to continue train the model to satisify the ML model requirement.

Thus, this paper is introduced for providing enhancement on ML model storage and discovery.

3. Proposal

It is proposed to agree the following pCR to TS 23.482.

* * * First Change * * * *

8.11.2	ML model information storage

Figure 8.11.2-1 illustrates the procedure of ML model information storage.

Pre-condition:

1.	The AIMLE server has either received application specific model details from AIML consumer or produced analytics model.

Figure 8.11.2-1: ML model information storage

1.	The AIMLE server sends a ML model information storage request to the ML repository to store an ML model. The ML model included in the request can be one trained by the ML repository consumer or its information can be provisioned to the ML repository consumer. The request contains information elements as described in Table 8.11.4.1-1. The request message may contain Indication of continuously training and continuously training model parameter.

NOTE 1:	The security requirements within the ML model storage requirements are out of scope.

2.	Upon receiving the ML model information storage request, the ML repository verifies if the AIMLE server is authorized to store the ML model identified by Analytics ID and/or list of the allowed vendors provided within the ML model profile attribute. If the AIMLE server is authorized, the ML repository processes the request and records information of the ML model (e.g., by creating a ML model profile). The ML repository sends an ML model information storage response to the AIMLE server with an identifier of the created ML model profile.

8.11.3	ML model information discovery

Figure 8.11.3-1 illustrates the procedure of ML model information discovery.

Pre-condition:

1.	Either AIMLE consumer has requested to discover ML model or AIMLE server decides to discover model for training.

Figure 8.11.3-1: ML model information discovery

1.	The AIMLE server sends an ML model information discovery request to the ML repository. The request contains information elements as described in Table 8.11.4.3-1. The request message may contain AIMLE server capability.

2.	Upon receiving the ML model information discovery request, the ML repository verifies if the requestor is authorized to discover the ML model(s) identified by the ML model ID. Further, it verifies whether the requestor is present in the list of allowed vendors or not. If the ML repository consumer is authorized, the ML repository processes the request and discovers the model based on ML model ID, Analytics ID, Base Model ID, and/or ML model interoperability information.

If the discovery request is for transfer of learning, the ML repository identifies all candidate models matching the domain and required accuracy level.

If the discovery request contains AIMLE server, then the ML repository determines the Model whether the model need further training to satify the ML model requirement of the AIMLE server based on the AIMLE server capability and ML model profile. For example, if the AIMLE server has the model continuiously training capability and the model stored in ML repository cannot satisify the ML requirement of the AIML E server, then the ML repository may determine the model which requires continuously training to satisify the ML model requirement of the AIMLE server and indicate AIME server the ML model need continuously training, or if the AIMLE server does not have the model continuiously training capability and the model stored in ML repository cannot satisify the ML requirement of the AIML E server, then the ML repository may perform the model training of stored ML model to satify the ML model requirement of AIMLE server.

The ML repository sends the response message to the ML repository consumer. The response includes information of the discovered ML model. Optionally, the ML repository can initiate a model transfer process to the consumer.

NOTE 1:	When the ML model is within training or re-training phase, the ML model is not available for discovery. The details is indicated within the Result IE in Table 8.11.4.4-1. The training and re-training phase is used during federated learning so that AIML enabler consumer can rely on model repository to get the model details.

NOTE 2:	The model transfer process mentioned in step 2 can use the AI/ML data transfer policies.

Editor’s Note 1: If List of allowed vendors is not present then how to get consent from model owner to share the model to the requestor is FFS.

Editor’s Note 2: How to consider whether model is discoverable or not is FFS.

8.11.4	Information flows

8.11.4.1	ML model information storage request

Table 8.11.4.1-1 describes the information flow from the AIMLE server to the ML repository as a request for the ML model information storage.

Table 8.11.4.1-1: ML model information storage request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the ML repository consumer performing the request. |
| ML model | O (NOTE) | Provides the ML model. |
| ML model address | O (NOTE) | Represents the ML model address that can be used by Model repository to download the ML model. |
| ML model information | M | Provides information of the ML model, as described in Table 8.11.4.1-2. |
| NOTE: At least one of these information elements shall be provided. |  |  |

Table 8.11.4.1-2: ML model information

| Information element | Status | Description |
| --- | --- | --- |
| ML model identifier | M | An identifier for the ML model |
| Analytics ID | O | Represents analytics ID for which the model can be used. |
| VAL service ID(s) | O | Identify the VAL service ID(s). |
| Domain | O | Specifies domain for which the model can be used (e.g., for speech recognition, image recognition, video processing, location predication, etc.). |
| List of allowed vendors | O (NOTE 1) | Indicates which vendors that are allowed to retrieve the ML model and thereby also are interoperable to the model. |
| ML model interoperability information | O (NOTE 1) | Represents the vendor-specific information that conveys, e.g., requested model file format, model execution environment, etc. The encoding, format, and value of ML Model Interoperable Information is not specified since it is vendor specific information, and is agreed between vendors, if necessary for sharing purposes. |
| ML Model phase | O (NOTE 1) | Represents the ML model phase, e.g., in training, trained, re-training, deployed. |
| > Observed accuracy | O (NOTE 2) | Provides accuracy of the model (if ML model is in trained phase). |
| > Indication of continuously model training | O | Indicate whether the model can be continuously trained or not. |
| > continuously training model parameter |  | Parameters required for continuing model training. |
| ML model storage and discovery requirements | O (NOTE 1) | Represents the requirements for the ML repository for the ML model storage and discovery. |
| > Storage duration | O | Represents the ML model storage duration time. When the storage duration time is expired, the stored ML model and the related information shall be deleted. |
| > Security and access requirements | O | Represents the information on security requirements for storing the ML model information and the ML model access requirements (e.g., publicly available, private use only, or available for the list of VAL server IDs). |
| NOTE 1: At least one of these information elements shall be provided. NOTE 2: This IE is included only if trained ML model is available. |  |  |

Editor’s Note:	More parameters for ML model phase is FFS.

8.11.4.2	ML model information storage response

Table 8.11.4.2-1 describes the information flow from the ML repository to the AIMLE server as a response for the ML model information storage request.

Table 8.11.4.2-1: ML model information storage response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request. |
| ML model address | O | Represents the ML model address that can be used by consumer to download the ML model. This IE is included when result IE indicates success. |
| ML model identifier | M | An identifier for the ML model. This IE is included when result IE indicates success. |

8.11.4.3	ML model information discovery request

Table 8.11.4.3-1 describes the information flow from the AIMLE server to the ML repository as a request for the ML model information discovery.

Table 8.11.4.3-1: ML model information discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the ML repository consumer performing the request. |
| Filtering criteria | M | Represents the filtering criteria, which can be any of the ML model information as in Table 8.11.4.1-2. |
| AIMLE server capabiltiy | O | Indicates the AIMLE server’s capability, e.g. model continuiously training capability. |

Editor’s Note: The inclusion of ML model ID and Base Model ID are FFS.

8.11.4.4	ML model information discovery response

Table 8.11.4.4-1 describes the information flow from the model repository to the AIMLE server as a response for the ML model information discovery request.

Table 8.11.4.4-1: ML model information discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request. |
| ML model ID list | O | Represents the identifiers of the discovered list of ML models. This information element shall be provided if the result is success. |
| > ML model | O (NOTE 1) (NOTE 2) | Represents the ML model(s). This information element may be provided if the result is success. |
| > ML model address(es) | O (NOTE 1) (NOTE 2) | Represents the ML model address(es) at Model repository that can be used to download the ML model. This information element may be provided if the result is success. |
| > Indication of continuously model training | O | Indicate whether the model need to be continuously trained or not |
| NOTE 1: Only one of these information elements shall be provided. NOTE 2: For transfer learning, the list of all possible models matching the filtering criteria can be included along with model address. |  |  |

* * * END of Change * * * *