3GPP TSG-SA WG6 Meeting #64	S6-245253

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245xxx)

Source:	Samsung

Title:	Pseudo-CR on updates to the VFL training procedures

Spec:	3GPP TS 23.482 V0.3.0

Agenda item:	9.10

Document for:	Approval

Contact:	Arunprasath R, arun.prasath@samsung.com

1. Introduction

This pCR proposes to update the VFL procedure.

2. Reason for Change

VFL procedure is not complete and details about the continuous monitor and selection of AIMLE clients , retrieval of ML model and mechanism to check the AIMLE clients of their willingness to participate is missing.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.3.0

* * * First Change * * * *

### 8.18.2	Procedure for supporting VFL

Pre-conditions:

1.	VFL members registered their VFL profile to the AIMLE Server. The VFL members may update their status or information in their VFL profile to the AIMLE Server.

2.	The datasets of each of the UEs (where the AIMLE Clients (as VFL members) are deployed on) belong to more than one different data domains.

3.	The VAL server has successfully subscribed/registered with the AIMLE server for model training notifications.

Figure 8.18.2-1: Procedure for supporting VFL

1.	An AIMLE server receives an ML model training request from a VAL server as described in clause 8.3.2. If ML model training request is received and the AIMLE server determines to use VFL training (VFL training between domains), the procedure continues to step 2.

2.	The AIMLE server retrieves the indicated ML model in step 1 using the ML model retrieval procedure as described in clause 8.1.

3.	For the VFL training, the AIMLE server performs AIMLE client selection using the AIMLE client selection criteria or a list of AIMLE clients provided in step 1. If AIMLE client selection criteria are provided in step 1, then the AIMLE server continuously monitors and selects AIMLE clients for the VFL training. If a list of AIMLE clients is provided in step 1, the AIMLE server selects the provided AIMLE clients for the VFL training.

4.	Based on the AIMLE client set determined in step 3, the AIMLE server checks with the selected AIMLE clients for their capability and participation in the VFL training as described in the clause 8.10.

5.	The AIMLE Server coordinates the selected VFL members for VFL training. During VFL training process, the VFL members send intermediate results to the AIMLE Server, and the AIMLE Server responds to the VFL members with the updated information (e.g. gradients). The information from the AIMLE server can be used to update the model parameters maintained at each VFL member.

5a.	The AIMLE server may report to the VAL server with the intermediate training status, the VAL server may adjust its request on the ML model training.

Editor’s Note 1:	Clarification on step 4a is needed.

Editor’s Note 2:	The procedure for AIMLE Server coordinates the selected VFL members to perform VFL training is FFS.

6.	The AIMLE server sends a ML model training notification to the VAL server as described in clause 8.3.2. If the training schedule is not complete (e.g., there are remaining training rounds), the AIMLE server configures the next set of training schedules and steps 3 to 5 are repeated for the next training round.

* * * Next Change * * * *

<Proposed change in revision marks>

* * * Next Change * * * *

<Proposed change in revision marks>