3GPP TSG-SA WG6 Meeting #64	S6-245275

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245xxx)

Source:	Nokia

Title:	Architecture update for Digital Assets Service

Spec:	3GPP TS 23.438 v0.1.0

Agenda item:	9.11

Document for:	Approval

Contact:	Niranth (niranth.amogh@nokia.com)

1. Introduction

This contribution provides a proposal for Architecture update.

2. Reason for Change

The architecture specified in figure 6.2-2 Digtal asset service exposure to API invoker using CAPIF architecture requires to be generalized for SEAL and not specific to Digital Asset service.

Also updating the functional entities desriptions.

3. Proposal

It is proposed to approve these changes for 3GPP TS 23.438 v0.1.0.

* * * First Change * * * *

# 6	Architecture

## 6.1	General

The digital asset service architecture enables the digital asset service capabilities and resources to be offered as APIs to be consumed by the VAL services deployed in the network and to be consumed by the VAL services and DA clients deployed on the UE. The different architecture representations are specified in this clause.

## 6.2	Architecture description

Figure 6.2-1 illustrates the digital asset service architecture to support Metaverse services using SEAL architecture specified in 3GPP TS 23.434 [4].

Figure 6.2-1: Digital asset service architecture to support Metaverse services using SEAL architecture

The SEAL architecture includes DA client and server which supports the VAL layer. The DA server interacts with the VAL server(s) over the DA-S reference point. The DA client interacts with the VAL client(s) over the DA-C reference point. The interactions between DA client and DA server is over the DA-UU reference point. The DA server may utilize the 5GC services over N33 reference point. The DA client may be located within the VAL client(s) in which case the DA-C reference point will be internal to VAL client(s). The service APIs offered by DA server on DA-UU and DA-S to the VAL layer functions (via DA client) follow the service-based architecture as shown in the figure 6.2-3.

Figure 6.2-2 illustrates the service-based SEAL architecture for digital asset service to support Metaverse services.

Figure 6.2-2: Service-based SEAL architecture for digital asset service to support Metaverse services

The DA server exposes the APIs over the Sda interface to the VAL layer. The VAL functions (VAL clients and servers) and the DA client (serving a VAL client) consume the services by interacting on the Sda interface of the DA server. The CAPIF can be used by VAL functions to discover the services of the DA server.

## 6.3	Functional elements

### 6.3.1	DA client

The following capabilities are supported by the DA client:

-	Supports the client-side functionalities for digital asset profile create, update, retrieve and delete operations.

-	Supports the client-side functionalities for digital asset discovery.

### 6.3.2	DA server

The following capabilities are supported by the DA server:

-	Supports the functionalities for digital asset profile create, update, retrieve and delete operations.

-	Supports the functionalities for digital asset discovery.

## 6.4	Reference points

### 6.4.1	DA-C

This reference point supports the interactions between VAL client(s) and DA client.

### 6.4.2	DA-UU

This reference point supports the interactions between DA client(s) and DA server. The DA client can consume the digital asset services supported by the DA server as specified in clause 6.3.2.

### 6.4.3	DA-S

This reference point supports the interactions between VAL server(s) and DA server. The VAL server consumes the digital asset services supported by the DA server as specified in clause 6.3.2.

## 6.5	Service-based interface

### 6.5.1	Sda

This interface supports the interactions between VAL function (VAL servers) and DA server. The VAL function consumes the digital asset service as specified in clause 6.3.2.

* * * End of Changes * * * *