3GPP TSG-SA WG6 Meeting #64	S6-245380

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245378)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0128 | rev | 3 | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS service area handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCOver5MBS |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SA2 has agreed a number CRs related to local MBS services and MBS service area creation without PCC rule (e.g., local MBS service area) The CRs are: S2-2311705, S2-2311285 discuss that different MB-SMFs may be assigned for different MBS service areas in an MBS session. In such cases, the same TMGI is allocated for this MBS session. The CRs S2-2409043, and S2-2409044 discuss the case of MBS session creation without PCC rule for MC service server outside a trusted domain, where the MBS service area is across multiple MB-SMFs. In this case the NEF/MBSF may need to group the MBS service area based on the supported MB-SMF, so that the MC service server sends an MBS session correctly. If the updated MBS service area is partially accepted by the 5GC, the NEF/MBSF sends a reduced MBS service area to the MC service server, where the latter creates a new MBS session, if not supported, it creates a unicast session instead. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - add a note in clause 4.7.1 referring to the local services according to clause 6.2 and location dependent broadcast services according to clause 7.3.4 of 3GPP TS 23.247. - update clause 7.3.3.1.2 to reflect the case of MBS session creation. - updates clauses 7.3.3.2.2 and 7.3.3.2.3 to reflect the described scenario in case of MBS service area update. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Aspects related to local services is not clearly mentioned and referred to in 3GPP TS 23.247. The behaviour of MC service server is not clear when it comes to MBS session creation and update procedures done by an untrusted MC service server when the MBS service area is not supported by the MB-SMF service area of a single MB-SMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 4.7.1, 7.3.3.1.2, 7.3.3.2.2 and 7.3.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev2: including the case of TMGI allocation request rejection. |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

For the purposes of the present document, the following terms given in 3GPP TS 23.280 [3] apply:

MC service

MC service user

MC service UE

MC system

MC user

For the purposes of the present document, the following terms given in 3GPP TS 23.247 [15] apply:

MBS session

Broadcast MBS session

Multicast communication service

Multicast MBS session

Broadcast communication service

MBS service area

MB-SMF service area

# * * * * Second change * * * *

### 4.7.1	General

The main purpose of 5G Multicast-Broadcast Service (MBS) use by mission critical services is to provide efficient downlink delivery of user traffic in group calls and communications. The architectural figures in this clause are aligned  with the 5GS architecture for MBS shown in Figure 5.1-2 of 3GPP TS 23.247 [15], which identifies both mandatory and optional functional entities and interfaces, in reference point representation, available for use by the MC services.

Multicast and broadcast communication services in 5G for MC group communications rely on the creation and establishment of MBS sessions to deliver user data in downlink. Shared and individual delivery from the MC service server to multiple MC users (i.e., users affiliated to a certain MC group) is supported either as point-to-point or point-to-multipoint over the radio. The MBS sessions are either broadcast or multicast type and consist of one or multiple QoS flows for different service requirements. For the broadcast MBS session or local MBS session, the MBS service area is configured with the MBS session.

NOTE 1:	Support of MBS and specific session types is an implementation choice.

NOTE 2:	Aspects related to MC services over local MBS sessions and location dependent broadcast services are considered according to 3GPP TS 23.247 [15] clause 6.2 and clause 7.3.4, respectively.

Within this arrangement, the MC service server decides whether to create broadcast or multicast MBS sessions to be associated with certain MC groups. The 5GC adaptively decides whether to deliver the MBS traffic from the MB-UPF in the form of shared delivery or individual delivery, where the latter is applicable to multicast MBS sessions only. The NG-RAN decides to utilize point-to-point or point-to-multipoint delivery methods applicable for shared delivery only. MBS provides reliability enhancements and minimizes loss of information, e.g., due to mobility and handover.

MBS group scheduling mechanism enables simultaneous reception of MBS and unicast user traffic by the MC service UEs. The UEs can receive broadcast MBS sessions irrespective of their RRC state (i.e., connected, inactive or idle) and multicast sessions only in RRC-CONNECTED and RRC-INACTIVE state.

The following capabilities (non-exhaustive list) provided by MBS could be used by MC service servers as described in 3GPP TS 23.247 [15]:

-	MBS session creation;

-	MBS session update;

-	MBS session release;

-	MBS session ID allocation;

-	Transparent MBS Data forwarding;

-	Dynamic PCC control for MBS session;

-	UE's MBS assistance information provision.

# * * * * Third change * * * *

##### 7.3.3.1.2	Procedure for pre-created MBS session and MBS session announcement

Pre-conditions:

-	The MC service server has decided to use an MBS session for MC service group communications associated to a certain MC service group based on transport only mode.

-	The MC service server has performed MB-SMF discovery and selection either directly or indirectly, via NEF/MBSF, unless the corresponding information is locally configured.

-	MC service clients 1 to n are attached to the 5GS, registered and affiliated to the same MC service group X.

-	The MC service server is aware whether to request the creation of the MBS service server with or without dynamic PCC rule.

Figure 7.3.3.1.2-1: Use of pre-created MBS session.

- The MC service server initiates an MBS session creation procedure towards the 5GC as described in 3GPP TS 23.247 [15]. The procedure starts once the MC service server initiates a TMGI allocation request (either directly to MB-SMF or indirectly via NEF). Upon the reception of the TMGI allocation response, the MC service server sends an MBS session creation request, including further information related to the MBS session, e.g., MBS session ID, MBS session mode and the QoS requirements if dynamic PCC rule is not considered. However, if dynamic PCC rule is considered, the MC service server defines these requirements at a later step, namely it sends an MBS authorization/policy create request towards PCF (either directly or to the NEF) indicating the QoS requirements.

In the case of an untrusted MC service server, when the requested MBS service area crosses several MB-SMF service areas, the TMGI allocation request is rejected by the NEF/MBSF and it guides the MC service server by dividing the requested MBS service area into groups as described in 3GPP TS 23.247 [15]. Hence, the MC service server initiates a new TMGI allocation request for each grouped MBS service area. If during MBS session creation request the 5GS discovers that requested MBS service area crosses several MB-SMF service areas, the request is rejected, and the NEF/MBSF guides the MC service server as described earlier. Hence, the MC service server initiates a new MBS session creation procedure for each grouped MBS service area. The MC service server may utilize a unicast session if any MBS service area is not supported by any MB-SMF.

NOTE 1:	In case of LTE eMBMS and 5G MBS co-existence, the MC service server may trigger the establishment of eMBMS bearers as described in 3GPP TS 23.280 [3] (or it may establish a unicast bearer) based on the RAT capabilities supported by the affiliated members in the MC service group X. If MBSF and BM-SC are co-located, TMGI used by 4G eMBMS can be the same as the MBS session ID.

NOTE 2:	For the case of multi carrier support for broadcast MBS sessions, the MC service server may indicate the frequencies within a broadcast MBS service area by providing the MBS frequency selection area ID(s) (MBS FSA ID(s)) to the MB-SMF, or indirectly, via NEF.

2.	The MC service server provides the MC service clients affiliated to MC service group X with the information related to the created MBS session via the MBS session announcement. As described in table 7.3.2.1-1, the MBS session announcement includes information such as the MBS session ID, MBS session mode (broadcast or multicast service type) and SDP information related to the MBS session under consideration.

NOTE 3:	The MC service server may send an MBS session announcement at an earlier step during the MBS session creation procedure towards the MC service clients once the MC group associated to the MBS session is known.

Optionally, the MC service server includes the information elements related to the established eMBMS bearer once the MC service server has determined the need, as indicated in table 7.3.2.1-1. The MC service clients which camp on LTE will subsequently react to the information elements related to the eMBMS bearer as described in 3GPP TS 23.280 [3].

3.	MC service clients store and process the received MBS session information.

4.	MC service clients may provide an MBS session announcement acknowledgment to the MC service server to indicate the reception of the corresponding MBS session announcement.

5.	Based on the MBS session mode (either multicast or broadcast), the following actions take place:

5a.	For multicast MBS sessions, MC service clients initiate a UE session join request towards the 5GC using the information provided via the MBS session announcement. Hence, upon the first successful UE session join request, the multicast is then established, and the radio resources are reserved, if the session is in an active state. The established session can either be in active or inactive state as indicated in 3GPP TS 23.247 [15]. The MC service clients sends a UE session join notification towards the server. If indicated in the MBS session announcement information, MC service clients report the monitoring state (i.e., the reception quality of the MBS session) back to the MC service server; or

5b.	For broadcast MBS sessions, if the MC service client is accessing over 5G, the session is established as part of the session creation procedures as described in 3GPP TS 23.247 [15], and the network resources are reserved both in 5GC and NG-RAN. The MC service clients start monitoring the reception quality of the broadcast MBS session. If indicated in the MBS session announcement information, MC service clients report the monitoring state (i.e., the reception quality of the MBS session) back to the MC service server.

NOTE 4:		It is implementation specific whether the MBS session reception quality level is determined per MBS session, per media stream or per MBS QoS flow level via e.g., measurements of radio level signals, such as the reference signals from the NG-RAN node(s), or packet loss.

6.	The MC service clients provide a listening status notification related to the announced session (multicast or broadcast session) in the form of an MBS listening status report.

7.	An MC service group communication setup takes place as specified in 3GPP TS 23.379 [6], 3GPP TS 23.281 [4], or 3GPP TS 23.282 [5]. The MC service server determines to use the pre-created MBS session for this group communication.

# * * * * Fourth change * * * *

##### 7.3.3.2.2	Procedure for updating MBS resources without dynamic PCC rule

The procedure shown in figure 7.3.3.2.2-1 presents an MBS session update procedure triggered by the MC service server (either directly to the MB-SMF, or indirectly via NEF/MBSF). Within the update request, either the service requirements, MBS service area, activity status of multicast MBS session, or all three are done, as indicated in 3GPP TS 23.247 [15].

Pre-conditions:

-	The MC service clients 1 to n are attached to the 5GS, registered and affiliated to the same active MC service group.

-	The MC service server has obtained the required information related to the MB-SMF, either locally configured or during initial session configuration.

-	The MBS session is created with certain service requirements and optionally with a certain broadcast/multicast service area. The MBS session is announced to be associated with the MC service group for group communication purposes.

Figure 7.3.3.2.2-1: MBS session update without dynamic PCC.

1.	An MBS session is established as described in in 3GPP TS 23.247 [15] (either a multicast or a broadcast session), and associated with a certain active MC group for group communication purposes. In the case of a multicast MBS session, the MC service clients have already joined the session.

2.	The MC service server invokes an MBS session update request towards the 5GC (either directly to the MB-SMF or indirectly via NEF/MBSF) once the need has emerged to modify some aspects for the given MBS session under consideration. Hence, the MC service server sends the MBS session update request as described in 3GPP TS 23.247 [15] and either directly to the MB-SMF or indirectly via NEF/MBSF, indicating the MBS session ID to be updated. Along with the update request, the updated aspects are sent, which are either service requirements (required QoS), service area, or both. In case of multicast MBS sessions, the MC service server may as well update the status (active or inactive) of the multicast MBS session once needed within the update request.

NOTE 1:	The updated service area information is required for local MBS and for broadcast MBS services.

3.	Based on the needed requirements, the corresponding MBS session is accordingly modified, as indicated in 3GPP TS 23.247 [15]. The update may lead to QoS Flow(s) addition, modification, or removal.

4.	The MC service server receives an MBS session update response as described in 3GPP TS 23.247 [15], once the requested modifications are performed, and the indicated MBS session is updated accordingly.

In case the of untrusted MC service server, if the MBS service area is partially accepted by the 5GC, the reduced MBS service area information is grouped and provided by the NEF/MBSF in the response. Hence, the MC service server should send a new MBS session creation request (as described in clause 7.3.3.1.3) for each grouped MC service area. The MC service server may utilize a unicast session if any MBS service area is not supported by any MB-SMF.

5.	The MC service server may initiate a session announcement towards the MC service clients associated with the ongoing session in order to announce the updated information, if required, e.g., the updated service area or SDP information.

NOTE 2:	The updated service area information is required for local MBS and for broadcast MBS services.

6.	The MC service server sends an MapGroupToSessionStream over the configured MBS session providing the required information to receive the media related to the established MC service group communication.

7.	The MC service clients process the received information over the MapGroupToSessionStream in order to receive the associated MC media over the specific MBS session stream.

8.	MC service client 1 sends media to the MC service server over unicast to be distributed for the established group communication.

9.	The MC service server distributes the MC media to the MC services clients 2 to n over the indicated streams.

##### 7.3.3.2.3	Procedure for updating MBS resources with dynamic PCC rule

The procedure shown in figure 7.3.3.2.3-1 presents an MBS session update procedure triggered by the MC service server to the 5GC, either directly or via NEF/MBSF. Based on the required updates to be done, the MC service server needs to interact with the MB-SMF to update the MBS service area and multicast activity status, with the PCF to update the required QoS requirements, or sequentially both to update all the above, as indicated in 3GPP TS 23.247 [15].

Pre-conditions:

-	The MC service clients 1 to n are attached to the 5GS, registered and affiliated to the same active MC service group.

-	The MC service server has obtained the required information related to the MB-SMF, either locally configured or during initial session configuration.

-	The MBS session is created with certain service requirements and optionally with a certain broadcast/multicast service area. The MBS session is announced to be associated with the MC service group for group communication purposes.

Figure 7.3.3.2.3-1: MBS session update with dynamic PCC.

1.	An MBS session is established as described in 3GPP TS 23.247 [15] (either a multicast or a broadcast MBS session), and associated with a certain active MC group for group communication purposes. In the case of a multicast MBS session, the MC service clients have already joined the MBS session.

2.	The MC service server, based on the update requirements (i.e., MBS service area and/or the multicast MBS session activity status as well as the service requirements), perform the MBS session update with PCC procedure towards the 5GS as described in 3GPP TS 23.247 [15] and/or the MBS session activate/deactivate procedure as described in clause 7.3.3.4.

In case the of untrusted MC service server, if the MBS service area is partially accepted by the 5GC, the reduced MBS service area information is grouped and provided by the NEF/MBSF in the response. Hence, the MC service server should send a new MBS session creation request (as described in clause 7.3.3.1.3) per grouped MC service area. The MC service server may utilize a unicast session if any MBS service area is not supported by any MB-SMF.

NOTE 1:	The updated service area information is required for local MBS and for broadcast MBS services.

3.	The MC service server may initiate a MBS session announcement towards the MC service clients associated with the ongoing MBS session in order to announce the updated information if required, e.g., the updated service area or SDP information.

NOTE 2:	The updated service area information is required for local MBS and for broadcast MBS services.

4.	The MC service server sends an MapGroupToSessionStream over the MBS session providing the required information to receive the media related to the established MC service group communication.

5.	The MC service clients process the received information over the MapGroupToSessionStream in order to receive the associated MC media over the specific MBS session stream.

6.	MC service client 1 sends media to the MC service server over unicast to be distributed for the established group communication.

7.	The MC service server distributes the MC media to the MC services clients 2 to n over the indicated streams.

# * * * * End of changes * * * *