3GPP TSG-SA WG6 Meeting #64	S6-245513

Orlando, USA, 18th – 22th Nov 2024	(revision of S6-245164)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0347 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve the EN for using SL/Ranging positioning method |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This contribution main resolves the following EN: Editor's note: Potential enhancement to on-demand location reporting procedure is FFS, e.g. whether using SL/Ranging positioning method to obtain target UE location. Based on the current description of clause 9.3.4, the off-network procedure as defined in clause 9.5 is triggered when the received UE positioning method is PC5 SEAL LM. As the LM-PC5 reference point in SEAM LM utilizes PC5 reference point as described in 3GPP TS 23.303 in which it’s stated the lower protocol layers of the PC5 reference point can be based on E-UTRA sidelink capabilities or on WLAN technology. So from my point view, obtaining the target UE location using Sidelink/Ranging positioning method has been in the scope of PC5 SEAL LM. Thus the EN is not needed. And the off-network location report/response information flows for PC5 SEAL LM need to add some examples to cover the SL/Ranging positioning method. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the left EN in clause 9.3.4. - Add example in clause 9.5.2.4 and 9.5.2.8. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The reader may be confused about the methods about the obtaining the target UE location. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.4, 9.5.2.4, 9.5.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.3.4	On-demand location reporting procedure

The location management server can request UE location information at any time by sending a location information request to the location management client, which may trigger location management client to immediately send the location report.

Figure 9.3.4-1: On-demand location information reporting procedure

1.	Based on configurations such as periodical location information timer, or location information request from other entities (e.g., another location management client, VAL server), location management server initiates the immediately request location information from the location management client.

2.	The location management server sends a location information request to the location management client.

3.	If the target VAL UE ID or user ID is not equal to its own identity in the LM client, the LM client triggers off-network location management procedures as described in clause 9.5 to obtain target VAL UE location. If the received UE positioning method is PC5 SEAL LM, target VAL UE location information (including velocity) is obtained via off-network procedure as defined in clause 9.5; if the received UE positioning method is non-3GPP (e.g. WiFi, BT), target VAL UE location information (including velocity) is obtained via the corresponding non-3GPP method. If the target VAL UE ID or user ID is equal to its own identity in the LM client, VAL user or VAL UE is notified and asked about the permission to share its location. VAL user can accept or deny the request.

4.	The location management client immediately responds to the location management server with a report containing location information identified by the location management server and available to the location management client.

5.	Upon receiving the report, the location management server updates location of the reporting location management client. If the location management server does not have location information of the reporting location management client before, then just stores the reporting location information for that location management client.

* * * Next Change * * * *

9.5.2.4	Off-network location report

Table 9.5.2.4-1 describes the information flow from the location management client-2 to the location management client-1 for the off-network location report.

Table 9.5.2.4-1: Off-network location report

| Information element | Status | Description |
| --- | --- | --- |
| Triggering event | M | Identity of the event that triggered the sending of the report |
| Location Information | M | Location information shared by VAL client e.g. retrieved from non-3GPP source, obtained via Sidelink positioning/Ranging method, etc. |
| Acknowledgement Required | O | If present, indicate the recipient of the message to acknowledge the message. |

* * * Next Change * * * *

9.5.2.8	Off-network location response

Table 9.5.2.8-1 describes the information flow from the location management client-2 to the location management client-1 for the off-network location response. The Off-network location response acts as an acknowledgement to the location management client-1.

Table 9.5.2.8-1: Off-network location response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure for the operation |
| Location Information | M | Location information shared by VAL client e.g. retrieved from non-3GPP source, obtained via Sidelink positioning/Ranging method, etc. |
| Velocity information | O | Velocity information |

* * * End of Changes * * * *