3GPP TSG-SA WG6 Meeting #64	S6-245626

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245173)

Source:	China Mobile

Title:	pCR on DC application profiles updating on UE

Spec:	3GPP TS 23.392 v0.2.0

Agenda item:	9.12

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

This pCR proposes the procedure and information flows of DC application profiles updating on UE.

2. Reason for Change

Add the procedure and information flows of DC application profiles updating on UE.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.392 v0.2.0.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS)".

[4]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction"

[23434]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL)".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

* * * Next Change * * * *

6.2	MMTel Enabler architecture

Figure 6.2-1 shows the MMTel Enabler architecture.

The MMTel Enabler Server is hosted within the PLMN operator network which supports the MMTel services capabilities exposure.

Figure 6.2.1-1: MMTel Enabler architecture

The MMTel functional entities on the UE and the server sides are MMTel Enabler client and MMTel Enabler server respectively.

The MMTel Enabler client communicates with the MMTel Enabler server over the MMTel-1 reference point to support the application layer interactions needed in MMTel service.

NOTE	The interaction between MMTel Enabler Client and DCMTSI Client.is internal interaction in the dialler application on the UE and implementation specific.

The MMTel Enabler client also communicates with the DCMTSI client over the MMTel-4 reference point to downloading behavior on DCMTSI Client based on the application policy provided by the MMTel Enabler Server. The DCMTSI client is specified in 3GPP TS 26.114 [4].

The Application Server communicates with the MMTel Enabler server over the MMTel-2 reference point to invoke the MMTel service related capabilities provided by MMTel Enabler server,

The Controlling Application Server communicates with the MMTel Enabler server over the MMTel-3 reference point to manage the information needed in MMTel service.

The Application Server communicates with the Application Client on the UE over the App-1 reference point to provide the Data Channel Application specific logic.

The MMTel Enabler client(s) interacts with SEAL Clients over the SEAL-C reference point specified for each SEAL service. The MMTel Enabler server(s) interacts with SEAL Servers over the SEAL-S reference point specified for each SEAL service. The interaction between a SEAL Client and the corresponding SEAL Server is supported by SEAL-UU reference point specified for each SEAL service as specified in 3GPP TS 23.434 [23434].

* * * Next Change * * * *

6.3.x1	SEAL Client

The following SEAL Clients for MMTel enablement service are supported:

-	Notification management client as specified in clause 17 of TS 23.434 [23434];

6.3.x2	SEAL server

The following SEAL servers for MMTel enablement service are supported:

-	Notification management server as specified in clause 17 of TS 23.434 [23434];;

* * * Next Change * * * *

6.4.y1	SEAL-C

The following SEAL-C reference points for MMTel enablement service are supported:

-	NM-C reference point for notification management as specified in 3GPP TS 23.434 [23434];

6.4.y2	SEAL-S

The following SEAL-S reference points for MMTel enablement service are supported:

-	NM-S reference point for notification management as specified in 3GPP TS 23.434 [23434];

6.4.y3	SEAL-UU

The following SEAL-UU reference points for MMTel enablement service are supported:

-	NM-UU reference point for notification management as specified in 3GPP TS 23.434 [23434];

* * * Next Change * * * *

8.3.x	DC application profiles updating on UE

#### 8.3.x.1	General

There are two modes in this procedure:

the MMTel Enabler server notifies the MMTel Enabler client in the UE to send application request messages to the MMTel Enabler server to request the profiles of data channel applications;

the MMTel Enabler server sends the profiles to the UE directly after the UE subscribes to the profile notification.

The two detailed procedures of DC application profiles updating on UE and the related information flow are specified in clause 8.3.x.2 and 8.3.x.3.

NOTE :	The updating procedures go over the top of DCSF. The detailed procedures to update data channel applications to the UE are out of scope of 3GPP TS 23.228 [3].

#### 8.3.x.2	Procedure

8.3.x.2.1	The MMTel Enabler client request DC application profiles based on notification

The MMTel Enabler server can notify the MMTel Enabler client in the UE to send application request messages to the MMTel Enabler server to request the DC application profiles, the detailed procedure is illustrated in Figure 8.3.x.2.1-1.

NOTE:	The MMTel Enabler server can instruct the MMTel Enabler client to request root application and data channel application profiles as required. e.g. root application version update, offline of a data channel application, etc.

Pre-condition:

1)	The bootstrap data channels have been established as specified in 3GPP TS 23.228 [3].

Figure 8.5.2.1-1: MMTel Enabler client request DC application profiles based on notification

1.	The notification channel has been established between notification management client and notification management Server as specified clause 17.3.3 of TS 23.434 [23434].

2.	The MMTel Enabler client subscribes to the MMTel Enabler server for receiving notifications.

3.	The MMTel Enabler server sends a Root application and profile update notification to notify the MMTel Enabler client in the UE to get the Root application or data channel profile. The Root application and profile update notification includes information elements as specified in Table 8.3.x.3.1-1.

The step 4 to step 8 are same  as steps 1 through 5 in clause 8.3.2.2.

8.3.x.2.2	The MMTel Enabler server notifies the DC application profile to the MMTel Enabler client

The MMTel Enabler server notifies the DC application profiles to the MMTel Enabler client is illustrated in Figure 8.3.x.2.2-1.

Pre-condition:

1)	The bootstrap data channels have been established as specified in 3GPP TS 23.228 [3].

Figure 8.3.x.2.2-1: MMTel Enabler server notifies the DC application profile list to UE

1.	The notification channel has been established between notification management client and notification management server as specified clause 17.3.3 of TS 23.434 [23434].

2.	The MMTel Enabler client subscribes to the MMTel Enabler server for receiving notifications.

3.	The MMTel Enabler server checks the provision of Root application and DC application profile list is needed, e.g. the UE has IMS DC capability, an application version or application profile update is available, etc.

NOTE:	The detail additional checklists in this step are out of scope of the present document.

4.	The MMTel Enabler server sends a Root application notification message to the MMTel Enabler client in the UE. The notification message includes information elements as specified in Table 8.3.x.3.2-1.

The MMTel Enabler server sends a data channel DC application profile List notification to the MMTel Enabler client in the UE. The notification message includes information elements are as same as the elements in Table 8.3.2.3.4-1.

#### 8.3.x.3	Information flows

8.3.x.3.1	Root application and profile update notification

Table 8.3.x.3.1-1 describes information elements for the Root application and profile update notification from the MMTel Enabler server to the MMTel Enabler client.

Table 8.3.2.3.1-1: Information elements in Root application and profile update notification

| Information element | Status | Description |
| --- | --- | --- |
| Urgent | O | Indicates whether the operation is urgent to UE, i.e. whether the Root application and profile need to be updated immediately. E.g. the update of Root application and profile is urgent if the available DC Application for the user is changed |

8.3.x.3.2	Root application notification

Table 8.3.x.3.2-1 describes information elements for the Root application notification from the MMTel Enabler server to the MMTel Enabler client.

Table 8.3.x.3.2-1: Information elements for Root application notification

| Information element | Status | Description |
| --- | --- | --- |
| Update needed | M | Indicates whether the Root application is needed to be updated by the MMTel Enabler client in the UE |
| Root application version | O | The newest version of the Root application. This IE presents only if the Root application is needed to be updated by the MMTel Enabler clientt in the UE. |
| Root application (see NOTE) | O | The Root application provided by the MMTel service provider, e.g. layout, and/or home page etc. of the Data Channel Application list. This IE presents if the update of the Root application in the UE is needed, i.e. the Root application version included in the Get Root application request is not equal to the newest version on the MMTel Enabler server. |
| NOTE: The detailed information of this IE is implementation specific and out of scope of the present document. |  |  |