| TSG-T Working Group1 SWG SIG meeting #11 |  |  | T1-00-0107 |
| --- | --- | --- | --- |
| Harpenden, UK |  | 5-7 June 2000 |  |
| Title | Proposal for RRC Default Messages to be included in TS 34.108 |  |  |
| Source | Chairman |  |  |
| Agenda Item | Update to TS 34.108 |  |  |
| Document for | Agreement |  |  |
| Contact | Dan Fox (Anritsu) dan.fox@eu.anritsu.com Tel: +44 1582 433357 |  |  |

Introduction

Part of the set of default messages for RRC test cases proposed by Panasonic will also be useful in a variety of other test groups (e.g. RLC, MAC, Radio Bearer).

This document proposes a set of the default messages that should be included in the Common Environment definitions document – TS 34.108 – for general use. Messages not included within this set would remain in the RRC test section of TS 34.123-1.

Common message set

| Message | Reason for inclusion |
| --- | --- |
| Contents of Master Information Block PLMN type is the case of GSM-MAP | General definition of environment |
| SIB 1 to 15 (except ANSI-41 cases) | General definition of environment |
| Default settings for cell 1 | Used for all test cases |
| Default settings for cells 2 – 8 | Consistancy |
| DOWNLINK DIRECT TRANSFER | All NAS signalling |
| INITIAL DIRECT TRANSFER | All NAS signalling |
| PAGING TYPE1 message: TM ( Speech in CS ) | All RRC connection MT |
| PAGING TYPE1 message: TM ( The others of speech in CS ) | All RRC connection MT |
| PAGING TYPE1 message: TM ( Packet in PS ) | All RRC connection MT |
| PHYSICAL CHANNEL RECONFIGURATION | One representative version? |
| RADIO BEARER SETUP | RLC, MAC, RB, RF. One parametisable. |
| RADIO BEARER SETUP COMPLETE | RLC, MAC, RB, RF. One parametisable. |
| RADIO BEARER RELEASE | Cleaning up after TCs |
| RADIO BEARER RELEASE COMPLETE | Cleaning up after TCs |
| RRC CONNECTION REQUEST | All RRC connection |
| RRC CONNECTION RELEASE | Cleaning up after TCs |
| RRC CONNECTION RELEASE COMPLETE | Cleaning up after TCs |
| RRC CONNECTION SETUP | All RRC connection |
| RRC CONNECTION SETUP COMPLETE | All RRC connection |
| SECURITY MODE COMMAND | All TCs with ciphering |
| SECURITY MODE COMPLETE | All TCs with ciphering |
| SIGNALLING CONNECTION RELEASE | Cleaning up after TCs |
| UPLINK DIRECT TRANSFER | All NAS signalling |