3GPP TSG-T1/SIG Meeting #17 	T1S-010095

- Melbourne, Australia, 14-16 May 2001

|  | 34.123-1 |  |  |  | CR |  | CR-Num |  |  |  rev |  |  |  |  |  |  Current version: |  |  |  |  | 3.3.0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  |  | (U)SIM |  |  | ME/UE |  | X |  | Radio Access Network |  |  |  |  |  |  | X |  | Core Network |  |  |
| Title:  |  | Updates to clause Annex A of TS 34.123-1 version 3.3.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Motorola India Electronics Limited |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 25 April 2001 |  |  |  |  |  |
| Category:  |  | D |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | R99 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | The IE UL-DL-Mode in IE TGPS-ConfigurationParams is a choice of UL or DL, going with convention in the Default Messages in Annex A, information regarding either UL or DL should only be present. For example, if DL mode is selected, DL compression mode is present and the UL compression mode should be absent. The presence of both reduces the readability. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | In the Default Message contents the “UL/DL Mode DL Downlink compressed mode method SF/2 Uplink compressed mode method Not Present “ is to be changed with “CHOICE UL/DL Mode DL Downlink compressed mode method SF/2 “ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | It may mislead to wrong interpretation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | Annex A of TS 34.123-1 version 3.3.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at: http://www.3gpp.org/3G_Specs/CRs.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://www.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2000-09 contains the specifications resulting from the September 2000 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

Annex A:
Default RRC Message Contents

This clause contains the default values of RRC messages, other than those specified in TS 34.108 clauses 6 and 9. Unless indicated otherwise in specific test cases, they shall be transmitted by the system simulator in RRC messages, and which are required to be received from the UE under test.

The necessary L3 messages are listed in alphabetic order, with the exception of the SYSTEM INFORMATION messages, where it is the information elements which are listed in alphabetic order (this is because some information elements occur in several SYSTEM INFORMATION types).

In this clause, decimal values are normally used. However, sometimes a hexadecimal value, indicated by an "H", or a binary value, indicated by a "B" is used.

# Contents of PHYSICAL CHANNEL RECONFIGURATION message: AM or UM (The others of speech in CS)

| Information Element | Value/remark |
| --- | --- |
| Message Type |  |
| RRC transaction identfier | Arbitrarily selects an integer between 0 and 3 |
| Integrity check info | The presence of this IE is dependent on IXIT statements in TS 34.1 present 23-2. If integrity protection is indicated to be active, this IE is with the values of the sub IEs as stated below. Else, this IE and the sub-IEs are omitted. |
| - message authentication code | SS calculates the value of MAC-I for this message and writes to this IE. |
| - RRC message sequence number | SS provides the value of this IE, from its internal counter. |
| Integrity protection mode info | Not Present |
| Ciphering mode info | Not Present |
| Activation time | (256+CFN-(CFN MOD 8 + 8))MOD 256 |
| New U-RNTI | Not Present |
| New C-RNTI | Not Present |
| RRC State indicator | CELL_DCH |
| UTRAN DRX cycle length coefficient | Not Present |
| CN information info | Not Present |
| URA identity | Not Present |
| RB with PDCP information | Not Present |
| Frequency info |  |
| - UARFCN uplink(Nu) | Reference to TS34.108 clause 6.10 Parameter Set |
| - UARFCN downlink(Nd) | Reference to TS34.108 clause 6.10 Parameter Set |
| Maximum allowed UL TX power | 33dBm |
| Uplink DPCH info |  |
| - Uplink DPCH power control info |  |
| - DPCCH power offset | -6dB |
| - PC Preamble | 8slot |
| - Power Control Algorithm | Algorithm1 |
| - TPC step size | 1dB |
| - Scrambling code type | Long |
| - Scrambling code number | 0 (0 to 16777215) |
| - Number of DPDCH | Not Present(1) |
| - spreading factor | SF is reference to TS34.108 clause 6.10 Parameter Set |
| - TFCI existence | TRUE |
| - Number of FBI bit | Not Present(0) |
| - Puncturing Limit | Reference to TS34.108 clause 6.10 Parameter Set |
| CHOICE Mode | FDD |
| - Downlink PDSCH information | Not Present |
| Downlink information common for all radio links |  |
| - Downlink DPCH info common for all RL |  |
| - Timing indicator | Maintain |
| - CFN-targetSFN frame offset | Not Present |
| - Downlink DPCH power control information |  |
| - DPC mode | 0 (single) |
| - DL rate matching restriction information | Not Present |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Fixed or Flexible Position | Flexible |
| - TFCI existence | TRUE |
| - Number of bits for Pilot bits(SF=128,256) | Not Present |
| - DPCH compressed mode info |  |
| - TGPSI | 1 |
| - TPGS status Flag | inactive |
| - TGCFN | (Current CFN + (256 – TTI/10msec)) mod 256 |
| - Transmission gap pattern sequence configuration parameters |  |
| - TGMP | FDD Measurement |
| - TGPRC | 62 |
| - TGSN | 8 |
| - TGL1 | 10 |
| - TGL2 | 5 |
| - TGD | 15 |
| - TGPL1 | 35 |
| - TGPL2 | 35 |
| - RPP | Mode 1 |
| - ITPITP | Mode 1 |
| - CHOICE UL/DL Mode | DL |
| - Downlink compressed mode method | SF/2 |
| - Downlink frame type | A |
| - DeltaSIR1 | 2.0 |
| - DeltaSIRafter1 | 1.0 |
| - DeltaSIR2 | Not Present |
| - DeltaSIRafter2 | Not Present |
| - TX Diversity mode | None |
| - SSDT information | Not Present |
| - Default DPCH Offset Value | 0 |
| Downlink information for each radio links |  |
| - Primary CPICH info |  |
| - Primary scrambling code | 100 |
| - PDSCH with SHO DCH info | Not Present |
| - PDSCH code mapping | Not Present |
| - Downlink DPCH info for each RL |  |
| - Primary CPICH usage for channel estimation | Primary CPICH may be used |
| - DPCH frame offset | 0 chips |
| - Secondary CPICH info | Not Present |
| - DL channelisation code |  |
| - Secondary scrambling code | 1 |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Code number | SF-1(SF is reference to TS34.108 clause 6.10 Parameter Set) |
| - Scrambling code change | No change |
| - TPC combination index | 0 |
| - SSDT Cell Identity | -a |
| - Closed loop timing adjustment mode | Not Present |
| - Secondary CCPCH info | Not Present |
| - TFCS | Not Present |
| - FACH/PCH information | Not Present |
| - References to system information blocks | Not Present |

# Contents of PHYSICAL CHANNEL RECONFIGURATION message: AM or UM (Speech in CS)

| Information Element | Value/remark |
| --- | --- |
| Message Type |  |
| RRC transaction identfier | Arbitrarily selects an integer between 0 and 3 |
| Integrity check info | The presence of this IE is dependent on IXIT statements in TS 34.123-2. If integrity protection is indicated to be active, this IE is present with the values of the sub IEs as stated below. Else, this IE and the sub-IEs are omitted. |
| - message authentication code | SS calculates the value of MAC-I for this message and writes to this IE. |
| - RRC message sequence number | SS provides the value of this IE, from its internal counter. |
| Integrity protection mode info | Not Present |
| Ciphering mode info | Not Present |
| Activation time | (256+CFN-(CFN MOD 8 + 8))MOD 256 |
| New U-RNTI | Not Present |
| New C-RNTI | Not Present |
| RRC State indicator | CELL_DCH |
| UTRAN DRX cycle length coefficient | Not Present |
| CN information info | Not Present |
| URA identity | Not Present |
| RB with PDCP information | Not Present |
| Frequency info |  |
| - UARFCN uplink(Nu) | Reference to TS34.108 clause 6.10 Parameter Set |
| - UARFCN downlink(Nd) | Reference to TS34.108 clause 6.10 Parameter Set |
| Maximum allowed UL TX power | 33dBm |
| Uplink DPCH info |  |
| - Uplink DPCH power control info |  |
| - DPCCH power offset | -6dB |
| - PC Preamble | 8slot |
| - Power Control Algorithm | Algorithm1 |
| - TPC step size | 1dB |
| - Scrambling code type | Long |
| - Scrambling code number | 0 (0 to 16777215) |
| - Number of DPDCH | Not Present(1) |
| - spreading factor | SF is reference to TS34.108 clause 6.10 Parameter Set |
| - TFCI existence | TRUE |
| - Number of FBI bit | Not Present(0) |
| - Puncturing Limit | Reference to TS34.108 clause 6.10 Parameter Set |
| CHOICE Mode | FDD |
| - Downlink PDSCH information | Not Present |
| Downlink information common for all radio links |  |
| - Downlink DPCH info common for all RL |  |
| - Timing indicator | Maintain |
| - CFN-targetSFN frame offset | Not Present |
| - Downlink DPCH power control information |  |
| - DPC mode | 0 (single) |
| - DL rate matching restriction information | Not Present |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Fixed or Flexible Position | Flexible |
| - TFCI existence | FALSE |
| - Number of bits for Pilot bits(SF=128,256) | Not Present |
| - DPCH compressed mode info |  |
| - TGPSI | 1 |
| TGPS Status Flag | inactive |
| - TGCFN | (Current CFN + (256 – TTI/10msec)) mod 256 |
| - Transmission gap pattern sequence configuration parameters |  |
| - TGMP | FDD Measurement |
| - TGPRC | 62 |
| - TGSN | 8 |
| - TGL1 | 10 |
| - TGL2 | 5 |
| - TGD | 15 |
| - TGPL1 | 35 |
| - TGPL2 | 35 |
| - RPP | Mode 1 |
| - ITP |  |
| - CHOICE UL/DL Mode | DL |
| - Downlink compressed mode method | SF/2 |
| - Downlink frame type | A |
| - DeltaSIR1 | 2.0 |
| - DeltaSIRafter1 | 1.0 |
| - DeltaSIR2 | Not Present |
| - DeltaSIRafter2 | Not Present |
| - TX Diversity mode | None |
| - SSDT information | Not Present |
| - Default DPCH Offset Value | 0 |
| Downlink information for each radio links |  |
| - Primary CPICH info |  |
| - Primary scrambling code | 100 |
| - PDSCH with SHO DCH info | Not Present |
| - PDSCH code mapping | Not Present |
| - Downlink DPCH info for each RL |  |
| - Primary CPICH usage for channel estimation | Primary CPICH may be used |
| - DPCH frame offset | 0 chips |
| - Secondary CPICH info | Not Present |
| - DL channelisation code |  |
| - Secondary scrambling code | 1 |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Code number | SF-1(SF is reference to TS34.108 clause 6.10 Parameter Set) |
| - Scrambling code change | No change |
| - TPC combination index | 0 |
| - SSDT Cell Identity | -a |
| - Closed loop timing adjustment mode | Not Present |
| - Secondary CCPCH info | Not Present |
| - TFCS | Not Present |
| - FACH/PCH information | Not Present |
| - References to system information blocks | Not Present |

# Contents of PHYSICAL CHANNEL RECONFIGURATION message: AM or UM (Packet to CELL_DCH from CELL_DCH in PS)

| Information Element | Value/remark |
| --- | --- |
| Message Type |  |
| RRC transaction identfier | Arbitrarily selects an integer between 0 and 3 |
| Integrity check info | The presence of this IE is dependent on IXIT statements in TS 34.123-2. If integrity protection is indicated to be active, this IE is present with the values of the sub IEs as stated below. Else, this IE and the sub-IEs are omitted. |
| - message authentication code | SS calculates the value of MAC-I for this message and writes to this IE. |
| - RRC message sequence number | SS provides the value of this IE, from its internal counter. |
| Integrity protection mode info | Not Present |
| Ciphering mode info | Not Present |
| Activation time | (256+CFN-(CFN MOD 8 + 8))MOD 256 |
| New U-RNTI | Not Present |
| New C-RNTI | Not Present |
| RRC State indicator | CELL_DCH |
| UTRAN DRX cycle length coefficient | Not Present |
| CN information info | Not Present |
| URA identity | Not Present |
| RB with PDCP information | Not Present |
| Frequency info |  |
| - UARFCN uplink(Nu) | Reference to TS34.108 clause 6.10 Parameter Set |
| - UARFCN downlink(Nd) | Reference to TS34.108 clause 6.10 Parameter Set |
| Maximum allowed UL TX power | 33dBm |
| Uplink DPCH info |  |
| - Uplink DPCH power control info |  |
| - DPCCH power offset | -6dB |
| - PC Preamble | 8slot |
| - Power Control Algorithm | Algorithm1 |
| - TPC step size | 1dB |
| - Scrambling code type | Long |
| - Scrambling code number | 0 (0 to 16777215) |
| - Number of DPDCH | Not Present(1) |
| - spreading factor | SF is reference to TS34.108 clause 6.10 Parameter Set |
| - TFCI existence | TRUE |
| - Number of FBI bit | Not Present(0) |
| - Puncturing Limit | Reference to TS34.108 clause 6.10 Parameter Set |
| CHOICE Mode | FDD |
| - Downlink PDSCH information | Not Present |
| Downlink information common for all radio links |  |
| - Downlink DPCH info common for all RL |  |
| - Timing indicator | Maintain |
| - CFN-targetSFN frame offset | Not Present |
| - Downlink DPCH power control information |  |
| - DPC mode | 0 (single) |
| - DL rate matching restriction information | Not Present |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Fixed or Flexible Position | Flexible |
| - TFCI existence | TRUE |
| - Number of bits for Pilot bits(SF=128,256) | Not Present |
| - DPCH compressed mode info |  |
| - TGPSI | 1 |
| - TGPS Status Flag | Inactive |
| - TGCFN | (Current CFN + (256 – TTI/10msec)) mod 256 |
| - Transmission gap pattern sequence configuration parameters |  |
| - TGMP | FDD Measurement |
| - TGPRC | 62 |
| - TGSN | 8 |
| - TGL1 | 10 |
| - TGL2 | 5 |
| - TGD | 15 |
| - TGPL1 | 35 |
| - TGPL2 | 35 |
| - RPP | Mode 1 |
| - ITP | Mode 1 |
| - CHOICE UL/DL Mode | DL |
| - Downlink compressed mode method | SF/2 |
| - Downlink frame type | A |
| - DeltaSIR1 | 2.0 |
| - DeltaSIRafter1 | 1.0 |
| - DeltaSIR2 | Not Present |
| - DeltaSIRafter2 | Not Present |
| - TX Diversity mode | None |
| - SSDT information | Not Present |
| - Default DPCH Offset Value | 0 |
| Downlink information for each radio links |  |
| - Primary CPICH info |  |
| - Primary scrambling code | 100 |
| - PDSCH with SHO DCH info | Not Present |
| - PDSCH code mapping | Not Present |
| - Downlink DPCH info for each RL |  |
| - Primary CPICH usage for channel estimation | Primary CPICH may be used |
| - DPCH frame offset | 0 chips |
| - Secondary CPICH info | Not Present |
| - DL channelisation code |  |
| - Secondary scrambling code | 1 |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Code number | SF-1(SF is reference to TS34.108 clause 6.10 Parameter Set) |
| - Scrambling code change | No change |
| - TPC combination index | 0 |
| - SSDT Cell Identity | -a |
| - Closed loop timing adjustment mode | Not Present |
| - Secondary CCPCH info | Not Present |
| - TFCS | Not Present |
| - FACH/PCH information | Not Present |
| - References to system information blocks | Not Present |

# Contents of PHYSICAL CHANNEL RECONFIGURATION message: AM or UM (Packet to CELL_DCH from CELL_FACH in PS)

| Information Element | Value/remark |
| --- | --- |
| Message Type |  |
| RRC transaction identfier | Arbitrarily selects an integer between 0 and 3 |
| Integrity check info | The presence of this IE is dependent on IXIT statements in TS 34.123-2. If integrity protection is indicated to be active, this IE is present with the values of the sub IEs as stated below. Else, this IE and the sub-IEs are omitted. |
| - message authentication code | SS calculates the value of MAC-I for this message and writes to this IE. |
| - RRC message sequence number | SS provides the value of this IE, from its internal counter. |
| Integrity protection mode info | Not Present |
| Ciphering mode info | Not Present |
| Activation time | (256+CFN-(CFN MOD 8 + 8))MOD 256 |
| New U-RNTI | Not Present |
| New C-RNTI | Not Present |
| RRC State indicator | CELL_DCH |
| UTRAN DRX cycle length coefficient | Not Present |
| CN information info | Not Present |
| URA identity | Not Present |
| RB with PDCP information | Not Present |
| Frequency info |  |
| - UARFCN uplink(Nu) | Reference to TS34.108 clause 6.10 Parameter Set |
| - UARFCN downlink(Nd) | Reference to TS34.108 clause 6.10 Parameter Set |
| Maximum allowed UL TX power | 33dBm |
| Uplink DPCH info |  |
| - Uplink DPCH power control info |  |
| - DPCCH power offset | -6dB |
| - PC Preamble | 8slot |
| - Power Control Algorithm | Algorithm1 |
| - TPC step size | 1dB |
| - Scrambling code type | Long |
| - Scrambling code number | 0 (0 to 16777215) |
| - Number of DPDCH | Not Present(1) |
| - spreading factor | SF is reference to TS34.108 clause 6.10 Parameter Set |
| - TFCI existence | TRUE |
| - Number of FBI bit | Not Present(0) |
| - Puncturing Limit | Reference to TS34.108 clause 6.10 Parameter Set |
| CHOICE Mode | FDD |
| - Downlink PDSCH information | Not Present |
| Downlink information common for all radio links |  |
| - Downlink DPCH info common for all RL |  |
| - Timing indicator | Maintain |
| - CFN-targetSFN frame offset | Not Present |
| - Downlink DPCH power control information |  |
| - DPC mode | 0 (single) |
| - DL rate matching restriction information | Not Present |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Fixed or Flexible Position | Flexible |
| - TFCI existence | TRUE |
| - Number of bits for Pilot bits(SF=128,256) | Not Present |
| - DPCH compressed mode info |  |
| - TGPSI | 1 |
| - TGPS Status Flag | Inactive |
| - TGCFN | (Current CFN + (256 – TTI/10msec)) mod 256 |
| - Transmission gap pattern sequence configuration parameters |  |
| - TGMP | FDD Measurement |
| - TGPRC | 62 |
| - TGSN | 8 |
| - TGL1 | 10 |
| - TGL2 | 5 |
| - TGD | 15 |
| - TGPL1 | 35 |
| - TGPL2 | 35 |
| - RPP | Mode 1 |
| - ITP | Mode 1 |
| - CHOICE UL/DL Mode | DL |
| - Downlink compressed mode method | SF/2 |
| - Downlink frame type | A |
| - DeltaSIR1 | 2.0 |
| - DeltaSIRafter1 | 1.0 |
| - DeltaSIR2 | Not Present |
| - DeltaSIRafter2 | Not Present |
| - TX Diversity mode | None |
| - SSDT information | Not Present |
| - Default DPCH Offset Value | 0 |
| Downlink information for each radio links |  |
| - Primary CPICH info |  |
| - Primary scrambling code | 100 |
| - PDSCH with SHO DCH info | Not Present |
| - PDSCH code mapping | Not Present |
| - Downlink DPCH info for each RL |  |
| - Primary CPICH usage for channel estimation | Primary CPICH may be used |
| - DPCH frame offset | 0 chips |
| - Secondary CPICH info | Not Present |
| - DL channelisation code |  |
| - Secondary scrambling code | 1 |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Code number | SF-1(SF is reference to TS34.108 clause 6.10 Parameter Set) |
| - Scrambling code change | No change |
| - TPC combination index | 0 |
| - SSDT Cell Identity | -a |
| - Closed loop timing adjustment mode | Not Present |
| - Secondary CCPCH info | Not Present |
| - TFCS | Not Present |
| - FACH/PCH information | Not Present |
| - References to system information blocks | Not Present |

# Contents of RADIO BEARER SETUP message: AM or UM (The others of speech in CS)

| Information Element | Value/remark |
| --- | --- |
| Message Type |  |
| RRC transaction identifier | Arbitrarily selects an integer between 0 and 3 |
| Integrity check info | The presence of this IE is dependent on IXIT statements in TS 34.123-2. If integrity protection is indicated to be active, this IE is present with the values of the sub IEs as stated below. Else, this IE and the sub-IEs are omitted. |
| - message authentication code | SS calculates the value of MAC-I for this message and writes to this IE. |
| - RRC message sequence number | SS provides the value of this IE, from its internal counter. |
| Integrity protection mode info | Not Present |
| Ciphering mode info | The presence of this IE is dependent on IXIT statements in TS 34.123-2. If ciphering is indicated to be active, this IE present with the values of the sub IEs as stated below. Else, this IE is omitted. |
| - Ciphering mode command | Start |
| - Ciphering algorithm | Use one of the supported ciphering algorithms |
| - Ciphering activation time for DPCH | (256+CFN-(CFN MOD 8 + 8))MOD 256 |
| - Radio bearer downlink ciphering activation time info | Not Present |
| Activation time | (256+CFN-(CFN MOD 8 + 8))MOD 256 |
| New U-RNTI | Not Present |
| New C-RNTI | Not Present |
| RRC State indicator | CELL_DCH |
| UTRAN DRX cycle length coefficient | Not Present |
| CN information info | Not Present |
| URA identity | Not Present |
| Signalling RB information to setup | Not Present |
| RAB information for setup |  |
| - RAB info |  |
| - RAB identity | 0000 0001B |
| - CN domain identity | CS domain |
| - NAS Synchronization Indicator | Not Present |
| - Re-establishment timer |  |
| - T314 | 20 seconds |
| - RB information to setup |  |
| - RB identity | 10 |
| - PDCP info | Not Present |
| - CHOICE RLC info type | RLC info |
| - CHOICE Uplink RLC mode | TM RLC |
| - Transmission RLC discard | Not Present |
| - Segmentation indication | TRUE |
| - CHOICE Downlink RLC mode | TM RLC |
| - Segmentation indication | TRUE |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 1 |
| - Logical channel identity | 7 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 1 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 6 |
| - Logical channel identity | 7 |
| RB information to be affected | (UM DCCH for RRC) |
| - RB identity | 1 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 1 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 1 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 1 |
| RB information to be affected | (AM DCCH for RRC) |
| - RB identity | 2 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 2 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 2 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 2 |
| RB information to be affected | (AM DCCH for NAS_DT High priority) |
| - RB identity | 3 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 3 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 3 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 3 |
| RB information to be affected | (AM DCCH for NAS_DT Low priority) |
| - RB identity | 4 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 4 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 4 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 4 |
| RB with PDCP information list | Not Present |
| UL Transport channel information for all transport channels |  |
| - TFC subset | (This IE is repeated for TFC number.) |
| - Allowed Transport Format combination | 0 to MaxTFCValue-1 (MaxTFCValue is refer to TS34.108 clause 6.10 Parameter Set.) |
| - PRACH TFCS | Not Present |
| - CHOICE mode | FDD |
| - UL DCH TFCS | (This IE is repeated for TFC number.) |
| - Normal |  |
| - TFCI Field 1 information |  |
| - CHOICE TFCS representation | Addition |
| - TFCS addition information |  |
| - CHOICE CTFC Size | Number of bits used must be enough to cover all combinations of CTFC from clause 6.10. |
| - CTFC information | Refer to TS34.108 clause 6.10 Parameter Set |
| - Power offset information |  |
| - CHOICE Gain Factors | Signalled Gain Factor |
| - Gain factor βc | 0 |
| - Gain factor βd | 0 |
| - Reference TFC ID | Not Present |
| - Power offset Pp-m | 0dB |
| Added or Reconfigured UL TrCH information |  |
| - Transport channel identity | 1 |
| - TFS |  |
| - CHOICE Transport channel type | Dedicated transport channels |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |
| - Transmission Time Interval | Not Present |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |
| - CHOICE Logical Channel list | ALL |
| - Semi-static Transport Format information |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |
| Added or Reconfigured UL TrCH information | If TrCH reconfiguration is executed then this is needed(e.g The rate of SRB for DCCH is changed.). |
| - Transport channel identity | 5 |
| - TFS |  |
| - CHOICE Transport channel type | Dedicated transport channels |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |
| - Transmission Time Interval | Not Present |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |
| - CHOICE Logical Channel list | ALL |
| - Semi-static Transport Format information |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |
| DRAC static information | Not Present |
| DL Transport channel information common for all transport channel |  |
| - SCCPCH TFCS | Not Present |
| - CHOICE DL parameters | Independent |
| - DL DCH TFCS | (This IE is repeated for TFC number.) |
| - Normal |  |
| - TFCI Field 1 information |  |
| - CHOICE TFCS representation | Addition |
| - TFCS addition information |  |
| - CHOICE CTFC Size | Number of bits used must be enough to cover all combinations of CTFC from clause 6.10. |
| - CTFC information | Refer to TS34.108 clause 6.10 Parameter Set |
| - Power offset information | Not Present |
| Added or Reconfigured DL TrCH information |  |
| - Transport channel identity | 6 |
| - CHOICE DL parameters | SameAsUL |
| UL TrCH identity | 1 |
| - DCH quality target |  |
| - BLER Quality value | -6.3 |
| - Transparent mode signalling info | Not Present |
| Added or Reconfigured DL TrCH information | If TrCH reconfiguration is executed then this is needed(e.g The rate of SRB for DCCH is changed.). |
| - Transport channel identity | 10 |
| - CHOICE DL parameters | Independent |
| - UL TrCH identity | 5 |
| - TFS |  |
| - CHOICE Transport channel type | Dedicated transport channels |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |
| - Transmission Time Interval | Not Present |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |
| - CHOICE Logical Channel list | ALL |
| - Semi-static Transport Format information |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |
| - DCH quality target |  |
| - BLER Quality value | -6.3 |
| - Transparent mode signalling info | Not Present |
| Frequency info |  |
| - UARFCN uplink(Nu) | Reference to TS34.108 clause 6.10 Parameter Set |
| - UARFCN downlink(Nd) | Reference to TS34.108 clause 6.10 Parameter Set |
| Maximum allowed UL TX power | 33dBm |
| Uplink DPCH info |  |
| - Uplink DPCH power control info |  |
| - DPCCH power offset | -6dB |
| - PC Preamble | 8slot |
| - Power Control Algorithm | Algorithm1 |
| - TPC step size | 1dB |
| - Scrambling code type | Long |
| - Scrambling code number | 0 (0 to 16777215) |
| - Number of DPDCH | Not Present(1) |
| - spreading factor | SF is reference to TS34.108 clause 6.10 Parameter Set |
| - TFCI existence | TRUE |
| - Number of FBI bit | Not Present(0) |
| - Puncturing Limit | Reference to TS34.108 clause 6.10 Parameter Set |
| CHOICE Mode | FDD |
| - Downlink PDSCH information | Not Present |
| Downlink information common for all radio links |  |
| - Downlink DPCH info common for all RL |  |
| - Timing indicator | Maintain |
| - CFN-targetSFN frame offset | Not Present |
| - Downlink DPCH power control information |  |
| - DPC mode | 0 (single) |
| - DL rate matching restriction information | Not Present |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Fixed or Flexible Position | Flexible |
| - TFCI existence | TRUE |
| - Number of bits for Pilot bits(SF=128,256) | Not Present |
| - DPCH compressed mode info |  |
| - TGPSI | 1 |
| - TGPS Status Flag | inactive |
| - TGCFN | (Current CFN + (256 – TTI/10msec)) mod 256 |
| - Transmission gap pattern sequence configuration parameters |  |
| - TGMP | FDD Measurement |
| - TGPRC | 62 |
| - TGSN | 8 |
| - TGL1 | 10 |
| - TGL2 | 5 |
| - TGD | 15 |
| - TGPL1 | 35 |
| - TGPL2 | 35 |
| - RPP | Mode 1 |
| - ITP | Mode 1 |
| - CHOICE UL/DL Mode | DL |
| - Downlink compressed mode method | SF/2 |
|  | \ |
| - Downlink frame type | A |
| - DeltaSIR1 | 2.0 |
| - DeltaSIRafter1 | 1.0 |
| - DeltaSIR2 | Not Present |
| - DeltaSIRafter2 | Not Present |
| - TX Diversity mode | None |
| - SSDT information | Not Present |
| - Default DPCH Offset Value | 0 |
| Downlink information for each radio links |  |
| - Primary CPICH info |  |
| - Primary scrambling code | 100 |
| - PDSCH with SHO DCH info | Not Present |
| - PDSCH code mapping | Not Present |
| - Downlink DPCH info for each RL |  |
| - Primary CPICH usage for channel estimation | Primary CPICH may be used |
| - DPCH frame offset | 0 chips |
| - Secondary CPICH info | Not Present |
| - DL channelisation code |  |
| - Secondary scrambling code | 1 |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Code number | SF-1(SF is reference to TS34.108 clause 6.10 Parameter Set) |
| - Scrambling code change | No change |
| - TPC combination index | 0 |
| - SSDT Cell Identity | -a |
| - Closed loop timing adjustment mode | Not Present |
| - Secondary CCPCH info | Not Present |
| - TFCS | Not Present |
| - FACH/PCH information | Not Present |
| - References to system information blocks | Not Present |

# Contents of RADIO BEARER SETUP message: AM or UM (Packet to CELL_DCH from CELL_DCH in PS)

| Information Element | Value/remark |  |
| --- | --- | --- |
| Message Type |  |  |
| RRC transaction identifier | Arbitrarily selects an integer between 0 and 3 |  |
| Integrity check info | The presence of this IE is dependent on IXIT statements in TS 34.123-2. If integrity protection is indicated to be active, this IE is present with the values of the sub IEs as stated below. Else, this IE and the sub-IEs are omitted. |  |
| - message authentication code | SS calculates the value of MAC-I for this message and writes to this IE. |  |
| - RRC message sequence number | SS provides the value of this IE, from its internal counter. |  |
| Integrity protection mode info | Not Present |  |
| Ciphering mode info | The presence of this IE is dependent on IXIT statements in TS 34.123-2. If ciphering is indicated to be active, this IE present with the values of the sub IEs as stated below. Else, this IE is omitted |  |
| - Ciphering mode command | Start |  |
| - Ciphering algorithm | Use one of the supported ciphering algorithms |  |
| - Ciphering activation time for DPCH | Not Present |  |
| - Radio bearer downlink ciphering activation time info |  |  |
| - Radio bearer identity | 10 |  |
| - RLC sequence number | Current RLC SN + 2 |  |
| Activation time | (256+CFN-(CFN MOD 8 + 8))MOD 256 |  |
| New U-RNTI | Not Present |  |
| New C-RNTI | Not Present |  |
| RRC State indicator | CELL_DCH |  |
| UTRAN DRX cycle length coefficient | Not Present |  |
| CN information info | Not Present |  |
| URA identity | Not Present |  |
| Signalling RB information to setup | Not Present |  |
| RAB information for setup |  |  |
| - RAB info |  |  |
| - RAB identity | 0000 0001B |  |
| - CN domain identity | PS domain |  |
| - NAS Synchronization Indicator | Not Present |  |
| - Re-establishment timer |  |  |
| - T314 | 20 seconds |  |
| - RB information to setup |  |  |
| - RB identity | 20 |  |
| - PDCP info | Not Present |  |
| - CHOICE RLC info type | RLC info |  |
| - CHOICE Uplink RLC mode | AM RLC |  |
| - Transmission RLC discard |  |  |
| - SDU discard mode | Max DAT retransmissions |  |
| - MAX_DAT | 4 |  |
| - Timer_MRW | 100 |  |
| - MaxMRW | 4 |  |
| - Transmission window size | 8 |  |
| - Timer_RST | 500 |  |
| - Max_RST | 4 |  |
| - Polling info |  |  |
| - Timer_poll_prohibit | 200 |  |
| - Timer_poll | 200 |  |
| - Poll_SDU | 1 |  |
| - Last transmission PU poll | TRUE |  |
| - Last retransmission PU poll | TRUE |  |
| - Poll_Windows | 99 |  |
| - CHOICE Downlink RLC mode | AM RLC |  |
| - In-sequence delivery | TRUE |  |
| - Receiving window size | 8 |  |
| - Downlink RLC status info |  |  |
| - Timer_status_prohibit | 200 |  |
| - Timer_EPC | 200 |  |
| - Missing PU indicator | TRUE |  |
| - RB mapping info |  |  |
| - Information for each multiplexing option |  |  |
| - RLC logical channel mapping indicator | Not Present |  |
| - Number of uplink RLC logical channels | 1 |  |
| - Uplink transport channel type | DCH |  |
| - UL Transport channel identity | 1 |  |
| - Logical channel identity | 7 |  |
| - CHOICE RLC size list | All |  |
| - MAC logical channel priority | 1 |  |
| - Downlink RLC logical channel info |  |  |
| - Number of downlink RLC logical channels | 1 |  |
| - Downlink transport channel type | DCH |  |
| - DL Transport channel identity | 6 |  |
| - Logical channel identity | 7 |  |
| RB information to be affected | (UM DCCH for RRC) |  |
| - RB identity | 1 |  |
| - RB mapping info |  |  |
| - Information for each multiplexing option |  |  |
| - RLC logical channel mapping indicator | Not Present |  |
| - Number of uplink RLC logical channels | 1 |  |
| - Uplink transport channel type | DCH |  |
| - Transport channel identity | 1 |  |
| - UL Logical channel identity | 5 |  |
| - CHOICE RLC size list | All |  |
| - MAC logical channel priority | 1 |  |
| - Downlink RLC logical channel info |  |  |
| - Number of downlink RLC logical channels | 1 |  |
| - Downlink transport channel type | DCH |  |
| - DL Transport channel identity | 10 |  |
| - Logical channel identity | 1 |  |
| RB information to be affected | (AM DCCH for RRC) |  |
| - RB identity | 2 |  |
| - RB mapping info |  |  |
| - Information for each multiplexing option |  |  |
| - RLC logical channel mapping indicator | Not Present |  |
| - Number of uplink RLC logical channels | 1 |  |
| - Uplink transport channel type | DCH |  |
| - UL Transport channel identity | 5 |  |
| - Logical channel identity | 2 |  |
| - MAC logical channel priority | 1 |  |
| - MAC logical channel priority | 2 |  |
| - Downlink RLC logical channel info |  |  |
| - Number of downlink RLC logical channels | 1 |  |
| - Downlink transport channel type | DCH |  |
| - DL Transport channel identity | 10 |  |
| - Logical channel identity | 2 |  |
| RB information to be affected | (AM DCCH for NAS_DT High priority) |  |
| - RB identity | 3 |  |
| - RB mapping info |  |  |
| - Information for each multiplexing option |  |  |
| - RLC logical channel mapping indicator | Not Present |  |
| - Number of uplink RLC logical channels | 1 |  |
| - Uplink transport channel type | DCH |  |
| - UL Transport channel identity | 5 |  |
| - Logical channel identity | 3 |  |
| - CHOICE RLC size list | All |  |
| - MAC logical channel priority | 3 |  |
| - Downlink RLC logical channel info |  |  |
| - Number of downlink RLC logical channels | 1 |  |
| - Downlink transport channel type | DCH |  |
| - DL Transport channel identity | 10 |  |
| - Logical channel identity | 3 |  |
| RB information to be affected | (AM DCCH for NAS_DT Low priority) |  |
| - RB identity | 4 |  |
| - RB mapping info |  |  |
| - Information for each multiplexing option |  |  |
| - RLC logical channel mapping indicator | Not Present |  |
| - Number of uplink RLC logical channels | 1 |  |
| - Uplink transport channel type | DCH |  |
| - UL Transport channel identity | 5 |  |
| - Logical channel identity | 4 |  |
| - CHOICE RLC size list | All |  |
| - MAC logical channel priority | 4 |  |
| - Downlink RLC logical channel info |  |  |
| - Number of downlink RLC logical channels | 1 |  |
| - Downlink transport channel type | DCH |  |
| - DL Transport channel identity | 10 |  |
| - Logical channel identity | 4 |  |
| RB with PDCP information list | Not Present |  |
| UL Transport channel information for all transport channels |  |  |
| - TFC subset | (This IE is repeated for TFC number.) |  |
| - Allowed Transport Format combination | 0 to MaxTFCValue-1 (MaxTFCValue is refer to TS34.108 clause 6.10 Parameter Set.) |  |
| - PRACH TFCS | Not Present |  |
| - CHOICE mode | FDD |  |
| - UL DCH TFCS | (This IE is repeated for TFC number.) |  |
| - Normal |  |  |
| - TFCI Field 1 information |  |  |
| - CHOICE TFCS representation | Addition |  |
| - TFCS addition information |  |  |
| - CHOICE CTFC Size | Number of bits used must be enough to cover all combinations of CTFC from clause 6.10. |  |
| - CTFC information | Refer to TS34.108 clause 6.10 Parameter Set |  |
| - Power offset information | Not Present |  |
| Added or Reconfigured UL TrCH information |  |  |
| - Transport channel identity | 1 |  |
| - TFS |  |  |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |  |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - RLC size | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - Semi-static Transport Format information |  |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |  |
| Added or Reconfigured UL TrCH information | If TrCH reconfiguration is executed then this is needed(e.g The rate of SRB for DCCH is changed.). |  |
| - Transport channel identity | 5 |  |
| - TFS |  |  |
| - CHOICE Transport channel type | Dedicated transport channels |  |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |  |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |  |
| - Transmission Time Interval | Not Present |  |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - CHOICE Logical Channel list | ALL |  |
| - Semi-static Transport Format information |  |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |  |
| DRAC static information | Not Present |  |
| DL Transport channel information common for all transport channel |  |  |
| - SCCPCH TFCS | Not Present |  |
| - CHOICE DL parameters | Independent |  |
| - DL DCH TFCS | (This IE is repeated for TFC number.) |  |
| - Normal |  |  |
| - TFCI Field 1 information |  |  |
| - CHOICE CTFC Size | Addition |  |
| - TFCS addition information |  |  |
| - CHOICE CTFC Size | Number of bits used must be enough to cover all combinations of CTFC from clause 6.10. |  |
| - CTFC information | Refer to TS34.108 clause 6.10 Parameter Set |  |
| - Power offset information | Not Present |  |
| Added or Reconfigured DL TrCH information |  |  |
| - Transport channel identity | 6 |  |
| - CHOICE DL parameters | Independent |  |
| - TFS |  |  |
| - CHOICE Transport channel type | Dedicated transport channels |  |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |  |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |  |
| - Transmission Time Interval | Not Present |  |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - CHOICE Logical Channel list | ALL |  |
| - Semi-static Transport Format information |  |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - DCH quality target |  |  |
| - BLER Quality value | -6.3 |  |
| - Transparent mode signalling info | Not Present |  |
| Added or Reconfigured DL TrCH information | If TrCH reconfiguration is executed then this is needed(e.g The rate of SRB for DCCH is changed.). |  |
| - Transport channel identity | 10 |  |
| - TFS |  |  |
| - CHOICE Transport channel type | Dedicated transport channels |  |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |  |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |  |
| - Transmission Time Interval | Not Present |  |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - CHOICE Logical Channel list | ALL |  |
| - Semi-static Transport Format information |  |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - DCH quality target |  |  |
| - BLER Quality value | -6.3 |  |
| - Transparent mode signalling info | Not Present |  |
| Frequency info |  |  |
| - UARFCN uplink(Nu) | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - UARFCN downlink(Nd) | Reference to TS34.108 clause 6.10 Parameter Set |  |
| Maximum allowed UL TX power | 33dBm |  |
| Uplink DPCH info |  |  |
| - Uplink DPCH power control info |  |  |
| - DPCCH power offset | -6dB |  |
| - PC Preamble | 8slot |  |
| - Power Control Algorithm | Algorithm1 |  |
| - TPC step size | 1dB |  |
| - Scrambling code type | Long |  |
| - Scrambling code number | 0 (0 to 16777215) |  |
| - Number of DPDCH | Not Present(1) |  |
| - spreading factor | SF is reference to TS34.108 clause 6.10 Parameter Set |  |
| - TFCI existence | TRUE |  |
| - Number of FBI bit | Not Present(0) |  |
| - Puncturing Limit | Reference to TS34.108 clause 6.10 Parameter Set |  |
| CHOICE Mode | FDD |  |
| - Downlink PDSCH information | Not Present |  |
| Downlink information common for all radio links |  |  |
| - Downlink DPCH info common for all RL |  |  |
| - Timing indicator | Maintain |  |
| - CFN-targetSFN frame offset | Not Present |  |
| - Downlink DPCH power control information |  |  |
| - DPC mode | 0 (single) |  |
| - DL rate matching restriction information | Not Present |  |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - Fixed or Flexible Position | Flexible |  |
| - TFCI existence | TRUE |  |
| - Number of bits for Pilot bits(SF=128,256) | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - DPCH compressed mode info |  |  |
| - TGPSI | 1 |  |
| - TGPS Status Flag | inactive |  |
| - TGCFN | (Current CFN + (256 – TTI/10msec)) mod 256 |  |
| - Transmission gap pattern sequence configuration parameters |  |  |
| - TGMP | FDD Measurement |  |
| - TGPRC | 62 |  |
| - TGSN | 8 |  |
| - TGL1 | 10 |  |
| - TGL2 | 5 |  |
| - TGD | 15 |  |
| - TGPL1 | 35 |  |
| - TGPL2 | 35 |  |
| - RPP | Mode 1 |  |
| - ITP | Mode 1 |  |
| -CHOICE UL/DL Mode | DL |  |
| - Downlink compressed mode method | SF/2 |  |
| - Downlink frame type | A |  |
| - DeltaSIR1 | 2.0 |  |
| - DeltaSIRafter1 | 1.0 |  |
| - DeltaSIR2 | Not Present |  |
| - DeltaSIRafter2 | Not Present |  |
| - TX Diversity mode | None |  |
| - SSDT information | Not Present |  |
| - Default DPCH Offset Value | 0 |  |
| Downlink information for each radio links |  |  |
| - Primary CPICH info |  |  |
| - Primary scrambling code | 100 |  |
| - PDSCH with SHO DCH info | Not Present |  |
| - PDSCH code mapping | Not Present |  |
| - Downlink DPCH info for each RL |  |  |
| - Primary CPICH usage for channel estimation | Primary CPICH may be used |  |
| - DPCH frame offset | 0 chips |  |
| - Secondary CPICH info | Not Present |  |
| - DL channelisation code |  |  |
| - Secondary scrambling code | 1 |  |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - Code number | SF-1(SF is reference to TS34.108 clause 6.10 Parameter Set) |  |
| - Scrambling code change | No change |  |
| - TPC combination index | 0 |  |
| - SSDT Cell Identity | -a |  |
| - Closed loop timing adjustment mode | Not Present |  |
| - Secondary CCPCH info | Not Present |  |
| - TFCS | Not Present |  |
| - FACH/PCH information | Not Present |  |
| - References to system information blocks | Not Present |  |

# Contents of RADIO BEARER SETUP message: AM or UM (Packet to CELL_DCH from CELL_FACH in PS)

| Information Element | Value/remark |
| --- | --- |
| Message Type |  |
| RRC transaction identifier | Arbitrarily selects an integer between 0 and 3 |
| Integrity check info | The presence of this IE is dependent on IXIT statements in TS 34.123-2. If integrity protection is indicated to be active, this IE is present with the values of the sub IEs as stated below. Else, this IE and the sub-IEs are omitted. |
| - message authentication code | SS calculates the value of MAC-I for this message and writes to this IE. |
| - RRC message sequence number | SS provides the value of this IE, from its internal counter. |
| Integrity protection mode info | Not Present |
| Ciphering mode info | The presence of this IE is dependent on IXIT statements in TS 34.123-2. If ciphering is indicated to be active, this IE present with the values of the sub IEs as stated below. Else, this IE is omitted. |
| - Ciphering mode command | Start |
| - Ciphering algorithm | Use one of the supported ciphering algorithms. |
| - Ciphering activation time for DPCH | Not Present |
| - Radio bearer downlink ciphering activation time info |  |
| - Radio bearer identity | 20 |
| - RLC sequence number | Current RLC SN + 2 |
| Activation time | (256+CFN-(CFN MOD 8 + 8))MOD 256 |
| New U-RNTI | Not Present |
| New C-RNTI | Not Present |
| RRC State indicator | CELL_DCH |
| UTRAN DRX cycle length coefficient | Not Present |
| CN information info | Not Present |
| URA identity | Not Present |
| Signalling RB information to setup | Not Present |
| RAB information for setup |  |
| - RAB info |  |
| - RAB identity | 0000 0001B |
| - CN domain identity | PS domain |
| - NAS Synchronization Indicator | Not Present |
| - Re-establishment timer |  |
| - T314 | 20 seconds |
| - RB information to setup |  |
| - RB identity | 20 |
| - PDCP info | Not Present |
| - CHOICE RLC info type | RLC info |
| - CHOICE Uplink RLC mode | AM RLC |
| - Transmission RLC discard |  |
| - CHOICE SDU discard mode | Max DAT retransmissions |
| - MAX_DAT | 4 |
| - Timer_MRW | 100 |
| - MaxMRW | 4 |
| - Transmission window size | 8 |
| - Timer_RST | 500 |
| - Max_RST | 4 |
| - Polling info |  |
| - Timer_poll_prohibit | 200 |
| - Timer_poll | 200 |
| - Poll_SDU | 1 |
| - Last transmission PU poll | TRUE |
| - Last retransmission PU poll | TRUE |
| - Poll_Windows | 99 |
| - CHOICE Downlink RLC mode | AM RLC |
| - In-sequence delivery | TRUE |
| - Receiving window size | 8 |
| - Downlink RLC status info |  |
| - Timer_status_prohibit | 200 |
| - Timer_EPC | 200 |
| - Missing PU indicator | TRUE |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 1 |
| - Logical channel identity | 7 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 1 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 6 |
| - Logical channel identity | 7 |
| RB information to be affected | (UM DCCH for RRC) |
| - RB identity | 1 |
| - RB mapping info |  |
| - RLC logical channel mapping indicator | Not Present |
| - Information for each multiplexing option |  |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 1 |
| - Logical channel identity | 5 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 1 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - UL Transport channel identity | 10 |
| - Logical channel identity | 1 |
| RB information to be affected | (AM DCCH for RRC) |
| - RB identity | 2 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 2 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 2 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - UL Transport channel identity | 10 |
| - Logical channel identity | 2 |
| RB information to be affected | (AM DCCH for NAS_DT High priority) |
| - RB identity | 3 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 3 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 3 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 3 |
| RB information to be affected | (AM DCCH for NAS_DT Low priority) |
| - RB identity | 4 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 4 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 4 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 4 |
| RB with PDCP information list | Not Present |
| UL Transport channel information for all transport channels |  |
| - TFC subset | (This IE is repeated for TFC number.) |
| - Allowed Transport Format combination | 0 to MaxTFCValue-1 (MaxTFCValue is refer to TS34.108 clause 6.10 Parameter Set.) |
| - PRACH TFCS | Not Present |
| - CHOICE mode | FDD |
| - UL DCH TFCS | (This IE is repeated for TFC number.) |
| - Normal |  |
| - TFCI Field 1 information |  |
| - CHOICE TFCS representation | Addition |
| - TFCS addition information |  |
| - CHOICE CTFC Size | Number of bits used must be enough to cover all combinations of CTFC from clause 6.10. |
| - CTFC information | Refer to TS34.108 clause 6.10 Parameter Set |
| - Power offset information |  |
| - CHOICE Gain Factors | Signalled Gain Factor |
| - Gain factor βc | 0 |
| - Gain factor βd | 0 |
| - Reference TFC ID | Not Present |
| - Power offset Pp-m | 0dB |
| Deleted UL TrCH information |  |
| - Transport channel identity | 15 |
| Added or Reconfigured UL TrCH information |  |
| - Transport channel identity | 1 |
| - TFS |  |
| - CHOICE Transport channel type | Dedicated transport channels |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |
| - Transmission Time Interval | Not Present |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |
| - CHOICE Logical Channel list | ALL |
| - Semi-static Transport Format information |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |
| Added or Reconfigured UL TrCH information |  |
| - Transport channel identity | 5 |
| - TFS |  |
| - CHOICE Transport channel type | Dedicated transport channels |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |
| - Transmission Time Interval | Not Present |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |
| - CHOICE Logical Channel list | ALL |
| - Semi-static Transport Format information |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |
| DRAC static information | Not Present |
| DL Transport channel information common for all transport channel |  |
| - SCCPCH TFCS | Not Present |
| - CHOICE DL parameters | Independent |
| - DL DCH TFCS | (This IE is repeated for TFC number.) |
| - Normal |  |
| - TFCI Field 1 information |  |
| - CHOICE TFCS representation | Addition |
| - TFCS addition information |  |
| - CHOICE CTFC Size | Number of bits used must be enough to cover all combinations of CTFC from clause 6.10. |
| - CTFC information | Refer to TS34.108 clause 6.10 Parameter Set |
| - Power offset information | Not Present |
| Deleted DL TrCH information |  |
| - Transport channel identity | 12 |
| - Transport channel identity | 13 |
| - Transport channel identity | 14 |
| Added or Reconfigured DL TrCH information |  |
| - Transport channel identity | 6 |
| - CHOICE DL parameters | Independent |
| - TFS |  |
| - CHOICE Transport channel type | Dedicated transport channels |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |
| - Transmission Time Interval | Not Present |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |
| - CHOICE Logical Channel list | ALL |
| - Semi-static Transport Format information |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |
| - DCH quality target |  |
| - BLER Quality value | -6.3 |
| - Transparent mode signalling info | Not Present |
| Added or Reconfigured DL TrCH information |  |
| - Transport channel identity | 10 |
| - CHOICE DL parameters | Independent |
| - TFS |  |
| - CHOICE Transport channel type | Dedicated transport channels |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |
| - Transmission Time Interval | Not Present |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |
| - CHOICE Logical Channel list | ALL |
| - Semi-static Transport Format information |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |
| - DCH quality target |  |
| - BLER Quality value | -6.3 |
| - Transparent mode signalling info | Not Present |
| Frequency info |  |
| - UARFCN uplink(Nu) | Reference to TS34.108 clause 6.10 Parameter Set |
| - UARFCN downlink(Nd) | Reference to TS34.108 clause 6.10 Parameter Set |
| Maximum allowed UL TX power | 33dBm |
| Uplink DPCH info |  |
| - Uplink DPCH power control info |  |
| - DPCCH power offset | -6dB |
| - PC Preamble | 8slot |
| - Power Control Algorithm | Algorithm1 |
| - TPC step size | 1dB |
| - Scrambling code type | Long |
| - Scrambling code number | 0 (0 to 16777215) |
| - Number of DPDCH | Not Present(1) |
| - spreading factor | SF is reference to TS34.108 clause 6.10 Parameter Set |
| - TFCI existence | TRUE |
| - Number of FBI bit | Not Present(0) |
| - Puncturing Limit | Reference to TS34.108 clause 6.10 Parameter Set |
| CHOICE Mode | FDD |
| - Downlink PDSCH information | Not Present |
| Downlink information common for all radio links |  |
| - Downlink DPCH info common for all RL |  |
| - Timing indicator | Maintain |
| - CFN-targetSFN frame offset | Not Present |
| - Downlink DPCH power control information |  |
| - DPC mode | 0 (single) |
| - DL rate matching restriction information | Not Present |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Fixed or Flexible Position | Flexible |
| - TFCI existence | TRUE |
| - Number of bits for Pilot bits(SF=128,256) | Reference to TS34.108 clause 6.10 Parameter Set |
| - DPCH compressed mode info |  |
| - TGPSI | 1 |
| - TGPS Status Flag | inactive |
| - TGCFN | (Current CFN + (256 – TTI/10msec)) mod 256 |
| - Transmission gap pattern sequence configuration parameters |  |
| - TGMP | FDD Measurement |
| - TGPRC | 62 |
| - TGSN | 8 |
| - TGL1 | 10 |
| - TGL2 | 5 |
| - TGD | 15 |
| - TGPL1 | 35 |
| - TGPL2 | 35 |
| - RPP | Mode 1 |
| - ITP | Mode 1 |
| - CHOICE UL/DL Mode | DL |
| - Downlink compressed mode method | SF/2 |
| - Downlink frame type | A |
| - DeltaSIR1 | 2.0 |
| - DeltaSIRafter1 | 1.0 |
| - DeltaSIR2 | Not Present |
| - DeltaSIRafter2 | Not Present |
| - TX Diversity mode | None |
| - SSDT information | Not Present |
| - Default DPCH Offset Value | 0 |
| Downlink information for each radio links |  |
| - Primary CPICH info |  |
| - Primary scrambling code | 100 |
| - PDSCH with SHO DCH info | Not Present |
| - PDSCH code mapping | Not Present |
| - Downlink DPCH info for each RL |  |
| - Primary CPICH usage for channel estimation | Primary CPICH may be used |
| - DPCH frame offset |  |
| - Secondary CPICH info | Not Present |
| - DL channelisation code |  |
| - Secondary scrambling code | 1 |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Code number | SF-1(SF is reference to TS34.108 clause 6.10 Parameter Set) |
| - Scrambling code change | No change |
| - TPC combination index | 0 |
| - SSDT Cell Identity | -a |
| - Closed loop timing adjustment mode | Not Present |
| - Secondary CCPCH info | Not Present |
| - TFCS | Not Present |
| - FACH/PCH information | Not Present |
| - References to system information blocks | Not Present |

# Contents of RADIO BEARER RECONFIGURATION message: AM or UM (The others of speech in CS)

| Information Element | Value/remark |
| --- | --- |
| Message Type |  |
| RRC transaction identifier | Arbitrarily selects an integer between 0 and 3 |
| Integrity check info | The presence of this IE is dependent on IXIT statements in TS 34.123-2. If integrity protection is indicated to be active, this IE is present with the values of the sub IEs as stated below. Else, this IE and the sub-IEs are omitted. |
| - message authentication code | SS calculates the value of MAC-I for this message and writes to this IE. |
| - RRC message sequence number | SS provides the value of this IE, from its internal counter. |
| Integrity protection mode info | Not Present |
| Ciphering mode info | Not Present |
| Activation time | (256+CFN-(CFN MOD 8 + 8))MOD 256 |
| New U-RNTI | Not Present |
| New C-RNTI | Not Present |
| RRC State indicator | CELL_DCH |
| UTRAN DRX cycle length coefficient | Not Present |
| CN information info | Not Present |
| URA identity | Not Present |
| RAB information to reconfigure list | Not Present |
| RB information to reconfigure list | Not Present |
| RB information to be affected | (UM DCCH for RRC) |
| - RB identity | 1 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 1 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 1 |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 1 |
| RB information to be affected | (AM DCCH for RRC) |
| - RB identity | 2 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 2 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 2 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 10 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 1 |
| - Logical channel identity | 2 |
| RB information to be affected | (AM DCCH for NAS_DT High priority) |
| - RB identity | 3 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 3 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 3 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 3 |
| RB information to be affected | (AM DCCH for NAS_DT Low priority) |
| - RB identity | 4 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 4 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 4 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 4 |
| RB information to be affected | (TM DTCH) |
| - RB identity | 10 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 1 |
| - Logical channel identity | 7 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 1 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 6 |
| - Logical channel identity | 7 |
| UL Transport channel information for all transport channels |  |
| - TFC subset | (This IE is repeated for TFC number.) |
| - Allowed Transport Format combination | 0 to MaxTFCValue-1 (MaxTFCValue is refer to TS34.108 clause 6.10 Parameter Set.) |
| - PRACH TFCS | Not Present |
| - CHOICE mode | FDD |
| - UL DCH TFCS | (This IE is repeated for TFC number.) |
| - Normal |  |
| - TFCI Field 1 information |  |
| - CHOICE TFCS representation | Addition |
| - TFCS addition information |  |
| - CHOICE CTFC Size |  |
| - CTFC information | Refer to TS34.108 clause 6.10 Parameter Set |
| - Power offset information |  |
| - CHOICE Gain Factor | Signalled Gain Factor |
| - Gain factor βc | 0 |
| - Gain factor βd | 0 |
| - Reference TFC ID | Not Present |
| - Power offset Pp-m | 0dB |
| Deleted UL TrCH information | Not Present |
| - Transport channel identity |  |
| Added or Reconfigured UL TrCH information |  |
| - Transport channel identity | 5 |
| - TFS |  |
| - CHOICE Transport channel type | Dedicated transport channels |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |
| - Transmission Time Interval | Not Present |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |
| - CHOICE Logical Channel list | ALL |
| - Semi-static Transport Format information |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |
| DRAC static information | Not Present |
| DL Transport channel information common for all transport channel |  |
| - SCCPCH TFCS | Not Present |
| - CHOICE DL parameters | Independent |
| - DL DCH TFCS | (This IE is repeated for TFC number.) |
| - Normal |  |
| - TFCI Field 1 information |  |
| - CHOICE TFCS representation | Addition |
| - TFCS addition information |  |
| - CHOICE CTFC Size | Number of bits used must be enough to cover all combinations of CTFC from clause 6.10. |
| - CTFC information | Refer to TS34.108 clause 6.10 Parameter Set |
| - Power offset information |  |
| - CHOICE Gain Factors | Signalled Gain Factor |
| - Gain factor βc | 0 |
| - Gain factor βd | 0 |
| - Reference TFC ID | Not Present |
| - Power offset Pp-m | 0dB |
| Deleted DL TrCH information | Not Present |
| Added or Reconfigured DL TrCH information |  |
| - Transport channel identity | 10 |
| - CHOICE DL parameters | SameAsUL |
| - UL TrCH Identity | 5 |
| - DCH quality target |  |
| - BLER Quality value | -6.3 |
| - Transparent mode signalling info | Not Present |
| Frequency info |  |
| - UARFCN uplink(Nu) | Reference to TS34.108 clause 6.10 Parameter Set |
| - UARFCN downlink(Nd) | Reference to TS34.108 clause 6.10 Parameter Set |
| Maximum allowed UL TX power | 33dBm |
| Uplink DPCH info |  |
| -Uplink DPCH power control info |  |
| - DPCCH power offset | -6dB |
| - PC Preamble | 8slot |
| - Power Control Algorithm | Algorithm1 |
| - TPC step size | 1dB |
| - Scrambling code type | Long |
| - Scrambling code number | 0 (0 to 16777215) |
| - Number of DPDCH | Not Present(1) |
| - spreading factor | SF is reference to TS34.108 clause 6.10 Parameter Set |
| - TFCI existence | TRUE |
| - Number of FBI bit | Not Present(0) |
| - Puncturing Limit | Reference to TS34.108 clause 6.10 Parameter Set |
| CHOICE Mode | FDD |
| - Downlink PDSCH information | Not Present |
| Downlink information common for all radio links |  |
| - Downlink DPCH info common for all RL |  |
| - Timing indicator | Maintain |
| - CFN-targetSFN frame offset | Not Present |
| - Downlink DPCH power control information |  |
| - DPC mode | 0 (single) |
| - DL rate matching restriction information | Not Present |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Fixed or Flexible Position | Flexible |
| - TFCI existence | TRUE |
| - Number of bits for Pilot bits(SF=128,256) | Not Present |
| - DPCH compressed mode info |  |
| - TGPSI | 1 |
| - TGPS Status Flag | Inactive |
| - TGCFN | (Current CFN + (256 – TTI/10msec)) mod 256 |
| - Transmission gap pattern sequence configuration parameters |  |
| - TGMP | FDD Measurement |
| - TGPRC | 62 |
| - TGSN | 8 |
| - TGL1 | 10 |
| - TGL2 | 5 |
| - TGD | 15 |
| - TGPL1 | 35 |
| - TGPL2 | 35 |
| - RPP | Mode 1 |
| - ITP | Mode 1 |
| - CHOICE UL/DL Mode | DL |
| - Downlink compressed mode method | SF/2 |
| - Downlink frame type | A |
| - DeltaSIR1 | 2.0 |
| - DeltaSIRafter1 | 1.0 |
| - DeltaSIR2 | Not Present |
| - DeltaSIRafter2 | Not Present |
| - TX Diversity mode | None |
| - SSDT information | Not Present |
| - Default DPCH Offset Value | 0 |
| Downlink information for each radio links |  |
| - Primary CPICH info | 100 |
| - Primary scrambling code | 100 |
| - PDSCH with SHO DCH info | Not Present |
| - PDSCH code mapping | Not Present |
| - Downlink DPCH info for each RL |  |
| - Primary CPICH usage for channel estimation | Primary CPICH may be used |
| - DPCH frame offset | 0 chips |
| - Secondary CPICH info | Not Present |
| - Secondary scrambling code |  |
| - channelisation code |  |
| - DL channelisation code |  |
| - Secondary scrambling code | 1 |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Code number | SF-1(SF is reference to TS34.108 clause 6.10 Parameter Set) |
| - Scrambling code change | No change |
| - TPC combination index | 0 |
| - SSDT Cell Identity | -a |
| - Closed loop timing adjustment mode | Not Present |
| - Secondary CCPCH info | Not Present |
| - TFCS | Not Present |
| - FACH/PCH information | Not Present |
| - References to system information blocks | Not Present |

# Contents of RADIO BEARER RECONFIGURATION message: AM or UM (Speech in CS)

| Information Element | Value/remark |
| --- | --- |
| Message Type |  |
| RRC transaction identifier | Arbitrarily selects an integer between 0 and 3 |
| Integrity check info | The presence of this IE is dependent on IXIT statements in TS 34.123-2. If integrity protection is indicated to be active, this IE is present with the values of the sub IEs as stated below. Else, this IE and the sub-IEs are omitted. |
| - message authentication code | SS calculates the value of MAC-I for this message and writes to this IE. |
| - RRC message sequence number | SS provides the value of this IE, from its internal counter. |
| Integrity protection mode info | Not Present |
| Ciphering mode info | Not Present |
| Activation time | (256+CFN-(CFN MOD 8 + 8))MOD 256 |
| New U-RNTI | Not Present |
| New C-RNTI | Not Present |
| RRC State indicator | CELL_DCH |
| UTRAN DRX cycle length coefficient | Not Present |
| CN information info | Not Present |
| URA identity | Not Present |
| RAB information to reconfigure list | Not Present |
| RB information to reconfigure list | Not Present |
| RB information to be affected | (UM DCCH for RRC) |
| - RB identity | 1 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 1 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 1 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 1 |
| RB information to be affected | (AM DCCH for RRC) |
| - RB identity | 2 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 2 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 2 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 2 |
| RB information to be affected | (AM DCCH for NAS_DT High priority) |
| - RB identity | 3 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 3 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 3 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 3 |
| RB information to be affected | (AM DCCH for NAS_DT Low priority) |
| - RB identity | 4 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 4 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 4 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 4 |
| RB information to be affected | (DTCH TM) |
| - RB identity | 10 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 1 |
| - Logical channel identity | 7 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 1 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 6 |
| - Logical channel identity | 7 |
| RB information to be affected | (DTCH TM) |
| - RB identity | 11 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 2 |
| - Logical channel identity | 8 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 1 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 7 |
| - Logical channel identity | 8 |
| RB information to be affected | (This IE is needed for 12.2 kbps and 10.2 kbps) |
| - RB identity | 12 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 3 |
| - Logical channel identity | 9 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 1 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 8 |
| - Logical channel identity | 9 |
| UL Transport channel information for all transport channels |  |
| - TFC subset | (This IE is repeated for TFC number.) |
| - Allowed Transport Format combination | 0 to MaxTFCValue-1 (MaxTFCValue is refer to TS34.108 clause 6.10 Parameter Set.) |
| - PRACH TFCS | Not Present |
| - CHOICE mode | FDD |
| - UL DCH TFCS | (This IE is repeated for TFC number.) |
| - Normal |  |
| - TFCI Field 1 information |  |
| - CHOICE TFCS representation | Addition |
| - TFCS addition information |  |
| - CHOICE CTFC Size | Number of bits used must be enough to cover all combinations of CTFC from clause 6.10. |
| - CTFC information | Refer to TS34.108 clause 6.10 Parameter Set |
| - Power offset information |  |
| - CHOICE Gain Factors | Signalled Gain Factor |
| - Gain factor βc | 0 |
| - Gain factor βd | 0 |
| - Reference TFC ID | Not Present |
| - Power offset Pp-m | 0dB |
| Deleted UL TrCH information | Not Present |
| - Transport channel identity |  |
| Added or Reconfigured UL TrCH information |  |
| - Transport channel identity | 5 |
| - TFS |  |
| - CHOICE Transport channel type | Dedicated transport channels |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |
| - Transmission Time Interval | Not Present |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |
| - CHOICE Logical Channel list | ALL |
| - Semi-static Transport Format information |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |
| DRAC static information | Not Present |
| DL Transport channel information common for all transport channel |  |
| - SCCPCH TFCS | Not Present |
| - CHOICE DL parameters | Independent |
| - DL DCH TFCS | (This IE is repeated for TFC number.) |
| - Normal |  |
| - TFCI Field 1 information |  |
| - CHOICE TFCS representation | Addition |
| - TFCS addition information |  |
| - CHOICE CTFC Size | Number of bits used must be enough to cover all combinations of CTFC from clause 6.10. |
| - CTFC information | Refer to TS34.108 clause 6.10 Parameter Set |
| - Power offset information | Not Present |
| Deleted DL TrCH information | Not Present |
| Added or Reconfigured DL TrCH information |  |
| - Transport channel identity | 10 |
| - CHOICE DL parameters | SameAsUL |
| - UL TrCH Identity | 5 |
| - DCH quality target |  |
| - BLER Quality value | -6.3 |
| - Transparent mode signalling info | Not Present |
| Frequency info |  |
| - UARFCN uplink(Nu) | Reference to TS34.108 clause 6.10 Parameter Set |
| - UARFCN downlink(Nd) | Reference to TS34.108 clause 6.10 Parameter Set |
| Maximum allowed UL TX power | 33dBm |
| Uplink DPCH info |  |
| - Uplink DPCH power control info |  |
| - DPCCH power offset | -6dB |
| - PC Preamble | 8slot |
| - Power Control Algorithm | Algorithm1 |
| - TPC step size | 1dB |
| - Scrambling code type | Long |
| - Scrambling code number | 0 (0 to 16777215) |
| - Number of DPDCH | Not Present(1) |
| - spreading factor | SF is reference to TS34.108 clause 6.10 Parameter Set |
| - TFCI existence | TRUE |
| - Number of FBI bit | Not Present(0) |
| - Puncturing Limit | Reference to TS34.108 clause 6.10 Parameter Set |
| CHOICE Mode | FDD |
| - Downlink PDSCH information | Not Present |
| Downlink information common for all radio links |  |
| - Downlink DPCH info common for all RL |  |
| - Timing indicator | Maintain |
| - CFN-targetSFN frame offset | Not Present |
| - Downlink DPCH power control information |  |
| - DPC mode | 0 (single) |
| - DL rate matching restriction information | Not Present |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Fixed or Flexible Position | Flexible |
| - TFCI existence | TRUE |
| - Number of bits for Pilot bits(SF=128,256) | Not Present |
| - DPCH compressed mode info |  |
| - TGPSI | 1 |
| - TGPS Status Flag | Inactive |
| - TGCFN | (Current CFN + (256 – TTI/10msec)) mod 256 |
| - Transmission gap pattern sequence configuration parameters |  |
| - TGMP | FDD Measurement |
| - TGPRC | 62 |
| - TGSN | 8 |
| - TGL1 | 10 |
| - TGL2 | 5 |
| - TGD | 15 |
| - TGPL1 | 35 |
| - TGPL2 | 35 |
| - RPP | Mode 1 |
| - ITP | Mode 1 |
| - CHOICE UL/DL Mode | DL |
| - Downlink compressed mode method | SF/2 |
| - Downlink frame type | A |
| - DeltaSIR1 | 2.0 |
| - DeltaSIRafter1 | 1.0 |
| - DeltaSIR2 | Not Present |
| - DeltaSIRafter2 | Not Present |
| - TX Diversity mode | None |
| - SSDT information | Not Present |
| - S field |  |
| - Code Word Set |  |
| - Default DPCH Offset Value | 0 |
| Downlink information for each radio links |  |
| - Primary CPICH info |  |
| - Primary scrambling code | 100 |
| - PDSCH with SHO DCH info | Not Present |
| - PDSCH code mapping | Not Present |
| - Downlink DPCH info for each RL |  |
| - Primary CPICH usage for channel estimation | Primary CPICH may be used |
| - DPCH frame offset | 0 chips |
| - Secondary CPICH info | Not Present |
| - DL channelisation code |  |
| - Secondary scrambling code | 1 |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Code number | SF-1(SF is reference to TS34.108 clause 6.10 Parameter Set) |
| - Scrambling code change | No change |
| - TPC combination index | 0 |
| - SSDT Cell Identity | -a |
| - Closed loop timing adjustment mode | Not Present |
| - Secondary CCPCH info | Not Present |
| - TFCS | Not Present |
| - FACH/PCH information | Not Present |
| - References to system information blocks | Not Present |

# Contents of RADIO BEARER RECONFIGURATION message: AM or UM (Packet to CELL_DCH from CELL_DCH in PS)

| Information Element | Value/remark |
| --- | --- |
| Message Type |  |
| RRC transaction identifier | Arbitrarily selects an integer between 0 and 3 |
| Integrity check info | The presence of this IE is dependent on IXIT statements in TS 34.123-2. If integrity protection is indicated to be active, this IE is present with the values of the sub IEs as stated below. Else, this IE and the sub-IEs are omitted. |
| - message authentication code | SS calculates the value of MAC-I for this message and writes to this IE. |
| - RRC message sequence number | SS provides the value of this IE, from its internal counter. |
| Integrity protection mode info | Not Present |
| Ciphering mode info | Not Present |
| Activation time | (256+CFN-(CFN MOD 8 + 8))MOD 256 |
| New U-RNTI | Not Present |
| New C-RNTI | Not Present |
| RRC State indicator | CELL_DCH |
| UTRAN DRX cycle length coefficient | Not Present |
| CN information info | Not Present |
| URA identity | Not Present |
| RAB information to reconfigure list | Not Present |
| RB information to reconfigure list | Not Present |
| RB information to be affected | (UM DCCH for RRC) |
| - RB identity | 1 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 1 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 1 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 1 |
| RB information to be affected | (AM DCCH for RRC) |
| - RB identity | 2 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 2 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 2 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 2 |
| RB information to be affected | (AM DCCH for NAS_DT High priority) |
| - RB identity | 3 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 3 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 3 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 3 |
| RB information to be affected | (AM DCCH for NAS_DT Low priority) |
| - RB identity | 4 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 4 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 4 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 4 |
| RB information to be affected | (AM DTCH) |
| - RB identity | 20 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 1 |
| - Logical channel identity | 7 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 1 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 6 |
| - Logical channel identity | 7 |
| UL Transport channel information for all transport channels |  |
| - TFC subset | (This IE is repeated for TFC number.) |
| - Allowed Transport Format combination | 0 to MaxTFCValue-1 (MaxTFCValue is refer to TS34.108 clause 6.10 Parameter Set.) |
| - PRACH TFCS | Not Present |
| - CHOICE mode | FDD |
| - UL DCH TFCS | (This IE is repeated for TFC number.) |
| - Normal |  |
| - TFCI Field 1 information |  |
| - CHOICE TFCS representation | Addition |
| - TFCS addition information |  |
| - CHOICE CTFC Size | Number of bits used must be enough to cover all combinations of CTFC from clause 6.10. |
| - CTFC information | Refer to TS34.108 clause 6.10 Parameter Set |
| - Power offset information |  |
| - CHOICE Gain Factors | Signalled Gain Factor |
| - Gain factor βc | 0 |
| - Gain factor βd | 0 |
| - Reference TFC ID | Not Present |
| - Power offset Pp-m | 0dB |
| Deleted UL TrCH information |  |
| - Transport channel identity | Not Present |
| Added or Reconfigured UL TrCH information |  |
| - Transport channel identity | 5 |
| - TFS |  |
| - CHOICE Transport channel type | Dedicated transport channels |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |
| - Transmission Time Interval | Not Present |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |
| - CHOICE Logical Channel list | ALL |
| - Semi-static Transport Format information |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |
| DRAC static information | Not Present |
| DL Transport channel information common for all transport channel |  |
| - SCCPCH TFCS | Not Present |
| - CHOICE DL parameters | Independent |
| - DL DCH TFCS | (This IE is repeated for TFC number.) |
| - Normal |  |
| - TFCI Field 1 information |  |
| - CHOICE TFCS representation | Additon |
| - TFCS addition information |  |
| - CHOICE CTFC Size | Number of bits used must be enough to cover all combinations of CTFC from clause 6.10. |
| - CTFC information | Refer to TS34.108 clause 6.10 Parameter Set |
| - Power offset information |  |
| - CHOICE Gain Factors | Signalled Gain Factor |
| - Gain factor βc | 0 |
| - Gain factor βd | 0 |
| - Reference TFC ID | Not Present |
| - Power offset Pp-m | 0dB |
| Deleted DL TrCH information |  |
| - Transport channel identity | Not Present |
| Added or Reconfigured DL TrCH information |  |
| - Transport channel identity | 10 |
| - CHOICE DL parameters | Independent |
| - TFS |  |
| - CHOICE Transport channel type | Dedicated transport channels |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |
| - Transmission Time Interval | Not Present |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |
| - CHOICE Logical Channel list | ALL |
| - Semi-static Transport Format information |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |
| - DCH quality target |  |
| - BLER Quality value | -6.3 |
| - Transparent mode signalling info | Not Present |
| Frequency info |  |
| - UARFCN uplink(Nu) | Reference to TS34.108 clause 6.10 Parameter Set |
| - UARFCN downlink(Nd) | Reference to TS34.108 clause 6.10 Parameter Set |
| Maximum allowed UL TX power | 33dBm |
| Uplink DPCH info |  |
| -Uplink DPCH power control info |  |
| -DPCCH Power offset | -6dB |
| -PC Preamble | 8slot |
| -Power Control Algorithm | Algorithm1 |
| -TPC step size | 1dB |
| - Scrambling code type | Long |
| - Scrambling code number | 0 (0 to 16777215) |
| - Number of DPDCH | Not Present(1) |
| - spreading factor | SF is reference to TS34.108 clause 6.10 Parameter Set |
| - TFCI existence | TRUE |
| - Number of FBI bits | Not Present(0) |
| - Puncturing Limit | Reference to TS34.108 clause 6.10 Parameter Set |
| CHOICE Mode | FDD |
| - Downlink PDSCH information | Not Present |
| Downlink information common for all radio links | Not Present |
| Downlink information for each radio links |  |
| - Primary CPICH info |  |
| - Primary scrambling code | 100 |
| - PDSCH with SHO DCH info | Not Present |
| - PDSCH code mapping | Not Present |
| - Downlink DPCH info for each RL |  |
| - Primary CPICH usage for channel estimation | Primary CPICH may be used |
| - DPCH frame offset | 0 chips |
| - Secondary CPICH info | Not Present |
| - DL channelisation code |  |
| - Secondary scrambling code | 1 |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Code number | SF-1(SF is reference to TS34.108 clause 6.10 Parameter Set) |
| - Scrambling code change | No change |
| - TPC combination index | 0 |
| - SSDT Cell Identity | -a |
| - Closed loop timing adjustment mode | Not Present |
| - Secondary CCPCH info | Not Present |
| - TFCS | Not Present |
| - FACH/PCH information | Not Present |
| - References to system information blocks | Not Present |

# Contents of RADIO BEARER RECONFIGURATION message: AM or UM (Packet to CELL_DCH from CELL_FACH in PS)

| Information Element | Value/remark |
| --- | --- |
| Message Type |  |
| RRC transaction identifier | Arbitrarily selects an integer between 0 and 3 |
| Integrity check info | The presence of this IE is dependent on IXIT statements in TS 34.123-2. If integrity protection is indicated to be active, this IE is present with the values of the sub IEs as stated below. Else, this IE and the sub-IEs are omitted. |
| - message authentication code | SS calculates the value of MAC-I for this message and writes to this IE. |
| - RRC message sequence number | SS provides the value of this IE, from its internal counter. |
| Integrity protection mode info | Not Present |
| Ciphering mode info | Not Present |
| Activation time | (256+CFN-(CFN MOD 8 + 8))MOD 256 |
| New U-RNTI | Not Present |
| New C-RNTI | Not Present |
| RRC State indicator | CELL_DCH |
| UTRAN DRX cycle length coefficient | Not Present |
| CN information info | Not Present |
| URA identity | Not Present |
| RAB information to reconfigure list | Not Present |
| RB information to reconfigure list |  |
| - RB information to reconfigure | (UM DCCH for RRC) |
| - RB identity | 1 |
| - PDCP info | Not Present |
| - CHOICE RLC info type | Not Present |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 1 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 1 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 1 |
| - RB stop/continue | Not Present |
| - RB information to reconfigure | (AM DCCH for RRC) |
| - RB identity | 2 |
| - PDCP info | Not Present |
| - CHOICE RLC info type | Not Present |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 5 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 1 |
| - Logical channel identity | 2 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 2 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 2 |
| - RB stop/continue | Not Present |
| - RB information to reconfigure | (AM DCCH for NAS_DT High priority) |
| - RB identity | 3 |
| - PDCP info | Not Present |
| - CHOICE RLC info type | Not Present |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 3 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 3 |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 3 |
| - RB stop/continue | Not Present |
| - RB information to reconfigure | (AM DCCH for NAS_DT Low priority) |
| - RB identity | 4 |
| - PDCP info | Not Present |
| - RLC info | Not Present |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 4 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 4 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 4 |
| - RB information to reconfigure | (AM DTCH) |
| - RB identity | 20 |
| - PDCP info | Not Present |
| - CHOICE RLC info type | Not Present |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 1 |
| - Logical channel identity | 7 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 1 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 6 |
| - Logical channel identity | 7 |
| - RB stop/continue | Not Present |
| RB information to be affected | Not Present |
| UL Transport channel information for all transport channels |  |
| - TFC subset | (This IE is repeated for TFC number.) |
| - Allowed Transport Format combination | 0 to MaxTFCValue-1 (MaxTFCValue is refer to TS34.108 clause 6.10 Parameter Set.) |
| - PRACH TFCS | Not Present |
| - CHOICE mode | FDD |
| - UL DCH TFCS | (This IE is repeated for TFC number.) |
| - Normal |  |
| - TFCI Field 1 information |  |
| - CHOICE TFCS representation |  |
| - TFCS addition information |  |
| - CHOICE CTFC Size | Number of bits used must be enough to cover all combinations of CTFC from clause 6.10. |
| - CTFC information | Refer to TS34.108 clause 6.10 Parameter Set |
| - Power offset information |  |
| - CHOICE Gain Factors | Signalled Gain Factor |
| - Gain factor βc | 0 |
| - Gain factor βd | 0 |
| - Reference TFC ID | Not Present |
| - Power offset Pp-m | 0dB |
| Deleted UL TrCH information |  |
| - Transport channel identity | 15 |
| Added or Reconfigured UL TrCH information |  |
| - Transport channel identity | 5 |
| - TFS |  |
| - CHOICE Transport channel type | Dedicated transport channels |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |
| - Transmission Time Interval | Not Present |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |
| - CHOICE Logical Channel list | ALL |
| - Semi-static Transport Format information |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |
| Added or Reconfigured UL TrCH information |  |
| - Transport channel identity | 1 |
| - TFS |  |
| - CHOICE Transport channel type | Dedicated transport channels |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |
| - Transmission Time Interval | Not Present |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |
| - CHOICE Logical Channel list | ALL |
| - Semi-static Transport Format information |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |
| DRAC static information | Not Present |
| DL Transport channel information common for all transport channel |  |
| - SCCPCH TFCS | Not Present |
| - CHOICE DL parameters | Independent |
| - DL DCH TFCS | (This IE is repeated for TFC number.) |
| - Normal |  |
| - TFCI Field 1 information |  |
| - CHOICE TFCS representation | Addition |
| - TFCS addition information |  |
| - CHOICE CTFC Size | Number of bits used must be enough to cover all combinations of CTFC from clause 6.10. |
| - CTFC information | Refer to TS34.108 clause 6.10 Parameter Set |
| - Power offset information | Not Present |
| Deleted DL TrCH information |  |
| - Transport channel identity | 12 |
| - Transport channel identity | 13 |
| - Transport channel identity | 14 |
| Added or Reconfigured DL TrCH information |  |
| - Transport channel identity | 10 |
| - CHOICE DL parameters | Independent |
| - TFS |  |
| - CHOICE Transport channel type | Dedicated transport channels |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |
| - Transmission Time Interval | Not Present |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |
| - CHOICE Logical Channel list | ALL |
| - Semi-static Transport Format information |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |
| - DCH quality target |  |
| - BLER Quality value | -6.3 |
| - Transparent mode signalling info | Not Present |
| Added or Reconfigured DL TrCH information |  |
| - Transport channel identity | 6 |
| - CHOICE DL parameters | Independent |
| - TFS |  |
| - CHOICE Transport channel type | Dedicated transport channels |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |
| - Transmission Time Interval | Not Present |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |
| - CHOICE Logical Channel list | ALL |
| - Semi-static Transport Format information |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |
| - DCH quality target |  |
| - BLER Quality value | -6.3 |
| - Transparent mode signalling info | Not Present |
| Frequency info |  |
| - UARFCN uplink(Nu) | Reference to TS34.108 clause 6.10 Parameter Set |
| - UARFCN downlink(Nd) | Reference to TS34.108 clause 6.10 Parameter Set |
| Maximum allowed UL TX power | 33dBm |
| Uplink DPCH info |  |
| -Uplink DPCH power control info |  |
| -DPCCH Power offset | -6dB |
| -PC Preamble | 8slot |
| -Power Control Algorithm | Algorithm1 |
| -TPC step size | 1dB |
| - Scrambling code type | Long |
| - Scrambling code number | 0 (0 to 16777215) |
| - Number of DPDCH | Not Present(1) |
| - spreading factor | SF is reference to TS34.108 clause 6.10 Parameter Set |
| - TFCI existence | TRUE |
| - Number of FBI bits | Not Present(0) |
| - Puncturing Limit | Reference to TS34.108 clause 6.10 Parameter Set |
| CHOICE Mode | FDD |
| - Downlink PDSCH information | Not Present |
| Downlink information common for all radio links |  |
| - Downlink DPCH info common for all RL |  |
| - Timing indicator | Maintain |
| - CFN-targetSFN frame offset | Not Present |
| - Downlink DPCH power control information |  |
| - DPC mode | 0 (single) |
| - DL rate matching restriction information | Not Present |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Fixed or Flexible Position | Flexible |
| - TFCI existence | TRUE |
| - Number of bits for Pilot bits(SF=128,256) | Reference to TS34.108 clause 6.10 Parameter Set |
| - DPCH compressed mode info |  |
| - TGPSI | 1 |
| - TGPS Status Flag | Inactive |
| - TGCFN | (Current CFN + (256 – TTI/10msec)) mod 256 |
| - Transmission gap pattern sequence configuration parameters |  |
| - TGMP | FDD Measurement |
| - TGPRC | 62 |
| - TGSN | 8 |
| - TGL1 | 10 |
| - TGL2 | 5 |
| - TGD | 15 |
| - TGPL1 | 35 |
| - TGPL2 | 35 |
| - RPP | Mode 1 |
| - ITP | Mode 1 |
| - CHOICE UL/DL Mode | DL |
| - Downlink compressed mode method | SF/2 |
| - Downlink frame type | A |
| - DeltaSIR1 | 2.0 |
| - DeltaSIRafter1 | 1.0 |
| - DeltaSIR2 | Not Present |
| - DeltaSIRafter2 | Not Present |
| - TX Diversity mode | None |
| - SSDT information | Not Present |
| - Default DPCH Offset Value | 0 |
| Downlink information for each radio links |  |
| - Primary CPICH info |  |
| - Primary scrambling code | 100 |
| - PDSCH with SHO DCH info | Not Present |
| - PDSCH code mapping | Not Present |
| - Downlink DPCH info for each RL |  |
| - Primary CPICH usage for channel estimation | Primary CPICH may be used |
| - DPCH frame offset | 0 chips |
| - Secondary CPICH info | Not Present |
| - DL channelisation code |  |
| - Secondary scrambling code | 1 |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Code number | SF-1(SF is reference to TS34.108 clause 6.10 Parameter Set) |
| - Scrambling code change | No change |
| - TPC combination index | 0 |
| - SSDT Cell Identity | -a |
| - Closed loop timing adjustment mode | Not Present |
| - Secondary CCPCH info | Not Present |
| - TFCS | Not Present |
| - FACH/PCH information | Not Present |
| - References to system information blocks | Not Present |

# Contents of RADIO BEARER RELEASE message: AM or UM (Speech in CS)

| Information Element | Value/remark |
| --- | --- |
| Message Type |  |
| RRC transaction identifier | Arbitrarily selects an integer between 0 and 3 |
| Integrity check info | The presence of this IE is dependent on IXIT statements in TS 34.123-2. If integrity protection is indicated to be active, this IE is present with the values of the sub IEs as stated below. Else, this IE and the sub-IEs are omitted. |
| - message authentication code | SS calculates the value of MAC-I for this message and writes to this IE. |
| - RRC message sequence number | SS provides the value of this IE, from its internal counter. |
| Integrity protection mode info | Not Present |
| Ciphering mode info | Not Present |
| Activation time | (256+CFN-(CFN MOD 8 + 8))MOD 256 |
| New U-RNTI | Not Present |
| New C-RNTI | Not Present |
| RRC State indicator | CELL_DCH |
| UTRAN DRX cycle length coefficient | Not Present |
| CN information info | Not Present |
| Signalling Connection release indication | Not Present |
| URA identity | Not Present |
| RAB information to reconfigure list | Not Present |
| RB information to release |  |
| - RB identity | 10 |
| RB information to release |  |
| - RB identity | 11 |
| RB information to release |  |
| - RB identity | 12 |
| RB information to be affected | (UM DCCH for RRC) |
| - RB identity | 1 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 1 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 1 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 1 |
| RB information to be affected | (AM DCCH for RRC) |
| - RB identity | 2 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 2 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 2 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 2 |
| RB information to be affected | (AM DCCH for NAS_DT High priority) |
| - RB identity | 3 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 3 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 3 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 3 |
| RB information to be affected | (AM DCCH for NAS_DT Low priority) |
| - RB identity | 4 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 4 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 4 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 4 |
| RB with PDCP information list | Not Present |
| UL Transport channel information for all transport channels |  |
| - TFC subset | (This IE is repeated for TFC number.) |
| - Allowed Transport Format combination | 0 to MaxTFCValue-1 (MaxTFCValue is refer to TS34.108 clause 6.10 Parameter Set.) |
| - PRACH TFCS | Not Present |
| - CHOICE mode | FDD |
| - UL DCH TFCS | (This IE is repeated for TFC number.) |
| - Normal |  |
| - TFCI Field 1 information |  |
| - CHOICE CTFC representation | Addition |
| - TFCS addition information |  |
| - CHOICE CTFC Size | Number of bits used must be enough to cover all combinations of CTFC from clause 6.10. |
| - CTFC information | Refer to TS34.108 clause 6.10 Parameter Set |
| - Power offset information |  |
| - CHOICE Gain Factor | Signalled Gain Factor |
| - Gain factor βc | 0 |
| - Gain factor βd | 0 |
| - Reference TFC ID | Not Present |
| - Power offset Pp-m | 0dB |
| Deleted UL TrCH Information |  |
| - Transport channel identity | 1 |
| Deleted UL TrCH Information |  |
| - Transport channel identity | 2 |
| Deleted UL TrCH Information |  |
| - Transport channel identity | 3 |
| Added or Reconfigured UL TrCH information | If TrCH reconfiguration is executed then this is needed(e.g The rate of SRB for DCCH is changed.). |
| - Transport channel identity | 5 |
| - TFS |  |
| - CHOICE Transport channel type | Dedicated transport channels |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |
| - Transmission Time Interval | Not Present |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |
| - CHOICE Logical Channel list | ALL |
| - Semi-static Transport Format information |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |
| CPCH set ID | Not Present |
| DRAC static information | Not Preaent |
| DL Transport channel information common for all transport channel |  |
| - SCCPCH TFCS | Not Present |
| - CHOICE DL parameters | Independent |
| - DL DCH TFCS | (This IE is repeated for TFC number.) |
| - Normal |  |
| - TFCI Field 1 information |  |
| - CHOICE CTFC representation | Addition |
| - TFCS addition information |  |
| - CHOICE CTFC Size | Number of bits used must be enough to cover all combinations of CTFC from clause 6.10. |
| - CTFC information | Refer to TS34.108 clause 6.10 Parameter Set |
| - Power offset information | Not Present |
| Deleted DL TrCH Information |  |
| - Transport channel identity | 6 |
| Deleted DL TrCH Information |  |
| - Transport channel identity | 7 |
| Deleted DL TrCH Information |  |
| - Transport channel identity | 8 |
| Added or Reconfigured DL TrCH information | If TrCH reconfiguration is executed then this is needed(e.g The rate of SRB for DCCH is changed.). |
| - Transport channel identity | 10 |
| - CHOICE DL parameters | Independent |
| - UL TrCH Identity | 5 |
| - TFS |  |
| - CHOICE Transport channel type | Dedicated transport channels |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |
| - Transmission Time Interval | Not Present |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |
| - CHOICE Logical Channel list | ALL |
| - Semi-static Transport Format information |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |
| - DCH quality target |  |
| - BLER Quality value | -6.3 |
| - Transparent mode signalling info | Not Present |
| Frequency info |  |
| - UARFCN uplink(Nu) | Reference to TS34.108 clause 6.10 Parameter Set |
| - UARFCN downlink(Nd) | Reference to TS34.108 clause 6.10 Parameter Set |
| Maximum allowed UL TX power | 33dBm |
| Uplink DPCH info |  |
| - Uplink DPCH power control info |  |
| - DPCCH power offset | -6dB |
| - PC Preamble | 8slot |
| - Power Control Algorithm | Algorithm1 |
| - TPC step size | 1dB |
| - Scrambling code type | Long |
| - Scrambling code number | 0 (0 to 16777215) |
| - Number of DPDCH | Not Present(1) |
| - spreading factor | SF is reference to TS34.108 clause 6.10 Parameter Set |
| - TFCI existence | TRUE |
| - Number of FBI bit | Not Present(0) |
| - Puncturing Limit | Reference to TS34.108 clause 6.10 Parameter Set |
| CHOICE Mode | FDD |
| - Downlink PDSCH information | Not Present |
| Downlink information common for all radio links |  |
| - Downlink DPCH info common for all RL |  |
| - Timing indicator | Maintain |
| - CFN-targetSFN frame offset | Not Present |
| - Downlink DPCH power control information |  |
| - DPC mode | 0 (single) |
| - DL rate matching restriction information | Not Present |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Fixed or Flexible Position | N/A |
| - TFCI existence | FALSE |
| - Number of bits for Pilot bits(SF=128,256) | Reference to TS34.108 clause 6.10 Parameter Set |
| - DPCH compressed mode info |  |
| - TGPSI | 1 |
| - TGPS Status Flag | inactive |
| - TGCFN | (Current CFN + (256 – TTI/10msec)) mod 256 |
| - Transmission gap pattern sequence configuration parameters |  |
| - TGMP | FDD Measurement |
| - TGPRC | 62 |
| - TGSN | 8 |
| - TGL1 | 10 |
| - TGL2 | 5 |
| - TGD | 15 |
| - TGPL1 | 35 |
| - TGPL2 | 35 |
| - RPP | Mode 1 |
| - ITP | Mode 1 |
| - CHOICE UL/DL Mode | DL |
| - Downlink compressed mode method | SF/2 |
| - Downlink frame type | A |
| - DeltaSIR1 | 2.0 |
| - DeltaSIRafter1 | 1.0 |
| - DeltaSIR2 | Not Present |
| - DeltaSIRafter2 | Not Present |
| - TX Diversity mode | None |
| - SSDT information | Not Present |
| - Default DPCH Offset Value | 0 |
| Downlink information for each radio links |  |
| - Primary CPICH info |  |
| - Primary scrambling code | 100 |
| - PDSCH with SHO DCH info | Not Present |
| - PDSCH code mapping | Not Present |
| - Downlink DPCH info for each RL |  |
| - Primary CPICH usage for channel estimation | Primary CPICH may be used |
| - DPCH frame offset | 0 chips |
| - Secondary CPICH info | Not Present |
| - DL channelisation code |  |
| - Secondary scrambling code | 1 |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Code number | SF-1(SF is reference to TS34.108 clause 6.10 Parameter Set) |
| - Scrambling code change | No change |
| - TPC combination index | 0 |
| - SSDT Cell Identity | -a |
| - Closed loop timing adjustment mode | Not Present |
| - Secondary CCPCH info | Not Present |
| - TFCS | Not Present |
| - FACH/PCH information | Not Present |
| - References to system information blocks | Not Present |

# Contents of RADIO BEARER RELEASE message: AM or UM (Packet to CELL_DCH from CELL_DCH in PS)

| Information Element | Value/remark |
| --- | --- |
| Message Type |  |
| RRC transaction identifier | Arbitrarily selects an integer between 0 and 3 |
| Integrity check info | The presence of this IE is dependent on IXIT statements in TS 34.123-2. If integrity protection is indicated to be active, this IE is present with the values of the sub IEs as stated below. Else, this IE and the sub-IEs are omitted. |
| - message authentication code | SS calculates the value of MAC-I for this message and writes to this IE. |
| - RRC message sequence number | SS provides the value of this IE, from its internal counter. |
| Integrity protection mode info | Not Present |
| Ciphering mode info | Not Present |
| Activation time | (256+CFN-(CFN MOD 8 + 8))MOD 256 |
| New U-RNTI | Not Present |
| New C-RNTI | Not Present |
| RRC State indicator | CELL_DCH |
| UTRAN DRX cycle length coefficient | Not Present |
| CN information info | Not Present |
| Signalling Connection release indication | Not Present |
| URA identity | Not Present |
| RAB information to reconfigure list | Not Present |
| RB information to release |  |
| - RB identity | 20 |
| RB information to be affected | (UM DCCH for RRC) |
| - RB identity | 1 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 1 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 1 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 1 |
| RB information to be affected | (AM DCCH for RRC) |
| - RB identity | 2 |
| - RB mapping info |  |
| - RLC logical channel mapping indicator | Not Present |
| - Information for each multiplexing option |  |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 2 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 2 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 2 |
| RB information to be affected | (AM DCCH for NAS_DT High priority) |
| - RB identity | 3 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 3 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 3 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 3 |
| RB information to be affected | (AM DCCH for NAS_DT Low priority) |
| - RB identity | 4 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 4 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 4 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 4 |
| RB with PDCP information list | Not Present |
| UL Transport channel information for all transport channels |  |
| - TFC subset | (This IE is repeated for TFC number.) |
| - Allowed Transport Format combination | 0 to MaxTFCValue-1 (MaxTFCValue is refer to TS34.108 clause 6.10 Parameter Set.) |
| - PRACH TFCS | Not Present |
| - CHOICE mode | FDD |
| - UL DCH TFCS | (This IE is repeated for TFC number.) |
| - Normal |  |
| - TFCI Field 1 information |  |
| - CHOICE CTFC representation | Addition |
| - TFCS addition information |  |
| - CHOICE CTFC Size | Number of bits used must be enough to cover all combinations of CTFC from clause 6.10. |
| - CTFC information | Refer to TS34.108 clause 6.10 Parameter Set |
| - Power offset information | Not Present |
| Deleted UL TrCH Information |  |
| - Transport channel identity | 1 |
| Added or Reconfigured UL TrCH information | If TrCH reconfiguration is executed then this is needed(e.g The rate of SRB for DCCH is changed.). |
| - Transport channel identity | 5 |
| - CHOICE DL parameters | Independent |
| - TFS |  |
| - CHOICE Transport channel type | Dedicated transport channels |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |
| - Transmission Time Interval | Not Present |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |
| - CHOICE Logical Channel list | ALL |
| - Semi-static Transport Format information |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |
| CPCH set ID | Not Present |
| DRAC static information | Not Preaent |
| DL Transport channel information common for all transport channel |  |
| - SCCPCH TFCS | Not Present |
| - CHOICE DL parameters |  |
| - DL DCH TFCS | (This IE is repeated for TFC number.) |
| - Normal |  |
| - TFCI Field 1 information |  |
| - CHOICE CTFC representation | Addition |
| - TFCS addition information |  |
| - CHOICE CTFC Size | Number of bits used must be enough to cover all combinations of CTFC from clause 6.10. |
| - CTFC information | Refer to TS34.108 clause 6.10 Parameter Set |
| - Power offset information | Not Present |
| Deleted DL TrCH Information |  |
| - Transport channel identity | 6 |
| Added or Reconfigured DL TrCH information | If TrCH reconfiguration is executed then this is needed(e.g The rate of SRB for DCCH is changed.). |
| - Transport channel identity | 10 |
| - CHOICE DL parameters | Independent |
| - TFS |  |
| - CHOICE Transport channel type | Dedicated transport channels |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |
| - Transmission Time Interval | Not Present |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |
| - CHOICE Logical Channel list | ALL |
| - Semi-static Transport Format information |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |
| - DCH quality target |  |
| - BLER Quality value | -6.3 |
| - Transparent mode signalling info | Not Present |
| Frequency info |  |
| - UARFCN uplink(Nu) | Reference to TS34.108 clause 6.10 Parameter Set |
| - UARFCN downlink(Nd) | Reference to TS34.108 clause 6.10 Parameter Set |
| Maximum allowed UL TX power | 33dBm |
| Uplink DPCH info |  |
| - Uplink DPCH power control info |  |
| - DPCCH power offset | -6dB |
| - PC Preamble | 8slot |
| - Power Control Algorithm | Algorithm1 |
| - TPC step size | 1dB |
| - Scrambling code type | Long |
| - Scrambling code number | 0 (0 to 16777215) |
| - Number of DPDCH | Not Present(1) |
| - spreading factor | SF is reference to TS34.108 clause 6.10 Parameter Set |
| - TFCI existence | TRUE |
| - Number of FBI bit | Not Present(0) |
| - Puncturing Limit | Reference to TS34.108 clause 6.10 Parameter Set |
| CHOICE Mode | FDD |
| - Downlink PDSCH information | Not Present |
| Downlink information common for all radio links |  |
| - Downlink DPCH info common for all RL |  |
| - Timing indicator | Maintain |
| - CFN-targetSFN frame offset | Not Present |
| - Downlink DPCH power control information |  |
| - DPC mode | 0 (single) |
| - DL rate matching restriction information | Not Present |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Fixed or Flexible Position | N/A |
| - TFCI existence | FALSE |
| - Number of bits for Pilot bits(SF=128,256) | Reference to TS34.108 clause 6.10 Parameter Set |
| - DPCH compressed mode info |  |
| - TGPSI | 1 |
| - TGPS Status Flag | inactive |
| - TGCFN | (Current CFN + (256 – TTI/10msec)) mod 256 |
| - Transmission gap pattern sequence configuration parameters |  |
| - TGMP | FDD Measurement |
| - TGPRC | 62 |
| - TGSN | 8 |
| - TGL1 | 10 |
| - TGL2 | 5 |
| - TGD | 15 |
| - TGPL1 | 35 |
| - TGPL2 | 35 |
| - RPP | Mode 1 |
| - ITP | Mode 1 |
| - CHOICE UL/DL Mode | DL |
| - Downlink compressed mode method | SF/2 |
| - Downlink frame type | A |
| - DeltaSIR1 | 2.0 |
| - DeltaSIRafter1 | 1.0 |
| - DeltaSIR2 | Not Present |
| - DeltaSIRafter2 | Not Present |
| - TX Diversity mode | None |
| - SSDT information | Not Present |
| - Default DPCH Offset Value | 0 |
| Downlink information for each radio links |  |
| - Primary CPICH info |  |
| - Primary scrambling code | 100 |
| - PDSCH with SHO DCH info | Not Present |
| - PDSCH code mapping | Not Present |
| - Downlink DPCH info for each RL |  |
| - Primary CPICH usage for channel estimation | Primary CPICH may be used |
| - DPCH frame offset | 0 chips |
| - Secondary CPICH info | Not Present |
| - DL channelisation code |  |
| - Secondary scrambling code | 1 |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Code number | SF-1(SF is reference to TS34.108 clause 6.10 Parameter Set) |
| - Scrambling code change | No change |
| - TPC combination index | 0 |
| - SSDT Cell Identity | -a |
| - Closed loop timing adjustment mode | Not Present |
| - Secondary CCPCH info | Not Present |
| - TFCS | Not Present |
| - FACH/PCH information | Not Present |
| - References to system information blocks | Not Present |

# Contents of RADIO BEARER RELEASE message: AM or UM (Packet to CELL_DCH from CELL_FACH in PS)

| Information Element | Value/remark |
| --- | --- |
| Message Type |  |
| RRC transaction identifier | Arbitrarily selects an integer between 0 and 3 |
| Integrity check info | The presence of this IE is dependent on IXIT statements in TS 34.123-2. If integrity protection is indicated to be active, this IE is present with the values of the sub IEs as stated below. Else, this IE and the sub-IEs are omitted. |
| - message authentication code | SS calculates the value of MAC-I for this message and writes to this IE. |
| - RRC message sequence number | SS provides the value of this IE, from its internal counter. |
| Integrity protection mode info | Not Present |
| Ciphering mode info | Not Present |
| Activation time | (256+CFN-(CFN MOD 8 + 8))MOD 256 |
| New U-RNTI | Not Present |
| New C-RNTI | Not Present |
| RRC State indicator | CELL_DCH |
| UTRAN DRX cycle length coefficient | Not Present |
| CN information info | Not Present |
| Signalling Connection release indication | Not Present |
| URA identity | Not Present |
| RAB information to reconfigure list | Not Present |
| RB information to release |  |
| - RB identity | 20 |
| RB information to release |  |
| - RB identity | 6 |
| RB information to release |  |
| - RB identity | 7 |
| RB information to release |  |
| - RB identity | 0 |
| RB information to be affected | (UM DCCH for RRC) |
| - RB identity | 1 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 1 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 1 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 1 |
| RB information to be affected | (AM DCCH for RRC) |
| - RB identity | 2 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 2 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 2 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 2 |
| RB information to be affected | (AM DCCH for NAS_DT High priority) |
| - RB identity | 3 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 3 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 3 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 100 |
| - Logical channel identity | 3 |
| RB information to be affected | (AM DCCH for NAS_DT Low priority) |
| - RB identity | 4 |
| - RB mapping info |  |
| - Information for each multiplexing option |  |
| - RLC logical channel mapping indicator | Not Present |
| - Number of uplink RLC logical channels | 1 |
| - Uplink transport channel type | DCH |
| - UL Transport channel identity | 5 |
| - Logical channel identity | 4 |
| - CHOICE RLC size list | All |
| - MAC logical channel priority | 4 |
| - Downlink RLC logical channel info |  |
| - Number of downlink RLC logical channels | 1 |
| - Downlink transport channel type | DCH |
| - DL Transport channel identity | 10 |
| - Logical channel identity | 4 |
| RB with PDCP information list | Not Present |
| UL Transport channel information for all transport channels |  |
| - TFC subset | (This IE is repeated for TFC number.) |
| - Allowed Transport Format combination | 0 to MaxTFCValue-1 (MaxTFCValue is refer to TS34.108 clause 6.10 Parameter Set.) |
| - PRACH TFCS | Not Present |
| - CHOICE mode | FDD |
| - UL DCH TFCS | (This IE is repeated for TFC number.) |
| - Normal |  |
| - TFCI Field 1 information |  |
| - CHOICE CTFC representation | Addition |
| - TFCS addition information |  |
| - CHOICE CTFC Siz | Number of bits used must be enough to cover all combinations of CTFC from clause 6.10. |
| - CTFC | Refer to TS34.108 clause 6.10 Parameter Set |
| - Power offset information |  |
| - CHOICE Gain Factors | Signalled Gain Factor |
| - Gain factor βc | 0 |
| - Gain factor βd | 0 |
| - Reference TFC ID | Not Present |
| - Power offset Pp-m | 0dB |
| Deleted UL TrCH Information | 15 |
| Added or Reconfigured UL TrCH information |  |
| - Transport channel identity | 5 |
| - TFS |  |
| - CHOICE Transport channel type | Dedicated transport channels |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |
| - Transmission Time Interval | Not Present |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |
| - CHOICE Logical Channel list | ALL |
| - Semi-static Transport Format information |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |
| CPCH set ID | Not Present |
| DRAC static information | Not Preaent |
| DL Transport channel information common for all transport channel |  |
| - SCCPCH TFCS | Not Present |
| - CHOICE DL parameters | Independent |
| - DL DCH TFCS | (This IE is repeated for TFC number.) |
| - Normal |  |
| - TFCI Field 1 information |  |
| - CHOICE CTFC representation | Addition |
| - TFCS addition information |  |
| - CHOICE CTFC Size | Number of bits used must be enough to cover all combinations of CTFC from clause 6.10. |
| - CTFC information | Refer to TS34.108 clause 6.10 Parameter Set |
| - Power offset information | Not Present |
| Deleted DL TrCH Information |  |
| - Transport channel identity | 12 |
| Deleted DL TrCH Information |  |
| - Transport channel identity | 13 |
| Deleted DL TrCH Information |  |
| - Transport channel identity | 14 |
| Added or Reconfigured DL TrCH information |  |
| - Transport channel identity | 10 |
| - CHOICE DL parameters | Independent |
| - TFS |  |
| - CHOICE Transport channel type | Dedicated transport channels |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |
| - Transmission Time Interval | Not Present |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |
| - CHOICE Logical Channel list | ALL |
| - Semi-static Transport Format information |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |
| - DCH quality target |  |
| - BLER Quality value | -6.3 |
| - Transparent mode signalling info | Not Present |
| Frequency info |  |
| - UARFCN uplink(Nu) | Reference to TS34.108 clause 6.10 Parameter Set |
| - UARFCN downlink(Nd) | Reference to TS34.108 clause 6.10 Parameter Set |
| Maximum allowed UL TX power | 33dBm |
| Uplink DPCH info |  |
| - Uplink DPCH power control info |  |
| - DPCCH power offset | -6dB |
| - PC Preamble | 8slot |
| - Power Control Algorithm | Algorithm1 |
| - TPC step size | 1dB |
| - Scrambling code type | Long |
| - Scrambling code number | 0 (0 to 16777215) |
| - Number of DPDCH | Not Present(1) |
| - spreading factor | SF is reference to TS34.108 clause 6.10 Parameter Set |
| - TFCI existence | TRUE |
| - Number of FBI bit | Not Present(0) |
| - Puncturing Limit | Reference to TS34.108 clause 6.10 Parameter Set |
| CHOICE Mode | FDD |
| - Downlink PDSCH information | Not Present |
| Downlink information common for all radio links |  |
| - Downlink DPCH info common for all RL |  |
| - Timing indicator | Maintain |
| - CFN-targetSFN frame offset | Not Present |
| - Downlink DPCH power control information |  |
| - DPC mode | 0 (single) |
| - DL rate matching restriction information | Not Present |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Fixed or Flexible Position | N/A |
| - TFCI existence | FALSE |
| - Number of bits for Pilot bits(SF=128,256) | Reference to TS34.108 clause 6.10 Parameter Set |
| - DPCH compressed mode info |  |
| - TGPSI | 1 |
| - TGPS Status Flag | inactive |
| - TGCFN | (Current CFN + (256 – TTI/10msec)) mod 256 |
| - Transmission gap pattern sequence configuration parameters |  |
| - TGMP | FDD Measurement |
| - TGPRC | 62 |
| - TGSN | 8 |
| - TGL1 | 10 |
| - TGL2 | 5 |
| - TGD | 15 |
| - TGPL1 | 35 |
| - TGPL2 | 35 |
| - RPP | Mode 1 |
| - ITP | Mode 1 |
| - CHOICE UL/DL Mode | DL |
| - Downlink compressed mode method | SF/2 |
| - Downlink frame type | A |
| - DeltaSIR1 | 2.0 |
| - DeltaSIRafter1 | 1.0 |
| - DeltaSIR2 | Not Present |
| - DeltaSIRafter2 | Not Present |
| - TX Diversity mode | None |
| - SSDT information | Not Present |
| - Default DPCH Offset Value | 0 |
| Downlink information for each radio links |  |
| - Primary CPICH info |  |
| - Primary scrambling code | 100 |
| - PDSCH with SHO DCH info | Not Present |
| - PDSCH code mapping | Not Present |
| - Downlink DPCH info for each RL |  |
| - Primary CPICH usage for channel estimation | Primary CPICH may be used |
| - DPCH frame offset | 0 chips |
| - Secondary CPICH info | Not Present |
| - DL channelisation code |  |
| - Secondary scrambling code | 1 |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Code number | SF-1(SF is reference to TS34.108 clause 6.10 Parameter Set) |
| - Scrambling code change | No change |
| - TPC combination index | 0 |
| - SSDT Cell Identity | -a |
| - Closed loop timing adjustment mode | Not Present |
| - Secondary CCPCH info | Not Present |
| - TFCS | Not Present |
| - FACH/PCH information | Not Present |
| - References to system information blocks | Not Present |

# Contents of TRANSPORT CHANNEL RECONFIGURATION message: AM or UM (The others of speech in CS)

| Information Element | Value/remark |
| --- | --- |
| Message Type |  |
| RRC transaction identifier | Arbitrarily selects an integer between 0 and 3 |
| Integrity check info | The presence of this IE is dependent on IXIT statements in TS 34.123-2. If integrity protection is indicated to be active, this IE is present with the values of the sub IEs as stated below. Else, this IE and the sub-IEs are omitted. |
| - message authentication code | SS calculates the value of MAC-I for this message and writes to this IE. |
| - RRC message sequence number | SS provides the value of this IE, from its internal counter. |
| Integrity protection mode info | Not Present |
| Ciphering mode info | Not Present |
| Activation time | (256+CFN-(CFN MOD 8 + 8))MOD 256 |
| New U-RNTI | Not Present |
| New C-RNTI | Not Present |
| RRC State indicator | CELL_DCH |
| UTRAN DRX cycle length coefficient | Not Present |
| CN information info | Not Present |
| URA identity | Not Present |
| RB with PDCP info | Not Present |
| UL Transport channel information for all transport channels |  |
| - TFC subset | (This IE is repeated for TFC number.) |
| - Allowed Transport Format combination | 0 to MaxTFCValue-1 (MaxTFCValue is refer to TS34.108 clause 6.10 Parameter Set.) |
| - PRACH TFCS | Not Present |
| - CHOICE mode | FDD |
| - UL DCH TFCS | (This IE is repeated for TFC number.) |
| - Normal |  |
| - TFCI Field 1 information |  |
| - CHOICE CTFC representation | Addition |
| - TFCS addition information |  |
| - CHOICE CTFC Size | Number of bits used must be enough to cover all combinations of CTFC from clause 6.10. |
| - CTFC information | Refer to TS34.108 clause 6.10 Parameter Set |
| - Power offset information |  |
| - CHOICE Gain Factors | Signalled Gain Factors |
| - Gain factor βc | 0 |
| - Gain factor βd | 0 |
| - Reference TFC ID | Not Present |
| - Power offset Pp-m | 0dB |
| Added or Reconfigured UL TrCH information |  |
| - Transport channel identity | 5 |
| - TFS |  |
| - CHOICE Transport channel type | Dedicated transport channels |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |
| - Transmission Time Interval | Not Present |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |
| - Semi-static Transport Format information |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |
| DRAC static information | Not Present |
| DL Transport channel information common for all transport channel |  |
| - SCCPCH TFCS | Not Present |
| - CHOICE DL parameters | Independent |
| - DL DCH TFCS | (This IE is repeated for TFC number.) |
| - Normal |  |
| - TFCI Field 1 information |  |
| - CHOICE CTFC representation | Addition |
| - TFCS addition information |  |
| - CHOICE CTFC Size | Number of bits used must be enough to cover all combinations of CTFC from clause 6.10. |
| - CTFC information | Refer to TS34.108 clause 6.10 Parameter Set |
| - Power offset information | Not Present |
| Added or Reconfigured DL TrCH information |  |
| - Transport channel identity | 10 |
| - CHOICE DL parameters | SameAsUL |
| - UL TrCH Identity | 5 |
| - DCH quality target |  |
| - BLER Quality value | -6.3 |
| - Transparent mode signalling info | Not Present |
| Frequency info |  |
| - UARFCN uplink(Nu) | Reference to TS34.108 clause 6.10 Parameter Set |
| - UARFCN downlink(Nd) | Reference to TS34.108 clause 6.10 Parameter Set |
| Maximum allowed UL TX power | 33dBm |
| Uplink DPCH info |  |
| -Uplink DPCH power control info |  |
| - DPCCH power offset | -6dB |
| - PC Preamble | 8slot |
| - Power Control Algorithm | Algorithm1 |
| - TPC step size | 1dB |
| - Scrambling code type | Long |
| - Scrambling code number | 0 (0 to 16777215) |
| - Number of DPDCH | Not Present(1) |
| - spreading factor | SF is reference to TS34.108 clause 6.10 Parameter Set |
| - TFCI existence | TRUE |
| - Number of FBI bit | Not Present(0) |
| - Puncturing Limit | Reference to TS34.108 clause 6.10 Parameter Set |
| CHOICE Mode | FDD |
| - Downlink PDSCH information | Not Present |
| Downlink information common for all radio links |  |
| - Downlink DPCH info common for all RL |  |
| - Timing indicator | Maintain |
| - CFN-targetSFN frame offset | Not Present |
| - Downlink DPCH power control information |  |
| - DPC mode | 0 (single) |
| - DL rate matching restriction information | Not Present |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Fixed or Flexible Position | Flexible |
| - TFCI existence | TRUE |
| - Number of bits for Pilot bits(SF=128,256) | Not Present |
| - DPCH compressed mode info |  |
| - TGPSI | 1 |
| - TGPS Status Flag | inactive |
| - TGCFN | (Current CFN + (256 – TTI/10msec)) mod 256 |
| - Transmission gap pattern sequence configuration parameters |  |
| - TGMP | FDD Measurement |
| - TGPRC | 62 |
| - TGSN | 8 |
| - TGL1 | 10 |
| - TGL2 | 5 |
| - TGD | 15 |
| - TGPL1 | 35 |
| - TGPL2 | 35 |
| - RPP | Mode 1 |
| - ITP | Mode 1 |
| - CHOICE UL/DL Mode | DL |
| - Downlink compressed mode method | SF/2 |
| - Downlink frame type | A |
| - DeltaSIR1 | 2.0 |
| - DeltaSIRafter1 | 1.0 |
| - DeltaSIR2 | Not Present |
| - DeltaSIRafter2 | Not Present |
| - TX Diversity mode | None |
| - SSDT information | Not Present |
| - Default DPCH Offset Value | 0 |
| Downlink information for each radio links |  |
| - Primary CPICH info |  |
| - Primary scrambling code | 100 |
| - PDSCH with SHO DCH info | Not Present |
| - PDSCH code mapping | Not Present |
| - Downlink DPCH info for each RL |  |
| - Primary CPICH usage for channel estimation | Primary CPICH may be used |
| - DPCH frame offset | 0 chips |
| - Secondary CPICH info | Not Present |
| - DL channelisation code |  |
| - Secondary scrambling code | 1 |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Code number | SF-1(SF is reference to TS34.108 clause 6.10 Parameter Set) |
| - Scrambling code change | No change |
| - TPC combination index | 0 |
| - SSDT Cell Identity | -a |
| - Closed loop timing adjustment mode | Not Present |
| - Secondary CCPCH info | Not Present |
| - TFCS | Not Present |
| - FACH/PCH information | Not Present |
| - References to system information blocks | Not Present |

# Contents of TRANSPORT CHANNEL RECONFIGURATION message: AM or UM (Speech in CS)

| Information Element | Value/remark |  |
| --- | --- | --- |
| Message Type |  |  |
| RRC transaction identifier | Arbitrarily selects an integer between 0 and 3 |  |
| Integrity check info | The presence of this IE is dependent on IXIT statements in TS 34.123-2. If integrity protection is indicated to be active, this IE is present with the values of the sub IEs as stated below. Else, this IE and the sub-IEs are omitted. |  |
| - message authentication code | SS calculates the value of MAC-I for this message and writes to this IE. |  |
| - RRC message sequence number | SS provides the value of this IE, from its internal counter. |  |
| Integrity protection mode info | Not Present |  |
| Ciphering mode info | Not Present |  |
| Activation time | (256+CFN-(CFN MOD 8 + 8))MOD 256 |  |
| New U-RNTI | Not Present |  |
| New C-RNTI | Not Present |  |
| RRC State indicator | CELL_DCH |  |
| UTRAN DRX cycle length coefficient | Not Present |  |
| CN information info | Not Present |  |
| URA identity | Not Present |  |
| RB with PDCP information | Not Present |  |
| UL Transport channel information for all transport channels |  |  |
| - TFC subset | (This IE is repeated for TFC number.) |  |
| - Allowed Transport Format combination | 0 to MaxTFCValue-1 (MaxTFCValue is refer to TS34.108 clause 6.10 Parameter Set.) |  |
| - PRACH TFCS | Not Present |  |
| - CHOICE mode | FDD |  |
| - UL DCH TFCS | (This IE is repeated for TFC number.) |  |
| - Normal |  |  |
| - TFCI Field 1 information |  |  |
| - CHOICE CTFC representation | Addition |  |
| - TFCS addition information |  |  |
| - CHOICE CTFC Size | Number of bits used must be enough to cover all combinations of CTFC from clause 6.10. |  |
| - CTFC information | Refer to TS34.108 clause 6.10 Parameter Set |  |
| - Power offset information |  |  |
| - CHOICE Gain Factors | Signalled Gain Factors |  |
| - Gain factor βc | 0 |  |
| - Gain factor βd | 0 |  |
| - Reference TFC ID | Not Present |  |
| - Power offset Pp-m | 0dB |  |
| Added or Reconfigured UL TrCH information |  |  |
| - Transport channel identity | 5 |  |
| - TFS |  |  |
| - CHOICE Transport channel type | Dedicated transport channels |  |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |  |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |  |
| - Transmission Time Interval | Not Present |  |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - CHOICE Logical Channel list | ALL |  |
| - Semi-static Transport Format information |  |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |  |
| DRAC static information | Not Present |  |
| DL Transport channel information common for all transport channel |  |  |
| - SCCPCH TFCS | Not Present |  |
| - CHOICE DL parameters | Independent |  |
| - DL DCH TFCS | (This IE is repeated for TFC number.) |  |
| - Normal |  |  |
| - TFCI Field 1 information |  |  |
| - CHOICE CTFC representation | Addition |  |
| - TFCS addition information |  |  |
| - CHOICE CTFC Size | Number of bits used must be enough to cover all combinations of CTFC from clause 6.10. |  |
| - CTFC information | Refer to TS34.108 clause 6.10 Parameter Set |  |
| - Power offset information | Not Present |  |
| Added or Reconfigured DL TrCH information |  |  |
| - Transport channel identity | 10 |  |
| - CHOICE DL parameters | SameAsUL |  |
| - UL TrCH Identity | 5 |  |
| - DCH quality target |  |  |
| - BLER Quality value | -6.3 |  |
| - Transparent mode signalling info | Not Present |  |
| Frequency info |  |  |
| - UARFCN uplink(Nu) | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - UARFCN downlink(Nd) | Reference to TS34.108 clause 6.10 Parameter Set |  |
| Maximum allowed UL TX power | 33dBm |  |
| Uplink DPCH info |  |  |
| -Uplink DPCH power control info |  |  |
| - DPCCH power offset | -6dB |  |
| - PC Preamble | 8slot |  |
| - Power Control Algorithm | Algorithm1 |  |
| - TPC step size | 1dB |  |
| - Scrambling code type | Long |  |
| - Scrambling code number | 0 (0 to 16777215) |  |
| - Number of DPDCH | Not Present(1) |  |
| - spreading factor | SF is reference to TS34.108 clause 6.10 Parameter Set |  |
| - TFCI existence | TRUE |  |
| - Number of FBI bit | Not Present(0) |  |
| - Puncturing Limit | Reference to TS34.108 clause 6.10 Parameter Set |  |
| CHOICE Mode | FDD |  |
| - Downlink PDSCH information | Not Present |  |
| Downlink information common for all radio links |  |  |
| - Downlink DPCH info common for all RL |  |  |
| - Timing indicator | Maintain |  |
| - CFN-targetSFN frame offset | Not Present |  |
| - Downlink DPCH power control information |  |  |
| - DPC mode | 0 (single) |  |
| - DL rate matching restriction information | Not Present |  |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - Fixed or Flexible Position | Flexible |  |
| - TFCI existence | TRUE |  |
| - Number of bits for Pilot bits(SF=128,256) | Not Present |  |
| - DPCH compressed mode info |  |  |
| - TGPSI | 1 |  |
| - TGPS Status Flag | inactive |  |
| - TGCFN | (Current CFN + (256 – TTI/10msec)) mod 256 |  |
| - Transmission gap pattern sequence configuration parameters |  |  |
| - TGMP | FDD Measurement |  |
| - TGPRC | 62 |  |
| - TGSN | 8 |  |
| - TGL1 | 10 |  |
| - TGL2 | 5 |  |
| - TGD | 15 |  |
| - TGPL1 | 35 |  |
| - TGPL2 | 35 |  |
| - RPP | Mode 1 |  |
| - ITP | Mode 1 |  |
| - CHOICE UL/DL Mode | DL |  |
| - Downlink compressed mode method | SF/2 |  |
| - Downlink frame type | A |  |
| - DeltaSIR1 | 2.0 |  |
| - DeltaSIRafter1 | 1.0 |  |
| - DeltaSIR2 | Not Present |  |
| - DeltaSIRafter2 | Not Present |  |
| - TX Diversity mode | None |  |
| - SSDT information | Not Present |  |
| - Default DPCH Offset Value | 0 |  |
| Downlink information for each radio links |  |  |
| - Primary CPICH info |  |  |
| - Primary scrambling code | 100 |  |
| - PDSCH with SHO DCH info | Not Present |  |
| - PDSCH code mapping | Not Present |  |
| - Downlink DPCH info for each RL |  |  |
| - Primary CPICH usage for channel estimation | Primary CPICH may be used |  |
| - DPCH frame offset | 0 chips |  |
| - Secondary CPICH info | Not Present |  |
| - DL channelisation code |  |  |
| - Secondary scrambling code | 1 |  |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |  |
| - Code number | SF-1(SF is reference to TS34.108 clause 6.10 Parameter Set) |  |
| - Scrambling code change | No change |  |
| - TPC combination index | 0 |  |
| - SSDT Cell Identity | -a |  |
| - Closed loop timing adjustment mode | Not Present |  |
| - Secondary CCPCH info | Not Present |  |
| - TFCS | Not Present |  |
| - FACH/PCH information | Not Present |  |
| - References to system information blocks | Not Present |  |

# Contents of TRANSPORT CHANNEL RECONFIGURATION message: AM or UM (Packet to CELL_DCH from CELL_DCH in PS)

| Information Element | Value/remark |
| --- | --- |
| Message Type |  |
| RRC transaction identifier | Arbitrarily selects an integer between 0 and 3 |
| Integrity check info | The presence of this IE is dependent on IXIT statements in TS 34.123-2. If integrity protection is indicated to be active, this IE is present with the values of the sub IEs as stated below. Else, this IE and the sub-IEs are omitted. |
| - message authentication code | SS calculates the value of MAC-I for this message and writes to this IE. |
| - RRC message sequence number | SS provides the value of this IE, from its internal counter. |
| Integrity protection mode info | Not Present |
| Ciphering mode info | Not Present |
| Activation time | (256+CFN-(CFN MOD 8 + 8))MOD 256 |
| New U-RNTI | Not Present |
| New C-RNTI | Not Present |
| RRC State indicator | CELL_DCH |
| UTRAN DRX cycle length coefficient | Not Present |
| CN information info | Not Present |
| URA identity | Not Present |
| RB with PDCP information | Not Present |
| UL Transport channel information for all transport channels |  |
| - TFC subset | (This IE is repeated for TFC number.) |
| - Allowed Transport Format combination | 0 to MaxTFCValue-1 (MaxTFCValue is refer to TS34.108 clause 6.10 Parameter Set.) |
| - PRACH TFCS | Not Present |
| - CHOICE mode | FDD |
| - UL DCH TFCS | (This IE is repeated for TFC number.) |
| - Normal |  |
| - TFCI Field 1 information |  |
| - CHOICE CTFC representation | Addition |
| - TFCS addition information |  |
| - CHOICE CTFC Size | Number of bits used must be enough to cover all combinations of CTFC from clause 6.10. |
| - CTFC information | Refer to TS34.108 clause 6.10 Parameter Set |
| - Power offset information |  |
| - CHOICE Gain Factors | Signalled Gain Factors |
| - Gain factor βc | 0 |
| - Gain factor βd | 0 |
| - Reference TFC ID | Not Present |
| - Power offset Pp-m | 0dB |
| Added or Reconfigured UL TrCH information |  |
| - Transport channel identity | 5 |
| - TFS |  |
| - CHOICE Transport channel type | Dedicated transport channels |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |
| - Transmission Time Interval | Not Present |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |
| - CHOICE Logical Channel list | ALL |
| - Semi-static Transport Format information |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |
| DRAC static information | Not Present |
| DL Transport channel information common for all transport channel |  |
| - SCCPCH TFCS | Not Present |
| - CHOICE DL parameters | Independent |
| - DL DCH TFCS | (This IE is repeated for TFC number.) |
| - Normal |  |
| - TFCI Field 1 information |  |
| - CHOICE TFC representation | Addition |
| - TFCS addition information |  |
| - CHOICE CTFC Size | Number of bits used must be enough to cover all combinations of CTFC from clause 6.10. |
| - CTFC information | Refer to TS34.108 clause 6.10 Parameter Set |
| - Power offset information | Not Present |
| Added or Reconfigured DL TrCH information |  |
| - Transport channel identity | 10 |
| - CHOICE DL parameters | Independent |
| - CHOICE Transport channel type | Dedicated transport channels |
| - TFS |  |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |
| - Transmission Time Interval | Not Present |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |
| - CHOICE Logical Channel list | ALL |
| - Semi-static Transport Format information |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |
| - DCH quality target |  |
| - BLER Quality value | -6.3 |
| - Transparent mode signalling info | Not Present |
| Frequency info |  |
| - UARFCN uplink(Nu) | Reference to TS34.108 clause 6.10 Parameter Set |
| - UARFCN downlink(Nd) | Reference to TS34.108 clause 6.10 Parameter Set |
| Maximum allowed UL TX power | 33dBm |
| Uplink DPCH info |  |
| -Uplink DPCH power control info |  |
| -DPCCH Power offset | -6dB |
| -PC Preamble | 8slot |
| -Power Control Algorithm | Algorithm1 |
| -TPC step size | 1dB |
| - Scrambling code type | Long |
| - Scrambling code number | 0 (0 to 16777215) |
| - Number of DPDCH | Not Present(1) |
| - spreading factor | SF is reference to TS34.108 clause 6.10 Parameter Set |
| - TFCI existence | TRUE |
| - Number of FBI bits | Not Present(0) |
| - Puncturing Limit | Reference to TS34.108 clause 6.10 Parameter Set |
| CHOICE Mode | FDD |
| - Downlink PDSCH information | Not Present |
| Downlink information common for all radio links | Not Present |
| - Downlink DPCH info common for all RL |  |
| - Timing indicator | Maintain |
| - CFN-targetSFN frame offset | Not Present |
| - Downlink DPCH power control information |  |
| - DPC mode | 0 (single) |
| - DL rate matching restriction information | Not Present |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Fixed or Flexible Position | Flexible |
| - TFCI existence | TRUE |
| - Number of bits for Pilot bits(SF=128,256) | Not Present |
| - DPCH compressed mode info |  |
| - TGPSI | 1 |
| - TGPS Status Flag | inactive |
| - TGCFN | (Current CFN + (256 – TTI/10msec)) mod 256 |
| - Transmission gap pattern sequence configuration parameters |  |
| - TGMP | FDD Measurement |
| - TGPRC | 62 |
| - TGSN | 8 |
| - TGL1 | 10 |
| - TGL2 | 5 |
| - TGD | 15 |
| - TGPL1 | 35 |
| - TGPL2 | 35 |
| - RPP | Mode 1 |
| - ITP | Mode 1 |
| - CHOICE UL/DL Mode | DL |
| - Downlink compressed mode method | SF/2 |
| - Downlink frame type | A |
| - DeltaSIR1 | 2.0 |
| - DeltaSIRafter1 | 1.0 |
| - DeltaSIR2 | Not Present |
| - DeltaSIRafter2 | Not Present |
| - TX Diversity mode | None |
| - SSDT information | Not Present |
| - S field |  |
| - Code Word Set |  |
| - Default DPCH Offset Value | 0 |
| Downlink information for each radio links |  |
| - Primary CPICH info |  |
| - Primary scrambling code | 100 |
| - PDSCH with SHO DCH info | Not Present |
| - PDSCH code mapping | Not Present |
| - Downlink DPCH info for each RL |  |
| - Primary CPICH usage for channel estimation | Primary CPICH may be used |
| - DPCH frame offset | 0 chips |
| - Secondary CPICH info | Not Present |
| - DL channelisation code |  |
| - Secondary scrambling code | 1 |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Code number | SF-1(SF is reference to TS34.108 clause 6.10 Parameter Set) |
| - Scrambling code change | No change |
| - TPC combination index | 0 |
| - SSDT Cell Identity | -a |
| - Closed loop timing adjustment mode | Not Present |
| - Secondary CCPCH info | Not Present |
| - TFCS | Not Present |
| - FACH/PCH information | Not Present |
| - References to system information blocks | Not Present |

# Contents of TRANSPORT CHANNEL RECONFIGURATION message: AM or UM (Packet to CELL_DCH from CELL_FACH in PS)

| Information Element | Value/remark |  |  |
| --- | --- | --- | --- |
| Message Type |  |  |  |
| RRC transaction identifier | Arbitrarily selects an integer between 0 and 3 |  |  |
| Integrity check info | The presence of this IE is dependent on IXIT statements in TS 34.123-2. If integrity protection is indicated to be active, this IE is present with the values of the sub IEs as stated below. Else, this IE and the sub-IEs are omitted. |  |  |
| - message authentication code | SS calculates the value of MAC-I for this message and writes to this IE. |  |  |
| - RRC message sequence number | SS provides the value of this IE, from its internal counter. |  |  |
| Integrity protection mode info | Not Present |  |  |
| Ciphering mode info | Not Present |  |  |
| Activation time | (256+CFN-(CFN MOD 8 + 8))MOD 256 |  |  |
| New U-RNTI | Not Present |  |  |
| New C-RNTI | Not Present |  |  |
| RRC State indicator | CELL_DCH |  |  |
| UTRAN DRX cycle length coefficient | Not Present |  |  |
| CN information info | Not Present |  |  |
| URA identity | Not Present |  |  |
| RB with PDCP information | Not Present |  |  |
| UL Transport channel information for all transport channels |  |  |  |
| - TFC subset | (This IE is repeated for TFC number.) |  |  |
| - Allowed Transport Format combination | 0 to MaxTFCValue-1 (MaxTFCValue is refer to TS34.108 clause 6.10 Parameter Set.) |  |  |
| - PRACH TFCS | Not Present |  |  |
| - CHOICE mode | FDD |  |  |
| - UL DCH TFCS | (This IE is repeated for TFC number.) |  |  |
| - Normal |  |  |  |
| - TFCI Field 1 information |  |  |  |
| - CHOICE CTFC representation | Addition |  |  |
| - TFCS addition information |  |  |  |
| - CHOICE CTFC Size | Number of bits used must be enough to cover all combinations of CTFC from clause 6.10. |  |  |
| - CTFC information | Refer to TS34.108 clause 6.10 Parameter Set |  |  |
| - Power offset information |  |  |  |
| - CHOICE Gain Factors | Signalled Gain Factors |  |  |
| - Gain factor βc | 0 |  |  |
| - Gain factor βd | 0 |  |  |
| - Reference TFC ID | Not Present |  |  |
| - Power offset Pp-m | 0dB |  |  |
| Added or Reconfigured UL TrCH information |  |  |  |
| - Transport channel identity | 1 |  |  |
| - TFS |  |  |  |
| - CHOICE Transport channel type | Dedicated transport channels |  |  |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |  |  |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |  |  |
| - Transmission Time Interval | Not Present |  |  |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - CHOICE Logical Channel list | ALL |  |  |
| - Semi-static Transport Format information |  |  |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| Added or Reconfigured UL TrCH information |  |  |  |
| - Transport channel identity | 5 |  |  |
| - TFS |  |  |  |
| - CHOICE Transport channel type | Dedicated transport channels |  |  |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |  |  |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |  |  |
| - Transmission Time Interval | Not Present |  |  |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - CHOICE Logical Channel list | ALL |  |  |
| - Semi-static Transport Format information |  |  |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| DRAC static information | Not Present |  |  |
| DL Transport channel information common for all transport channel |  |  |  |
| - SCCPCH TFCS | Not Present |  |  |
| - CHOICE DL parameters |  |  |  |
| - DL DCH TFCS | (This IE is repeated for TFC number.) |  |  |
| - Normal |  |  |  |
| - TFCI Field 1 information |  |  |  |
| - CHOICE CTFC representation | Addition |  |  |
| - TFCS addition information |  |  |  |
| - CHOICE CTFC Size | Number of bits used must be enough to cover all combinations of CTFC from clause 6.10. |  |  |
| - CTFC information | Refer to TS34.108 clause 6.10 Parameter Set |  |  |
| - Power offset information | Not Present |  |  |
| Added or Reconfigured DL TrCH information |  |  |  |
| - Transport channel identity | 6 |  |  |
| - CHOICE DL parameters | Independent |  |  |
| - TFS |  |  |  |
| - CHOICE Transport channel type | Dedicated transport channels |  |  |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |  |  |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |  |  |
| - Transmission Time Interval | Not Present |  |  |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - CHOICE Logical Channel list | ALL |  |  |
| - Semi-static Transport Format information |  |  |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - DCH quality target |  |  |  |
| - BLER Quality value | -6.3 |  |  |
| - Transparent mode signalling info | Not Present |  |  |
| Added or Reconfigured DL TrCH information |  |  |  |
| - Transport channel identity | 10 |  |  |
| - CHOICE DL parameters | Independent |  |  |
| - TFS |  |  |  |
| - CHOICE Transport channel type | Dedicated transport channels |  |  |
| - Dynamic Transport format information | (This IE is repeated for TFI number) |  |  |
| - RLC Size | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - Number of TBs and TTI List | (This IE is repeated for TFI number.) |  |  |
| - Transmission Time Interval | Not Present |  |  |
| - Number of Transport blocks | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - CHOICE Logical Channel list | ALL |  |  |
| - Semi-static Transport Format information |  |  |  |
| - Transmission time interval | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - Type of channel coding | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - Coding Rate | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - Rate matching attribute | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - CRC size | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - DCH quality target |  |  |  |
| - BLER Quality value | -6.3 |  |  |
| - Transparent mode signalling info | Not Present |  |  |
| Frequency info |  |  |  |
| - UARFCN uplink(Nu) | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - UARFCN downlink(Nd) | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| Maximum allowed UL TX power | 33dBm |  |  |
| Uplink DPCH info |  |  |  |
| -Uplink DPCH power control info |  |  |  |
| -DPCCH Power offset | -6dB |  |  |
| -PC Preamble | 8slot |  |  |
| -Power Control Algorithm | Algorithm1 |  |  |
| -TPC step size | 1dB |  |  |
| - Scrambling code type | Long |  |  |
| - Scrambling code number | 0 (0 to 16777215) |  |  |
| - Number of DPDCH | Not Present(1) |  |  |
| - spreading factor | SF is reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - TFCI existence | TRUE |  |  |
| - Number of FBI bits | Not Present(0) |  |  |
| - Puncturing Limit | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| CHOICE Mode | FDD |  |  |
| - Downlink PDSCH information | Not Present |  |  |
| Downlink information common for all radio links |  |  |  |
| - Downlink DPCH info common for all RL |  |  |  |
| - Timing indicator | Maintain |  |  |
| - CFN-targetSFN frame offset | Not Present |  |  |
| - Downlink DPCH power control information |  |  |  |
| - DPC mode | 0 (single) |  |  |
| - DL rate matching restriction information | Not Present |  |  |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - Fixed or Flexible Position | Flexible |  |  |
| - TFCI existence | TRUE |  |  |
| - Number of bits for Pilot bits(SF=128,256) | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - DPCH compressed mode info |  |  |  |
| - TGPSI | 1 |  |  |
| - TGPS Status Flag | inactive |  |  |
| - TGCFN | (Current CFN + (256 – TTI/10msec)) mod 256 |  |  |
| - Transmission gap pattern sequence configuration parameters |  |  |  |
| - TGMP | FDD Measurement |  |  |
| - TGPRC | 62 |  |  |
| - TGSN | 8 |  |  |
| - TGL1 | 10 |  |  |
| - TGL2 | 5 |  |  |
| - TGD | 15 |  |  |
| - TGPL1 | 35 |  |  |
| - TGPL2 | 35 |  |  |
| - RPP | Mode 1 |  |  |
| - ITP | Mode 1 |  |  |
| - CHOICE UL/DL Mode | DL |  |  |
| - Downlink compressed mode method | SF/2 |  |  |
| - Downlink frame type | A |  |  |
| - DeltaSIR1 | 2.0 |  |  |
| - DeltaSIRafter1 | 1.0 |  |  |
| - DeltaSIR2 | Not Present |  |  |
| - DeltaSIRafter2 | Not Present |  |  |
| - TX Diversity mode | None |  |  |
| - SSDT information | Not Present |  |  |
| - Default DPCH Offset Value | 0 |  |  |
| Downlink information for each radio links |  |  |  |
| - Primary CPICH info |  |  |  |
| - Primary scrambling code | 100 |  |  |
| - PDSCH with SHO DCH info | Not Present |  |  |
| - PDSCH code mapping | Not Present |  |  |
| - Downlink DPCH info for each RL |  |  |  |
| - Primary CPICH usage for channel estimation | Primary CPICH may be used |  |  |
| - DPCH frame offset | 0 chips |  |  |
| - Secondary CPICH info | Not Present |  |  |
| - DL channelisation code |  |  |  |
| - Secondary scrambling code | 1 |  |  |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |  |  |
| - Code number | SF-1(SF is reference to TS34.108 clause 6.10 Parameter Set) |  |  |
| - Scrambling code change | No change |  |  |
| - TPC combination index | 0 |  |  |
| - SSDT Cell Identity | -a |  |  |
| - Closed loop timing adjustment mode | Not Present |  |  |
| - Secondary CCPCH info | Not Present |  |  |
| - TFCS | Not Present |  |  |
| - FACH/PCH information | Not Present |  |  |
| - References to system information blocks | Not Present |  |  |