| TSG-T Working Group1 SWG SIG meeting #17 |  |  | T1S-01-0125 |
| --- | --- | --- | --- |
| Melbourne, Australia |  | 14th to 16th May 2001 |  |
| Title | Update to 34.123-3: MAC ASP definitions to include MAC header control |  |  |
| Source | Anritsu |  |  |
| Document for | Agreement |  |  |
| Contact | Dan Fox (Anritsu) dan.fox@eu.anritsu.com Tel: +44 1582 433357 |  |  |

##### 7.3.2.2.13	CMAC_Config_REQ

| ASN.1 ASP Type Definition |  |
| --- | --- |
| Type Name | CMAC_Config_REQ |
| PCO Type | CSAP |
| Comment | To request to configure MAC entity |
| Type Definition |  |
| SEQUENCE { cellId INTEGER(0..15), routingInfo RoutingInfo, ratType RatType, configMessage CmacConfigReq } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | CmacConfigReq |
| Comment | To request to configure MAC |
| Type Definition |  |
| SEQUENCE { uE_Info UE_Info, trCHInfo TrCHInfo, trCH_LogCHMapping TrCH_LogCHMappingList1 -- RACHTrasmissionCtrolElements TBD, -- CPCHTransmissionControlElements TBD } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | UE_Info |
| Comment |  |
| Type Definition |  |
| SEQUENCE { u_RNTI SEQUENCE { srnc_Identity BIT STRING (SIZE (12)), s_RNTI BIT STRING (SIZE (20)) } OPTIONAL, c_RNTI BIT STRING (SIZE (16)) OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | TrCH_LogCHMappingList1 |
| Comment |  |
| Type Definition |  |
| SEQUENCE { UlconnectedTrCHList SEQUENCE (SIZE (1..maxulTrCH)) OF SEQUENCE { trchid TransportChannelIdentity, trCH_LogCHMappingList TrCH_LogCHMappingList }, OPTIONAL, dlconnectedTrCHList SEQUENCE (SIZE (1..maxdlTrCH)) OF SEQUENCE { trchid TransportChannelIdentity, trCH_LogCHMappingList TrCH_LogCHMappingList }, OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | TrCH_LogCHMappingList |
| Comment |  |
| Type Definition |  |
| SEQUENCE (SIZE (1..maxLogCHperTrCH)) OF TrCH_LogicalChannelMapping |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | TrCHInfo |
| Comment |  |
| Type Definition |  |
| SEQUENCE { ulconnectedTrCHList SEQUENCE (SIZE (1..maxulTrCH)) OF SEQUENCE { trchid TransportChannelIdentity, TransportChannelInfo CHOICE { ul_CommonTrCHInfo CommonTransChTFS, ul_DedTrCHInfo DedicatedTransChTFS}, } OPTIONAL, ulctfcList SEQUENCE (SIZE (0..maxTFC)) OF SEQUENCE { ctfc INTEGER (0..1023), } OPTIONAL, dlconnectedTrCHList SEQUENCE (SIZE (1..maxdlTrCH)) OF SEQUENCE { trchid TransportChannelIdentity, TransportChannelInfo CHOICE { dl_CommonTrCHInfo CommonTransChTFS, dl_DedTrCHInfo DedicatedTransChTFS}, } OPTIONAL, dlctfcList SEQUENCE (SIZE (0..maxTFC)) OF SEQUENCE { ctfc INTEGER (0..1023) } OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | TrCH_LogicalChannelMapping |
| Comment |  |
| Type Definition |  |
| SEQUENCE { CHOICE { ul_LogicalChannelMapping SS_UL_LogicalChannel_Mapping, dl_LogicalChannelMapping SS_DL_LogicalChannel_Mapping }, rB_Identity INTEGER {-32..31}, cn-DomainIdentity CN-DomainIdentity OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_UL_LogicalChannel_Mapping |
| Comment |  |
| Type Definition |  |
| SEQUENCE { ul_TransportChannelType SS_UL_TransportChannel_Type, logicalChannelIdentity LogicalChannelIdentity, ctValue INTEGER(0..15) OPTIONAL, tctfValue BITSTRING(SIZE(2..8)) OPTIONAL, ueIdentity CHOICE { u-RNTI U-RNTI, c-RNTI C-RNTI } OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_DL_LogicalChannel_Mapping |
| Comment |  |
| Type Definition |  |
| SEQUENCE { dlTransportChannelType SS_DL_TransportChannel_Type, logicalChannelIdentity LogicalChannelIdentity, mac_LogicalChannelPriority MAC_LogicalChannelPriority OPTIONAL ctValue INTEGER(0..15) OPTIONAL, tctfValue BITSTRING(SIZE(2..8)) OPTIONAL, ueIdentity CHOICE { u-RNTI U-RNTI, c-RNTI C-RNTI } OPTIONAL } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_UL_TransportChannel_Type |
| Comment |  |
| Type Definition |  |
| ENUMELATED { dch, rach, cpch, usch } |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | MAC_LogicalChannelPriority |
| Comment |  |
| Type Definition |  |
| INTEGER (1..8) |  |

| ASN.1 Type Definition |  |
| --- | --- |
| Type Name | SS_DL_TransportChannel_Type |
| Comment |  |
| Type Definition |  |
| ENUMELATED { dch, fach, bch, pch, dsch } |  |

Rapporteur: Shicheng Hu

Shicheng.hu@etsi.fr