This document provides a first attempt at a table listing the requirements for a MExE ME as mentioned in 02.57, MExE Service Description, Stage 1.

The aim of this table is to provide a requirements framework in order to allow subsequent specification work such as API definition to be performed more efficiently.

The following provides information relating to the table headings:

| Key to Requirements Table Headings |  |
| --- | --- |
| Heading | Explanation |
| i.d. | A unique identifier for each requirement. An identifier can be assigned only once. This heading also contains the [i.d.] of the next requirement to be entered - embedded as a table function (F9 to update). |
| Category | A series of broad categories into which requirements are placed. "J" means this requirement relates directly to the specification of a Java API. "I" means this requirement relates to implementation, and puts the specification of a Java API into context. |
| Requirement | The actual requirement, which should be as unambiguous (and indivisible) as possible |
| Reference | A reference into a document within which the requirement is mentioned, if applicable |
| Priority | Requirements may be mandatory or optional |
| Classmark | An indication as to which classmark the requirement is applicable or in-applicable (in which case the classmark figure is shown in brackets) |
| Release | An indication of whether the requirement is applicable to Release 98, 99 etc. |
| Comment | A general purpose comment field |
| Applicable API | Applies to Java enabled devices. A list of the APIs which satisfy the requirement |
| Chosen API | Applies to Java enabled devices. The API chosen from the "Applicable API" list. |

| i.d. [107] | Category | Requirement | Reference | Priority | Class-mark | Release | Comment | Applicable APIs | Chosen API |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | High Level |  |  |  |  |  |  |  |  |
| 1 | J | The MExE ME shall provide a means for applications, applets and content to access the user interface |  | Mandatory |  |  |  |  |  |
| 3 | J | The MExE ME shall provide the means to allow the transfer of applications, applets or content either automatically or on demand according to MExE security requirements | 02.57 v7.0.0 6.1.0 | Mandatory |  |  |  |  |  |
| 4 | I | The MExE ME shall provide the means to allow the upgrade of existing applications across the GSM/UMTS network subject to MExE security requirements and user authorisation | 02.57 v7.0.0 6.1.0 | Mandatory |  |  |  |  |  |
| 5 | I | The MExE ME shall allow the user to personalise services | 02.57 v7.0.0 6.1.0 | Mandatory |  |  | FFS – relates to user profile (i.e. the way the information is displayed on the MExE ME , not the type/nature of information). DELETE FROM CLARIFICATION LIST BUT DO NOT CR |  |  |
| 6 | J | The MExE terminal shall allow the user to utilise both internet and intranet applications, applets and content according to MExE security requirements | 02.57 v7.0.0 6.1.0 | Mandatory |  |  |  |  |  |
| 54 | J | The MExE ME shall possess an architecture that is capable of supporting applications, applets and content in a standard execution environment independently of the MExE ME manufacturer | 02.57 v7.0.0 9.1.0 | Mandatory |  |  | CR |  |  |
|  | Classmarks |  |  |  |  |  |  |  |  |
| 11 | I | MExE applications and applets conforming to a particular MExE classmark shall be supportable by all MExE MS’s of that classmark regardless of MExE MS manufacturer | 02.57 v7.0.0 5.3 | Mandatory |  |  |  |  |  |
|  | User Interface Configuration Management |  |  |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |  |  |  |
| 2 | # I | The MExE ME shall allow the user to personalise the user interface within bounds defined by the manufacturer | 02.57 v7.0.0 6.1.0 | Mandatory |  |  | The extent of "personalisation" will be limited by the terminal manufacturer. E.g. screen colours, ring patterns, screen saver This needs further clarification – Nash |  |  |
| 15 | I | The user shall be able to manage the user interface configuration of the MExE MS within bounds defined by the manufacturer | 02.57 v7.0.0 6.2.1 | Mandatory |  |  | The extent of "personalisation" will be limited by the terminal manufacturer. |  |  |
| 16 | I | The MExE ME shall allow the user to identify the current user interface configuration within bounds defined by the manufacturer | 02.57 v7.0.0 6.2.1 | Mandatory |  |  |  |  |  |
| 17 | I | The MExE ME shall allow the user to add, modify or delete a particular user configuration within bounds defined by the manufacturer | 02.57 v7.0.0 6.2.1 | Mandatory |  |  |  |  |  |
| 18 | I | It shall be possible to obtain the user interface configuration information, subject to MExE security requirements, which is resident on the MExE ME, the SIM or the network | 02.57 v7.0.0 6.2.1 | Mandatory |  |  | FFS – similar to “.dt”, “.xdefaults” in Unix. [Further contribution expected from Nash]. Change Request |  |  |
| 20 |  | The user interface configuration management information may be stored on the MExE ME, the SIM or the network, subject to MExE security requirements. |  | Mandatory |  |  |  |  |  |
| 22 | J | Any MExE ME shall be able to access and interpret user interface configuration data, according to MExE security requirements |  | Mandatory |  |  | Assume “user profile” means personal details i.e. name, e-mail etc. Clarification required on defn of terms “The user profile data shall be associated with a given user” is part of defn (req 23) “User profile data” is a superset of “user interface configuration” – partitioning/access is FFS. NB: “interpret” may need to be clarified! CR – add to end of para3.6.2.1, 02.57 |  |  |
|  | Service Management |  |  |  |  |  |  |  |  |
| 25 | I | The MExE ME shall allow the user to request the range of MExE services available from the PLMN MExE service provider | 02.57 v7.0.0 6.2.2 | Mandatory |  |  | Misgivings about how registration is carried out, notification messages etc. What is the notification mechanism? This is perceived as a common address (e.g. URL. Same address could be used when roaming – use (e.g.) DNS to resolve actual server address). This address is used to provide the first point of access to a MExE network. The user is provided with a list of MexE services available from the PLMN service provider. MExE is allowed to stipulate only access to GSM/GPRS/UMTS MExE SPs – PLMN independent SPs lie outside of the scope of MExE standardisation. When roaming, the user could collect a number of addresses, including HPLMN and VPLMN. |  |  |
| 26 | I | The MExE ME shall allow the user to determine which MExE applications, applets and content are currently resident on the MExE ME | 02.57 v7.0.0 6.2.2 | Mandatory |  |  | Resident in terminal only or also in network? |  |  |
| 103 |  | The MExE ME shall be able to determine whether or not the GSM/GPRS/UMTS connection required by an application has been broken |  | Mandatory |  |  |  |  |  |
| 102 |  | The MExE ME shall allow the user to control , via the user profile, whether or not an application is automatically executed once it has been downloaded, according to MExE security requirements | 03.57 4.x | Optional |  |  | Execution may in some instance include configuration |  |  |
|  | Bearer Services |  |  |  |  |  |  |  |  |
| 30 | J | The MExE ME shall provide access to a range of one or more GSM/GPRS/UMTS bearer services, within the capabilities of the ME and network | 02.57 v7.0.0 7.0.0 | Mandatory |  |  | Transport protocols shall be defined in other specs (stage2,3?). Use only standard protocols (no proprietary) |  |  |
|  | Protocols |  |  |  |  |  |  |  |  |
| 34 | J | MExE protocols shall be independent of the type of underlying bearer | 02.57 v7.0.0 8.2.0 | Mandatory |  |  | What are upper layer MExE protocols? Definition required – suggest all layers above the bearer. Definition accepted – CR to definition section of 02.57 |  |  |
| 36 | J | Applications and applets shall be able to request specific bearer services and to be notified as to their availability | 02.57 v7.0.0 8.2.0 | Optional |  |  |  |  |  |
| 38 | J | MExE protocols shall be able to add new bearers | 02.57 v7.0.0 8.3.0 | Mandatory |  |  |  |  |  |
| 40 | I | It shall be possible for applications and services to install new protocols | 02.57 v7.0.0 8.4.0 | Optional |  |  | CR |  |  |
| 41 | I | MExE protocols shall be independent of the network nodes being communicated with | 02.57 v7.0.0 8.5.0 | Mandatory |  |  |  |  |  |
| 43 | I | MExE protocols shall support a generic technology independent capability negotiation, notification and enquiry mechanism between the MExE ME and the MExE service environment | 02.57 v7.0.0 8.6.0 | Mandatory |  |  |  |  |  |
| 100 | I | The MExE ME shall allow a MExE service provider to determine the MExE ME capabilities, according to MExE security requirements. | 02.57 v7.0.0 6.1.0 | Mandatory |  |  | NB: MExE Service Provider can be anybody – hence apply MExE security policy |  |  |
| 101 | I | The MExE ME shall perform capability and content negotiation according to Section 4.4. of 03.57 | 03.57 Section 4.4 | Mandatory |  |  | Table reference IN NEW VERSION |  |  |
| 48 | I | The capability negotiation mechanism shall be extensible. | 02.57 v7.0.0 8.6.0 | Mandatory |  |  | CR |  |  |
| 50 | I | MExE protocols shall support the capability to transfer new applications, applets and content to and from the MExE ME as required by the user according to MexE security requirements | 02.57 v7.0.0 8.8.0 | Mandatory |  |  | CR required |  |  |
| 51 | I | The MExE protocols shall support both user initiated and MexE server initiated transfer of applications, applets and content as required by the user, according to MExE security requirements | 02.57 v7.0.0 8.8.0 | Mandatory |  |  | CR required to section 8.8 of 02.57v7.0.0 |  |  |
|  | MS Application Execution Environment |  |  |  |  |  |  |  |  |
| 55 | J | Ability to cleanly and safely install applications and associated data (e.g. icons) shall be provided. |  | Optional |  |  |  |  |  |
| 56 | J | Ability to cleanly and safely de-install applications and associated data (e.g. icons) shall be provided. |  | Optional |  |  |  |  |  |
| 57 | I | The MExE classmark 2 terminal shall be able to implement HTML |  | Mandatory |  |  | WML for classmark 1, HTML for classmark 2 This an an API req, relating to stage 2. |  |  |
| 58 | I | The MExE classmark 2 terminal shall be able to implement WML and WMLScript |  | Mandatory |  |  | This an an API req, relating to stage 2. |  |  |
| 105 | I | The MExE Classmark2 device shall contain a pre-installed WAP browser |  | Mandatory |  |  | I.e. browser capable of rendering WML and WMLScript |  |  |
| 106 | I | The MExE Classmark2 device shall allow the pre-installed WAP browser to be updated |  | Mandatory |  |  |  |  |  |
| 59 | I | MExE shall define a set of standardised content coding schemes i.e. vCard, vCalendar |  | Mandatory |  |  | Is vCard, vCalendar support required? This an an API req, relating to stage 2. |  |  |
| 60 | I | The MExE ME shall be able to support HTML and/or WML |  | Mandatory |  |  |  |  |  |
| 61 | J | MExE ME shall support the 7bit GSM default character set |  | Mandatory |  |  | As defined in 03.38 v6.0.1 Note: User defined 8bit scheme is not mentioned here. Should it be? CR to section 9.2 (last para) of 02.57v7.0.0 – replace “defined” with “identified” |  |  |
| 104 | I | The MExE ME shall be capable of supporting the UCS2 character coding scheme |  | Optional |  |  | Unicode 16bit character coding |  |  |
| 63 | I | The MExE ME shall support a core API, known as the core MExE API | 02.57 v7.0.0 9.3.0 | Mandatory |  |  | The core MExE API defines the minimal API set that a MExE ME needs to support |  |  |
| 64 | I | The core MExE API shall consist of both generic and GSM/UMTS specific aspects | 02.57 v7.0.0 9.3.0 | Mandatory |  |  |  |  |  |
| 65 | I | The MExE ME shall support standardised MExE API extensions in addition to the core MExE API | 02.57 v7.0.0 9.3.0 | Optional |  |  |  |  |  |
| 66 | J | The standardised MExE API extensions shall include access to location information, according to MExE security | 02.57 v7.0.0 9.3.0 | Mandatory |  |  | What is the meaning of "mobility information"? CR Location |  |  |
| 67 | J | The MExE APIs shall provice support for applications and applets to access the GSM/UMTS functionality present in the GSM/UMTS terminal according to MExE security . |  | Mandatory |  |  | Functionality includes telephony, e-mail, SMS, fax. |  |  |
| 68 | J | All Java APIs specified by MExE for use on a MExE ME shall be approved by Sun Microsystems. |  | Mandatory |  |  |  |  |  |
| 69 | J | The MExE ME shall provide local file system support, according to MExE security | 02.57 v7.0.0 6.1.0 | Optional |  |  |  |  |  |
| 70 | I | The MExE ME shall allow the user to identify the version number of applications, applets and content contained within it | 02.57 v7.0.0 6.1.0 | Optional |  |  |  |  |  |
| 71 | J | A MExE ME shall notify applications of any power state changes. | 02.57 v7.0.0 6.1.0 | Optional |  |  |  |  |  |
| 72 | J | A MExE ME shall support standardised access of a smartcard if a smartcard reader is present on the MExE ME | 02.57 v7.0.0 6.1.0 | Optional |  |  | Support of smartcard reader is not a mandatory requirement. |  |  |
|  | Security |  |  |  |  |  |  |  |  |
| 76 | I | Transferred MExE applications, applets and content shall be secure and verified | 02.57 v7.0.0 11.0.0 | Mandatory |  |  | “secure” – received in same state as it was sent i.e. has not been tampered with |  |  |
| 77 | I | The MExE ME shall provide support for four security levels | 02.57 v7.0.0 11.0.0 | Mandatory |  |  | See Table XX in Section 9.1 of 03.57 for an elaboration of security levels. Multiple security levels |  |  |
| 79 | I | The security level of applications, applets and content shall be verified by the MExE ME | 02.57 v7.0.0 11.0.0 | Mandatory |  |  |  |  |  |
| 81 | I | The MExE subscriber shall be able to control whether applications, applets and content of specific security levels may be transferred to the MExE ME | 02.57 v7.0.0 11.0.0 | Mandatory |  |  |  |  |  |
| 82 | J | An application/applet shall specifiy whether or not it can share its own files with other applications/applets | 02.57 v7.0.0 11.0.0 | Mandatory |  |  | More input required here |  |  |
| 83 | I | Within the constraints of the defined security levels, the user shall have ultimate control over the use of bearer services | 02.57 v7.0.0 11.0.0 | Mandatory |  |  | CR Incorporate USSD and SS signalling (comment by K Holley at MExE security meeting, SMG4 plen 11 – 14th Dec 1998) |  |  |
| 84 | I | A means to authenticate the MExE subscriber and MExE server shall be provided | 02.57 v7.0.0 11.0.0 | Mandatory |  |  | CR This may be performed by the MExE ME, an intermediate entity or the GSM/UMTS network |  |  |
| 85 | I | MExE shall continue to support the inherent security and integrity of the GSM/GPRS/UMTS network | 02.57 v7.0.0 11.0.0 | Mandatory |  |  |  |  |  |
| 86 | I | The integrity of the SIM and existing GSM/UMTS security mechanisms shall not be compromised by the introduction of MExE services | 02.57 v7.0.0 11.0.0 | Mandatory |  |  |  |  |  |
| 87 | I | All aspects relating to GSM security shall be performed by the SIM | 02.57 v7.0.0 6.1.0 | Mandatory |  |  |  |  |  |
|  | Interworking with other Network Features |  |  |  |  |  |  |  |  |
| 90 | I | All services available in the network shall continue to be offered in addition to MExE services | 02.57 v7.0.0 12.0.0 | Mandatory |  |  |  |  |  |
|  | Network Interworking |  |  |  |  |  |  |  |  |
| 92 | I | All services offered in co-operation with other networks shall continue to be offered in parallel with MExE services | 02.57 v7.0.0 13.0.0 | Mandatory |  |  |  |  |  |
|  | Communications |  |  |  |  |  |  |  |  |
| 93 | J | MExE MEs shall support peer to peer communication between MExE devices | 02.57 v7.0.0 6.1.0 | Optional |  |  | Includes e.g. IrDA, Bluetooth, USB (which are outside the scope of MExE specifications), user-to-user signalling, USSD (which are within the scope of MExE specifications) etc. |  |  |
| 95 | J | A MExE application shall be able to access any or all external interfaces present on the MExE ME, subject to the limitations of the MExE ME |  | Optional |  |  | Serial, parallel, infra-red etc. |  |  |
| 96 | J | The means for MExE service provider specific applications and applets on the MExE ME to communicate with other GSM network nodes shall be provided. | 02.57 v7.0.0 6.1.0 | Optional |  |  | USSD main bearer |  |  |