ETSI SMG4 / 3GPP TSG-T MExE	Tdoc SMG4 4M99-051

Helsinki, Finland

17th-19th February, 1999

| CHANGE REQUEST No : |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 03.57 |  | Version |  | 1.4.0 |  |  |  |
| Submitted to SMG |  |  | For approval |  |  | without presentation ("non-strategic") |  |  |  |  |  |
| list plenary meeting or STC here  |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | X | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | MExE |
| --- | --- |

| Source: | Motorola | Date: | 12/01/99 |
| --- | --- | --- | --- |

| Subject: | User profile changes from the security table |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature | X |  | Release 98 | X |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | Changes are required to the user profile to bring it in line with the security table. The specific changes involve:- - Pre-prescribed directories in read-file/write-file/delete-file/get-file-information actions may not be performed - User limit on the total write-space allocated to an application. - Pre-prescribed directories in which a saved application or applet may not be deleted (note that the restriction has been inverted!) [Note that this CR contains an updated security table featured in a different CR]. |
| --- | --- |

| Clauses affected: |  |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

The user profile acts as a repository (which is always available in the MExE MS) defining the MExE MS behaviour, identifying one or more of the following:-

|  | ME specific | Location(s) | Description |
| --- | --- | --- | --- |
| user interface |  |  | Refer to sub-clause 4.6 User interface customisation for details of the specific user interface customisation characterisation which may be stored in the user profile |
| background - colour - pattern/image |  |  |  |
| - font - type - colour - size |  |  |  |
| - interfaces - softkey definition - additional menu items - enabled/disabled extended menus - icon settings |  |  |  |
| - country - language - date - time - currency |  |  |  |
| - Modify supplementary services settings (i.e. diversion, times) | - ? |  | - ? |
| ###### Modify phone settings - ringer tone/vibrate - enabling SIM Lock - Keypad Lock |  |  |  |
| - Preferences |  |  | - 03.57, section 4.4.1 - ”Support of the user profile preferences is subject to the capabilities of the device. Generally the combination of user profile preferences and ME logic will determine the information sent in the capability negotiation from the MExE device to MSE.” |
| - terminal properties - language in which the user wishes to work - preferred input mode - amount of memory authorised by the user for MExE |  |  |  |
| - data transfer - prioritised list of preferred bearers, e.g. SMS, USSD, GPRS - maximum size of transfer - maximum duration of transfer - transfer of location information - transfer of the user profile or parts of it |  |  | - may, or may not be, sent to the MSE - could be shifted in Security section - could be shifted in Security section |
| - codecs - prioritised list of preferred audio codecs - prioritised list of preferred image codecs - prioritised list of preferred video codecs - prioritised list of preferred multimedia codecs |  |  |  |
| Service Management |  |  | Refer to sub-clauses 4.5, 4.7.1, 4.7.2 controls the ability to determine which services are transferred to, resident, configured or executing on the MS, which may be refined by use of one or more of the following characteristics: identified security level specific permitted services types of permitted services date or times for actions priorities of services and identifies service preferences which may be set globally for all services per security level on a per service basis and may be identified at installation of the service, or be subsequently modified. Pre-prescribed directories in which read-file/write-file/delete-file/get-file-information actions may not be performed is identified. User limit on the total write-space allocated to each specific application is identified. Pre-prescribed directories in which a saved application or applet may not be deleted is identified. |
| transfer of services |  |  |  |
| automatic configuration |  |  |  |
| execution of services |  |  |  |
| residence on the MExE MS |  |  |  |
| personalisation of services |  |  | 02.57, section 6.1 for further details see section VHE of this table |
| Security |  |  | control of the following permissions maybe refined by use of one or more of the following characteristics: - universal authorisation for all calls/connections/ messages (i.e. yes/no) - authorisation by calls/connections/messages type (i.e. SMS, USSD etc.) - authorisation by identified services - authorisation by service type (i.e. security level) - authorisation by date/time/day of week |
| call authorisation |  |  | 03.57, section 4.5 |
| Implicit permission for the initiation of events (includes responses made by the MS to external events) of transferred applications, applets |  |  | 02.57, section 11.1 and 11.2 (Timothy Wright) |
| Implicit permission for the transfer of applications, applets and content |  |  | 02.57, section 11.1 and 11.2 (Timothy Wright) |
| Network event journal - status of the optional network event journal which is used to maintain a record of application initiated network events (i.e. activated/deactivated) |  |  |  |
| Miscellaneous |  |  |  |
| phonebook entry |  |  | 03.57, section 4.6 |
| ###### Redirect - voicemail box number - fax-mail box number |  |  |  |
| - reset call meters |  |  | - 03.57, section 4.6 |
| Classmark 1 specific Entries |  |  |  |
| Classmark 2 specific Entries |  |  |  |
| ###### WAP browser - pre-installed browser overwrite on upgrade - location of user preferred WAP browser |  |  | - Applicable to MExE classmark 2 |
| VHE |  |  | TS 22.xx :Provision of Services in UMTS – The Virtual Home Environment (Tdoc SMG1 825/98) |
| - for each VHE profile: - Incoming session routing address |  |  |  |
| - for each service within one VHE profile: - service state - service appearance - service appearance customisation parameters - service functionality customisation parameters - service data |  |  | - e.g. activate/deactivate/not available - e.g. level of functionality complexity - e.g. forwarded-to numbers |

The above list is not exhaustive.

Support of the user profile preferences is subject to the capabilities of the device.

4.6	User interface customisation

The MS interface consists of the buttons, menus, screens and MMI as designed and provided by the MS manufacturer; the nature of this MS interface is naturally evolving, MS specific and proprietary to the individual manufacturers of the industry.  This interface is the one normally seen by the user in normal operation of his MS.  This specification does not place any requirements or limitations on the individual manufacturers’ MS interface.

|  | MExE Security Levels |  |  |  |
| --- | --- | --- | --- | --- |
| Permission / Privilege |  | 1 | 2 | 3 | 4 |
| - Read file - Write file - Get file information - Delete file | Yes (except in user profile specified directories) |  |  | Yes (only in application/applet’s own directory) |
| Load native code libraries | No | Yes | No |  |
| - Initiate a voice/data connection - Accept a voice/data connection - Intercept an incoming voice/data connection. (A network connection may be via any supported bearer service). | Yes | Yes (with user permission) |  | No |
| Call forward | Yes (with user permission) | No | Yes (with user permission, to pre-defined numbers in the user profile) | No |
| - Terminate an existing connection - Hold an existing connection - Resume an existing connection | Yes |  | Yes (with user permission) | No |
| Modification of user invoked connection | Yes |  | Yes (with user permission) | No |
| - Add a phonebook entry - Delete a phonebook entry - Get a phonebook entry - Modify a phonebook entry | Yes (with user permission) |  |  | No |
| Send an SMS | Yes | Yes (with user permission) |  | No |
| Add an application or applet | Yes (with user permission) |  |  | No |
| Delete a saved application or applet | Yes (with user permission) |  | Yes (with user permission, except in user profile specified directory) | No |
| - Add a certificate for a given level - Delete a certificate for a given level - Replace a certificate for a given level | Only the organisation whose public key is certified (or the organisation that certified the public key) can add, delete or replace a particular certificate (with user permission) |  |  | No |
| Audio access (e.g. wake-up service) | Yes |  |  | No |
| HTTP Cache manipulation | Yes |  |  | No |
| Get IMEI | Yes |  |  | No |
| Location Information | Yes (with user permission) | No | Yes (with user permission) | No |
| - Activate a user profile - Modify a user profile | Yes (with user permission) | No | Yes (with user permission) | No |