ETSI SMG4	Tdoc SMG4 4M99-064

Helsinki

17-19.2.1999

| CHANGE REQUEST No : |  |  |  |  |  |  | . |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 03.57 |  | Version |  | 1.4.0 |  |  |  |
| Submitted to SMG |  |  | for approval |  | x | without presentation ("non-strategic") |  |  |  |  |  |
|  |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | x | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: |  |
| --- | --- |

| Source: | Nokia | Date: | 10.2.1999 |
| --- | --- | --- | --- |

| Subject: | Capability tokens |
| --- | --- |

| Category: | F Correction | x | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 | x |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | SMS is one possible bearer, so compactness of the tokens and values should be considered. I suggest that instead of using "MExE-" as a prefix, using "M" would be sufficient, and instead of using dashes ("-") in the tokens, using uppercase to indicate the start of a word. e.g. "MKbdId" rather than "MexE-kbd-id". |
| --- | --- |

| Clauses affected: | 4.4.1 |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  | ® List of CRs: |  |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: | The document does not explicitly say that ISO 639 shall be used for language names. |
| --- | --- |

### 4.4.1	Capability negotiation characteristics

The generic set of capabilities which may be negotiated between the client and the server consists of the subsequently identified characteristics.:-

| Token | Characteristics |  |  | Optionality |  | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | MS to MSE | MSE to MS |  |
| MClass | MExE Classmark |  |  | M | O | Classmark of the MExE device, e.g. 1, 2, etc. Example coding: MClass: 1 |
| MSpec | MExE specification |  |  | O | O | Version of the MExE Stage 2 standard supported by the MS; represents the first two digits of the 03.57 specification version number. Example coding: MSpec: 7.0 |
| MSrvcs | MExE services |  |  | O | O | Identification on where information on the PLMN-provided MExE services may be found, e.g. URL Example coding: MSrvcs: http:://telco.mexe.com/services |
| MManuf | Device | Manufacturer |  | O | O | Identification of the manufacturer, e.g. Motorola, Nokia, Ericsson, etc Example coding: MManuf: Motorola |
| MModel |  | Model |  | O | O | Model type, e.g. 130, 8810, 688, etc. Example coding: MModel: 130 |
| MSN |  | Serial Number |  | O | O | Manufacturer’s model, e.g. 1.1.0 etc. Example coding: MSN: 1.1.0 |
| MSV |  | SoftwareVersion |  | O | O | Manufacturer’s software version, IMEISV Example coding: MSV: nnnnnnnnnnnnnnn |
| MRes | Display | Size | resolution | O | O | Pixel size of the display, e.g. 200x150 etc. Example coding: MResol: 1.1.0 |
| MChars |  |  | Characters | O | O | Number of lines and characters per line, e.g. 3x20, 4x15 etc. Example coding: MChars: 3x20 |
| MClrs |  | Colour/monochrome | Number of colours | O | O | The number of colours supported, e.g. . 1, 256, 16,000 etc. Example coding: MClrs: 256 |
| MGrey |  |  | Greyscale | O | O | The number of gray scale, e.g. 2 for black and white only Example coding: MGrey: nnn |
| MVoice | Input mode | Voice |  | O | O | Indication of whether voice capture is supported, e.g. yes/no Example coding: MVoice: yes |
| MKbdId |  | Keyboard type | Keyboard identity | O | O | Type of keyboard supported, e.g. default, PC-AT-102 Example coding: MKbdId: default |
| MKbdLg |  |  | Human language | O | O | ISO 639 encoded language used on keyboard, i.e. EN-UK etc. Example coding: MKbdLg: EN-UK |
| MKeys |  |  | Number of keys | O | O | The number of keys supported by the keyboard, i.e. default, 15, 102, etc. Example coding: MKeys:default |
| MPntr |  | Pointing device | Identity | O | O | Type of pointing device, i.e. none, touchscreen, mouse, etc. Example coding: MPntr: none |
| MHLang | Human Language |  |  | O | O | ISO 639 encoded preferred language in which the user wishes to work, i.e EN-UK etc. Example coding: MHLang: EN-UK |
| MFLang | Formal Language |  |  | O | O | Prioritised list of renditions for formal languages, e.g. HTML, WML, etc. Example coding: MFLang: WML/HTML |
| MAdPrt | Additional Protocols |  |  | O | O | List of additionally supported protocols of the MExE device, e.g. SAT (SIM Application Toolkit) Example coding: MAdPrt: SAT |
| MBear | Supported Bearers |  |  | O | O | Prioritised list of preferred bearers, e.g. SMS, USSD, GPRS, etc. Example coding: MBear: GPRS/SMS |
| MClDif | Difference to base Classmark |  |  | O | O | Identification of the devices’ components differing from the base Classmark definition Example coding: MClDif: Java.rmi |
| MMemTy | User authorised available memory | Type of memory |  | O | O | Type of memory, e.g. RAM, memory, hard disk, etc. Example coding: MMemTy: RAM |
| MMem |  | Amount of memory |  | O | O | Amount of memory authorised by the user for MExE, e.g. 1, 6, 10, etc. Example coding: MMem: 1 |
| MAudio | Multimedia | Audio codec type |  | O | O | Prioritised list of preferred audio codecs (cf. H.323) e.g. G.711, G.723 etc. Example coding: MAudio: G.711 |
| MVideo |  | Video codec type |  | O | O | Prioritised list of preferred video codecs (cf. H.323), e.g. H.261, H.263 Example coding: MVideo: H.261 |
| MTSize | Transfer limits | Size |  | O | O | Maximum size of transfers in Kb, e.g. 1,2, 4, 8, etc. Example coding: MTSize: 4 |
| MTTime |  | Time |  | O | O | Maximum duration of transfers in minutes, eg. 1,2, 4, 8, etc. Example coding: MTTime: 2 |
| MLoc | Location Information | Support of location information |  | O | O | Type of location informationsupported, if any, e.g. No, GPRS, cell, other E-OTD, TOA, etc. Example coding: MLoc: E-OTD |
| MLocIn |  | Location information |  | O | O | Actual location information (as defined by type) Example coding: MLocIn: xxxxxxxxxxxxxxx |
| MUProfile | User profile |  |  | O | O | Transfer of the User Profile; individual characteristic(s) of the profile Example coding: MUProf: nnnnnnnn |
| MURL |  |  |  | O | O | An indirect reference to a location where the MExE MS’s characteristics may be obtained Example coding: MURL: http:://name.co.com/profile |
| MAddCh | Additional characteristics |  |  | O | O | Support of additional characteristics, e.g. none, smart card reader, docking station, etc. Example coding: MAddCh: docking-station |
| MAddIn | Additional Information |  |  | O | O | Manufacturer defined additional characteristics; free format Example coding: MAddIn: XYZ |

Table 1: Capability negotiation characteristics