ETSI SMG4	Tdoc SMG4 4M99-065

Helsinki

17-19.2.1999

| CHANGE REQUEST No : |  |  |  |  |  |  | . |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 03.57 |  | Version |  | 1.4.0 |  |  |  |
| Submitted to SMG |  |  | for approval |  | x | without presentation ("non-strategic") |  |  |  |  |  |
|  |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | x | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: |  |
| --- | --- |

| Source: | IBM | Date: | 17.2.1999 |
| --- | --- | --- | --- |

| Subject: | Smartcard API |
| --- | --- |

| Category: | F Correction | x | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 | x |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | Reference to OCF 1.0. to be replaced by OCF 1.1. OCF to be added to list of Normative References and Abbreviations |
| --- | --- |

| Clauses affected: | 2.1., 3.2., 6.2.2. |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  | ® List of CRs: |  |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: |  |
| --- | --- |

## 2.1	Normative references

[1]	GSM 01.04 (ETR 350): “Digital cellular telecommunications system (Phase 2+); Abbreviations and acronyms”.

[2]	GSM 02.57: "Digital cellular telecommunications system (Phase 2+);MExE Stage 1 Description"

[3]	Personal Java, this specification is available from Sun Microsystems’s website at http://java.sun.com/products/personaljava/index.html.

[4]	MNCRS WG 4 specifications are still under development. When completed, MNCRS WG 4 specifications will be available at http://www.mncrs.org/.

[5]	JTAPI 1.2, this specification, currently in the ECTF approval process, is available from Sun Microsystems’s website at http://www.java.sun.com.

[6]	Wireless Application Protocol (WAP) version 1.0, this specification is available from WAP‘s website at http://www.wapforum.org.

[7]	vCard – The Electronic Business Card Exchange Format – Version 2.1, The Internet Mail Consortium (IMC), September 1996, http://www.imc.org/pdi/vcard-21.doc.

[8]	vCalendar – The Electronic Calendaring and Scheduling Exchange Format – Version 1.0, The Internet Mail Consortium (IMC), September 1996, http://www.imc.org/pdi/

[9]	Hypertext Transfer Protocol – HTTP/1.1, IETF document RFC2068, http://www.w3.org/Protocols/rfc2068/rfc2068

[10]	Java Mail API version 1.0.2, available from http://www. java.sun.com

[11]	UMTS TR 22.70: “Universal Mobile Telecommunications System (UMTS); Service aspects; Provision of Services in UMTS - The Virtual Home Environment”.

[12]	UMTS TS 22.xx: “Universal Mobile Telecommunications System (UMTS); Provision of Services in UMTS - The Virtual Home Environment: Stage 1”.

[13]	ISO 639 International Standard - codes for the representation of language names

[14]	OCF 1.1.: “Smartcard API specified by OpenCard Consortium (http://www.opencard.org)

## 3.2	Abbreviations

For the purposes of this TS the following abbreviations apply:

API	Application Programming Interface

CGI	Common Gateway Interface

DHCP	Dynamic Host Configuration Protocol

GSM	Global System for Mobile Communication

HTTP	HyperText Transfer Protocol

IP	Internet Protocol

JNDI	Java Naming Directory Interface

JTAPI	Java Telephony Application Programming Interface

JAR file	Java Archive File

MSE	MExE Service Environment

OCF	OpenCard Framework

RFC	Request For Comments

SAP	Service Access Point

SMS	Short Message Service

TLS	Transport Layer Security

UDP	User Datagram Protocol

URL	Uniform Resource Locator

USSD	Unstructured Supplementary Service Data

WAE	Wireless Application Environment

WAP	Wireless Application Protocol

WSP	Wireless Session Protocol [WSP]

WTA	Wireless Telephony Applications

WTAI	Wireless Telephony Applications Interface

WTLS	Wireless Transport Layer Security

WTP	Wireless Transaction Protocol

WWW	World Wide Web

Further abbreviations are given in GSM 02.57 (MExE stage 2) and GSM 01.04 [1].

### 6.2.2	Required and optional APIs

MEXE devices shall provide APIs specified in http://java.sun.com/products/personaljava/spec-1-1/pJavaSpec.html – PersonalJava 1.1.

The PersonalJava 1.1 specification identifies some JDK 1.1 APIs as optional. MExE-compliant devices shall provide these “PersonalJava 1.1-optional” JDK 1.1 APIs:

Editor’s note: Investigate possibility of using Java check to establish if API list is complete.

- Java.math – this API provides arbitrary-precision integer arithmetic, useful for security key calculation

- Java.security, java.security.interfaces – this API is required to allow applications to discover operations which are disallowed on a particular MEXE device

Editor’s note: MNCRS is investigating use of JDK 1.2 security with PersonalJava 1.1. A final MExE recommendation should take this into account (“for further study”) – an alternative would be to leave the realisation of the security manager implementation dependent in the first release of MExE.

In addition to the PersonalJava 1.1 APIs listed above, MExE devices shall provide the following APIs:

- Telephony API (JTAPI) refer to section 6.2.3

- Optionally: Messaging API – Java Mail API version 1.0.2 [10], available from http://www. java.sun.com

- Optionally: Directory API – Java Naming and Directory Interface (JNDI) version 1.1, available from http://java.sun.com/products/jndi/

Editor’s note: need to detemine which JNDI interface to support, possibility of using Personal JNDI if this avaiable

- SmartCard API – OpenCard  1.1., available from http://www.opencard.org/index-docs.html.  If the device has a smartcard reader (other than the SIM), then the Smartcard API shall be used to access the card.

Editor's note: SIM card APIs need investigating.

The following APIs are for further study (in several cases, they are still under development):

- Adaptability API

- Data Link Control API

- Power Management API

Other APIs are under development by relevant Java groups.

It must be noted that the Classmark 2 device may or may not have a pointing device. This is a challenge for the AWT implementation: it may need to emulate a pointing device, but must not alter the AWT API, and the emulation must be completely transparent to application programs.