| TSG-T2 / SMG4 MExE 26th-28th May 1999 Böblingen, Germany | Tdoc 99-050 |
| --- | --- |

| CHANGE REQUEST No : |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 03.57 |  | Version |  | 1.7.1 |  |  |  |
| Submitted to SMG |  |  | for approval |  |  | without presentation ("non-strategic") |  |  |  |  |  |
| list plenary meeting or STC here  |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM | x | ME | x | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: |  |
| --- | --- |

| Source: | Motorola | Date: |  |
| --- | --- | --- | --- |

| Subject: | A Provisioned mechanism for designating administrative responsibilities of the MExE device. |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| And one release | C Functional modification of feature | X |  | Release 98 | x |
| Only shall be | D Editorial modification |  |  | Release 99 | x |
| Marked with an X) |  |  |  | UMTS |  |

| Reason for change: | A mechanism to provide administrative privileges to an entity that is designated by the owner of the device is required to complete Stage 2 of MExE. The mechanism should work for all possible scenarios, namely 1) Change of SIM 2) Roaming across networks 3) Power-Up |
| --- | --- |

| Clauses affected: | New Clauses |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| Affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how t

# X	A Provisioned Mechanism for Designating Administrative Responsibilities and adding Trusted Third Parties for the MExE Device.

## X.1 Definitions

The following definitions will help clarify the terminologies used in the provisioned mechanism.

a.) User : The User is defined as the user of the MExE Device.

b.) Owner: The owner of the MExE Device could be the User, the operator, the service provider, and/or a third party.

c. ) Administrator : The administrator of the device is the entity who has the control of the third party trusted domain, and all resources associated with the domain. The administrator of the device could be the user, the operator, the service provider, or the third party as designated by the owner of the device.

d. ) LeSIM: LeSIM is defined as the legacy GSM SIM.

e. )  MExE-SIM :  MExE-SIM is as defined as a SIM which is not the legacy SIM, i.e., one from which the ME can get a certificate using a standardised mechanism.

f. ) Power-up Event : The Power-up event is an abstract event that is caused when the MExE device is cold started.

X.2 The Problem Statement

All applications in the TTP Domain are to be signed by a key which can be verified back to a trusted root  TTP certificate on the MExE device. The trusted root certificates can be managed (addition/deletion/mark trusted/mark untrusted/change fine grained access privileges) by an administrator  that is designated by the owner of the device using the MExE Administrator  Provisioning mechanism. A mechanism is required to be provided to enable the owner of the device to dynamically assign an administrator. The mechanism should provide for the following cases:

Case 1: The User is the owner.

Case 2: The owner is at a remote location. In this case the owner could be the operator, a service provider or a third party.

Case 3: The owner of the MExE-SIM wants to be a temporary administrator.

## X.3 Determining the Administrator of the Device.

The administrator of the device shall be determined by the logical mechanism shown  in the flow-chart in figure 1. During Power-Up, the provisioned mechanism  shall look for an administrator root certificate that is stored on the ME. If the certificate is absent , then the user shall automatically be the administrator of the device. If an administrator root certificate is present, this certificate shall be used for all remote administration authentication, implying that the owner of the administrator certificate is the administrator. The rest of the mechanism is defined here, however it is a future release implementation.  The  Enhanced Administrator Update Mechanism  is initiated after a power-up event is processed or when a MExE-SIM is detected.

(The text below assumes that TTP certificates  can be added using the SIM, however TTP certificates could  be added using a non-SIM approach.)

If the administrator is the user, then check to see if there is a  SIM. If a  SIM is present, then check to see if there is a certificate in the SIM ). The format that will be defined  will allow the MExE device to determine whether the owner of the certificate wants to do one of the following, namely:

(1)Certificate present in the SIM wishes to be a Trusted Third Party certificate. (CP-TTP)

(2)Certificate present in the SIM wishes to be a temporary Administrator certificate. (CP-Admin)

(3)Certificate not present in the SIM. (CNP)

If a certificate is present in the SIM that wishes to be a temporary administrator, the user is queried to allow the certificate on the  SIM to take temporary control of the trusted third party domain. By temporary control, it is meant that once the card is removed the administrator reverts back to the user administrator settings. The above mechanism implies that the configuration settings for the administrator at power-up needs to be saved, so that it can be restored. If the user disallows the  SIM certificate, THE TTP Domain CANNOT USE ANY OF THE NETWORK ABILITIES  IN THE TRUSTED THIRD PARTY DOMAIN as identified in the network access section of the security table

If a certificate is present in the SIM and wishes to be a TTP, the user is queried to allow/disallow the certificate as a Trusted Third Party. There are no affects to the system by disallowing the certificate.

If a certificate is not present on the  SIM and the administrator is the user, the user continues to be the administrator and can make use of all functionality.

If the administrator is not the user, then check to see if there is a  SIM. If a  SIM is present, then check to see if there is a certificate in the SIM. If a certificate is present in the SIM, then check the certificate on the  SIM with the certificate on the ME for the following cases:-

- they are the same

- they are not the same, but the ME certificate is cross-certified with the  SIM certificate (a cross-certificate exists on the ME)

- they are not the same, but the ME certificate has a line of trust back to the  SIM certificate domain

- they are not the same.

- If the certificate wishes to be a temporary administrator (CP-Admin) , then in the case of (a), (b) and (c), the temporary administrator is the owner of the CP-Admin certificate. In the case of (d),  THE TTP Domain  CANNOT USE ANY OF THE NETWORK ABILITIES IN THE TRUSTED THIRD PARTY DOMAIN as identified in the network access section of the security table. If the certificate just wants to be a TTP, then the certificate (CP-TTP) is verified with the CCM and based on the content and permissions of the CCM, the certificate is added to the TTP list or rejected.

Is Administrator  Certificate present in ME ????

Power –Up Event

User is the Administrator

No

Yes

MExE-98

Administrator certificate owner is the administrator

IF  SIM INTERFACE EXISTS ON DEVICE

After  Power-up routine, and/or   SIM-Insert Event

Enhanced Administator MechanismEnhancement

Is the user the administrator ???

Yes

No

Is Certificate present on  SIM

Is Certificate present on  SIM

No

Yes

No

CP-TTP

User is the Administrator

CP-TTP

CP-Admin

Current  Administrator is retained

CP-Admin

Query user and add CP-TTP to TTP if agreed

Query user

Does user wish to have temporary Admin for CP-Admin

User is the Administrator BUT TTP Domain LOSES NETWORK ACCESS

CP-Admin certificate owner is the TEMPORARY ADMINISTRATOR

Yes

No

Check with CCM to add/disallow to TTP List

Check with Administrator

Certificate

Cases (a), (b), (c)

Case (d)

Current Administrator retained BUT  TTP Domain  LOSES NETWORK ACCESS

## X.4 Authorized CCM download mechansims

The download of trusted third party certificate lists by a remote administrator is done by using a secure mechanism as defined below. The download mechanisms shall use  HTTP over IP and/or the WAP Protocol. The URL from which the CCM is downloaded is either stored with the administrator certificate or in the administrator certificate. The format for storing the URL information with the certificate is

Urltype

CharacterSet

UrlLength

URL

Figure 2 CCM Message URL storage format

Urltype= one byte,  enumerated { WAP (0), HTTP (1) }.

CharacterSet = one byte,  Internet Assigned Numbers Authority assigned character set.

UrlLength = one byte unsigned integer, length of the URL in octets.

The format for storing the URL information in the certificate will be  defined as part of the enhanced administrator mechanism.

The download is initiated by (1) Change in Administrator (2) a provisioned mechanism for requesting   a CCM. All downloads will be signed by a certificate in the administrator domain with administrator privileges, and hence are securely authenticated.

## X.5 Mandatory Provisioning in a MExE Device.

The MExE device shall support the administrator designation mechanism and the secure downloading of CCMS explained in the previous sections

The system shall support a mechanism (as part of a provisioned application and/or inherently part of the MExE implementation) allowing the owner of the device to manage the Administrator certificate (including the download of a new administrator certificate) as defined in Section 8.8.2. This mechanism shall be secure so that only the owner can use this functionality.