SWG6 ad hoc meeting					TSG T2C(00)005

21February, 2000

Reading, UK

Title: Proposed changes to TR 21.904 to incorporate support of GSM System Features

Source: Samsung Electronics Research Institute

Document for: Decision

_____________________________________________________________________

This document proposes text on support of GSM System Features, to be included in the references and section 7.1.1.5 of TR 21.904. Note that no additions are proposed to the annexes.

[39]	3G TS 22.042: “Network and Identity Timezone (NITZ); Service description, Stage 1”

[40]	3G TS 22.090: “Unstructured Supplementary Service Data (USSD) – Stage 1”

[41]	3G TS 24.080: “Mobile Radio Interface Layer 3 supplementary services specification; Formats and Coding”

#### 7.1.1.5	GSM System Features

##### Network Identities Time Zone (NITZ)

The feature Network Identities and Timezone (NITZ) is defined in [39]. NITZ enables a serving PLMN to transfer its current identity, universal time, DST and LTZ to UE, so that the UE can store and use this information if required. Support of NITZ is optional in both UE and Network. There are no specific UE implementation capabilities required to support the service other than those necessary for the baseline functionality.

##### USSD

Two modes of USSD (MMI-mode and application mode) are described in [40]. Information relating to the formats and coding of supplementary services for layer 3 can be found in [41].