| 3GPP TSG-T2 SWG3#09 MMS ad hoc Paris, France 14-18 January, 2002 |  |  |  |  |  |  |  |  | T2M020064 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR-Form-v5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23.140 |  |  |  | CR | CRNum |  |  |  rev |  |  |  | - |  Current version: |  |  |  |  | 5.1.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  |  |  | Radio Access Network |  |  |  |  |  |  |  | Core Network |  | X |
| Title:  |  | MM7 and Delivery report functionality |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Nokia |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  | MESS5-MMS |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | January 17th, 2002 |  |  |  |  |  |
| Category:  |  | C |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-5 |  |  |  |  |  |
|  |  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | MM7 reference point requires modifications to Delivery report functionality |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Described the behaviour when VASP has requested Delivery report to be generated and re-structuring the Delivery report section |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | MM7 does not support Delivery report functionality |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 3.1, 3.2, 7.1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at: http://www.3gpp.org/3G_Specs/CRs.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://ftp.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2001-03 contains the specifications resulting from the March 2001 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

# 3	Definitions and Abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions defined in 3GPP TR 21.905 [2] and 3GPP TS 22.140 [1] and the following apply:

Abstract message: information which is transferred between two MMS entities used to convey an MM and/or associated control information between these two entities

NOTE 1:	The application protocol framework and technical realisation of MMS service features is described in terms of abstract messages in the present document.

Delivery Report: feedback information provided to an originator of MM (MMS User Agent or VASP) by an MMS Relay/Server about the status of the delivery of an MM

External Server: network entity/application of an external system such as Internet email, unified messaging system or facsimile to which MMs may be sent to and/or from which MMs may be received by an MMS User Agent via an MMS service provider

NOTE 2:	An External Server is connected to that MMS Service Provider via non-MMS-specific protocols.

Forwarding MMS User Agent: MMS User Agent that is the intended recipient of an MM, that requests forwarding of the MM for delivery to other recipient(s) without having to first download the MM

Forwarded MM: MM originally sent from a sender to an intended recipient which is then forwarded to other recipient(s) and to which a delivery report and/or read-reply report may refer and which may be subject to further forwarding

MM Delivery: act of a recipient MMS Relay/Server delivering an MM to a recipient MMS User Agent

MM Submission: act of an originator MMS User Agent submitting an MM to the originator MMS Relay/Server

MMSNA: Multimedia Messaging Service Network Architecture encompasses all the various elements that provide a complete MMS to a user

MMSE: collection of MMS-specific network elements under the control of a single administration

MMS Relay/Server: MMS-specific network entity/application that is under the control of an MMS service provider

NOTE 3:	An MMS Relay/Server transfers messages, provides operations of the MMS that are specific to or required by the mobile environment and provides (temporary and/or persistent) storage services to the MMS.

MMS User Agent: application residing on a UE, an MS or an external device that performs MMS-specific operations on a user's behalf

NOTE 4:	An MMS User Agent is not considered part of an MMSE.

MMS VAS Applications: Applications providing Value Added Services (e.g. news service or weather forecasts) to MMS users.

Original MM: (initial) MM sent from a sender to a recipient and to which a delivery report and/or a read-reply report and/or a reply-MM may refer and/or which may be subject to being forwarded

Originator MMSE: MMSE associated with the sender of an MM

Originator MMS Relay/Server: MMS Relay/Server associated with the sender of an MM

Originator MMS User Agent: MMS User Agent associated with the sender of an MM

Read-Reply Report: feedback information to an originator MMS User Agent by a recipient MMS User Agent about the status of handling/rendering of an original MM in a recipient MMS User Agent

Recipient MMSE: MMSE associated with the recipient of an MM

Recipient MMS Relay/Server: MMS Relay/Server associated with the recipient of an MM

Recipient MMS User Agent: MMS User Agent associated with the recipient of an MM

Reply-MM: the first reply accepted by the recipient MMS Relay/Server (after checking the reply charging limitations, such as the latest time of submission)in case of reply-charging

Transaction: message pair sent between an MMS User Agent and MMS Relay/Server, or between MMS Relay/Servers

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations defined in [1] and [2] and the following apply:

CDR	Call Data Record

DNS	Domain Name System

EMA	Electronic Message Association

E-Mail	Electronic Mail

ENUM	Electronic Numbering

FQDN	Fully Qualified Domain Name

GW	Gateway

HTTP	Hypertext Transfer Protocol

IANA	Internet Assigned Numbering Authority

IETF	Internet Engineering Task Force

IMAP4	Internet Message Access Protocol

MIME	Multipurpose Internet Mail Extensions

MM	Multimedia Message

MMS	Multimedia Messaging Service

MMSE	Multimedia Messaging Service Environment

MMSNA	Multimedia Messaging Service Network Architecture

MTA	Mail Transfer Agent

PDU	Protocol Data Unit

POP3	Post Office Protocol Version 3

RADIUS	Remote Authentication Dial In User Service

RDF	Resource Description Format

RFC	Request for Comments

SMIL	Synchronised Multimedia Integration Language

SMTP	Simple Mail Transfer Protocol

UA	User Agent

UAProf	User Agent Profile

URI	Uniform Resource Identifiers

VAS	Value Added Service

VASP	Value Added Service Provider

VPIM	Voice Profile for Internet Mail

W3C	WWW Consortium

WAP	Wireless Application Protocol

WIM	WAP Identity Module

WML	Wireless Markup Language

WSP	WAP Session Protocol

WTLS	Wireless Transport Layer Security

…

7.1.5	Delivery Report

The MMS Relay/Server shall support the delivery reporting service. Delivery reports shall only be generated for MMs.

Both MMS User Agent and VASP may be able to request a delivery report for a specific MM. However the behavior of Delivery Report functionality differs depending of the entity requesting delivery report.

7.1.5.1	MMS User Agent originated delivery report request

Within an MM notification or upon MM retrieval the recipient MMS User Agent may receive an indication that a delivery report is requested for the MM.

Within either a response to a notification or a response to an MM’s delivery, the recipient MMS User Agent may request a delivery report not to be generated by the MMS Relay/Server.

The originator MMS Relay/Server shall generate a delivery report if a delivery report has been requested by the originator MMS User Agent

- upon routing forward the MM, in case the peer entity is not known by the MMS Relay/Server

The recipient MMS Relay/Server shall generate a delivery report if a delivery report has been requested by the originator MMS User Agent and if the recipient MMS User Agent did not request a delivery report not to be generated

- upon receipt of a response to a notification, in case the MM is rejected by the recipient MMS User Agent

- upon receipt of a forwarding request, in case the MM is forwarded  by the recipient MMS User Agent to other MM recipient(s), without prior retrieval.

- upon receipt of a response to an MM’s delivery, in case the MM is retrieved by the MM recipient

- upon expiry of the MM, in case the MM is not rejected and not retrieved by the MM recipient before the expiry

The originator MMS User Agent, i.e. the MMS User Agent receiving the delivery report, may match the delivery report to the sent MM by retaining the message identification of the sent MM and comparing it to the received delivery report, which shall contain the message identification of the original MM. In case of multiple MM recipients, it is necessary for the originator MMS User Agent to retain the MM recipient addresses as well, to match the delivery report to the sent MM.

If a delivery report has been requested by the originator MMS User Agent and if the recipient MMS User Agent did not request a delivery report not to be generated, the recipient MMS Relay/Server

- shall generate the delivery report

- shall deliver the delivery report to the originator MMS Relay/Server.

- shall be responsible for the storage of delivery reports in the network until the originator MMS Relay/Server becomes reachable or until the delivery report expires

Within the delivery report the recipient MMS Relay/Server

- shall provide the MM originator address to the originator MMS Relay/Server.

- shall provide the MM recipient address to the originator MMS Relay/Server.

- shall provide the identification of the original MM for which the delivery report has been generated to the originator MMS Relay/Server.

- shall provide status information how the MM was handled (e.g. expired, rejected, delivered, forwarded or indeterminate) to the originator MMS Relay/Server

- shall provide a time stamp when the MM was handled to the originator MMS Relay/Server

For each MM recipient of the original MM for which the delivery report has been generated and becomes available at the originator MMS Relay/Server, the originator MMS Relay/Server

- shall deliver the delivery report to the originator MMS User Agent (i.e. the recipient MMS User Agent of the delivery report).

Within the delivery report the originator MMS Relay/Server

- shall provide the MM recipient’s address to the originator MMS User Agent (the recipient MMS User Agent of the delivery report).

- shall provide the identification of the original MM for which the delivery report has been generated to the originator MMS User Agent (the recipient MMS User Agent of the delivery report).

- shall be responsible for the storage of delivery reports in the network until the originator MMS User Agent becomes reachable (e.g. user moves back into coverage, switches MMS User Agent on) or until the delivery report expires

7.1.5.2	VASP originated delivery report request

The originator MMS Relay/Server shall generate a delivery report if a delivery report has been requested by the VASP

- upon routing forward the MM, in case the peer entity is not known by the MMS Relay/Server

If a delivery report has been requested by the VASP, the recipient MMS Relay/Server

- shall generate the delivery report

- shall deliver the delivery report to the VASP.

- shall provide the MM recipient’s address to the VASP.

- shall provide the identification of the original MM for which the delivery report has been generated to the VASP.

- shall provide status information how the MM was handled (e.g. expired, rejected, delivered, forwarded or indeterminate) to the originator MMS Relay/Server

- shall be responsible for the storage of delivery reports in the network until the VASP becomes reachable or until the delivery report expires

- shall provide a time stamp when the MM was handled to the originator MMS Relay/Server

For each MM recipient of the original MM for which the delivery report has been generated and becomes available at the originator MMS Relay/Server, the originator MMS Relay/Server

- shall deliver the delivery report to the VASP.

In a case of the recipient MMS User agent request a forwarding of the VASP originated MM, the forwarding MMS Relay/Server

- Shall generate a delivery report indicating “indeterminate” status of the MM’s delivery if a delivery report was requested by the VASP.

The VASP may match the delivery report to the sent MM by retaining the message identification of the sent MM and comparing it to the received delivery report, which shall contain the message identification of the original MM. In case of multiple MM recipients, it is necessary for the VASP to retain the MM recipient addresses as well, to match the delivery report to the sent MM.

Note: 	If the VASP has requested delivery report, MMS Relay/Server shall deliver it every time although the recipient MMS User Agent has denied creating delivery reports.

Note:	In the case of the recipient of the MM is outside of the MMSE delivery report behaves as normal delivery report.

Note:	In case of MM is transferred to another MMS Relay/Server the recipient MMS User Agent may request a delivery report not to be generated. In that case VASP does not receive any delivery report