| 3GPP TSG-T2 SWG3#9 Paris, France 14th – 18th January 2002 |  |  |  |  |  |  |  |  | T2M020067 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR-Form-v3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23.140 |  |  |  | CR | CR-Num |  |  |  rev |  |  |  | - |  |  Current version: |  |  |  |  | 5.1.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  | X |  | Radio Access Network |  |  |  |  |  |  |  |  | Core Network |  |  |
| Title:  |  | Clarification of existing request status codes over MM4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Ericsson |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  | MESS5-MMS |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 020117 |  |  |  |  |  |
| Category:  |  | B |  |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-5 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | The MM4 interface needs to be of carrier grade robustness. One component to accomplish this is to have solid error handling. In the current specification the only means of application level error handling are the request status codes, unfortunately their meaning is unclear in some cases. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | - Table added to clarify the interpretation of request status codes. - Meaningless request status code made obsolete |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | More unpredictable application level error handling over MM4. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 8.4.4.8, new section 8.4.4.10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at: http://www.3gpp.org/3G_Specs/CRs.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://www.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2000-09 contains the specifications resulting from the September 2000 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request

      - Header Field Value Range

MMS information elements that are mapped to standard STD 11 "header fields", i.e. which do not have an "X-MMS-" prefix, should be used according to [5].

The rest of the header definitions used in this clause, including the mechanisms and pre-defined tokens, are described in an augmented Backus-Naur Form (BNF) defined in [48], similar to that used by RFC 822 [5]. Implementors will need to be familiar with the notation in order to understand these definitions.

For the residual MMS information elements the following applies:

X-Mms-3GPP-MMS-Version:

3GPP-MMS-Version = "X-Mms-3GPP-MMS-Version" ":"  1*DIGIT "." 1*DIGIT "." 1*DIGIT

Note that the numbers MUST be treated as separate integers and that each may be incremented higher than a single digit. Thus, 2.1.4 is a lower version than 2.1.13, which in turn is lower than 2.3.0 Leading zeros shall be ignored by recipient MMS Relay/Server and shall NOT be sent. The version is according to the version of the present document (see also clause "Foreword").

X-Mms-Message-Type:

Message-type = "X-Mms-Message-Type" ":" ( "MM4_forward.REQ" | "MM4_forward.RES" | "MM4_delivery_report.REQ" | "MM4_delivery_report.RES" | "MM4_read_reply_report.REQ" | "MM4_read_reply_report.RES" )

X-Mms-Transaction-Id:

Transaction-id =  "X-Mms-Transaction-ID" ":" quoted-string

X-Mms-Message-Id:

Message-id =  "X-Mms-Message-ID" ":" quoted-string

X-Mms-Message-Class:

Message-class = "X-Mms-Message-Class" ":" ( Class-identifier | quoted-string )

Class-identifier = "Personal" | "Advertisement" | "Informational" | "Auto"

X-Mms-Expiry:

Expiry-value = "X-Mms-Expiry" ":" ( HTTP-date | delta-seconds )

X-Mms-Delivery-Report:

Delivery-report =  "X-Mms-Delivery-Report" ":" ( "Yes" | "No" )

X-Mms-Priority:

Priority = "X-Mms-Priority" ":" ( "Low" | "Normal" | "High" )

X-Mms-Sender-Visibility:

Sender-visibility = "X-Mms-Sender-Visibility" ":" ( "Hide" | "Show" )

X-Mms-Read-Reply:

Read-reply = "X-Mms-Read-Reply" ":" ( "Yes" | "No" )

X-Mms-Ack-Request:

Ack-Request = "X-Mms-Ack-Request" ":" ( "Yes" | "No" )

X-Mms-Request-Status-Code:

Request-status-Code =  "X-Mms-Request-Status-Code" ":" ( "Ok" | "Error-unspecified" | "Error-service-denied" | "Error-message-format-corrupt" | "Error-sending-address-unresolved" | "Error-message-not-found" | "Error-network-problem" | 	"Error-content-not-accepted" | "Error-unsupported-message" )

The meaning of the X-Mms-Request-Status-Code header field is further described in section 8.4.4.10 of this specification.

X-Mms-MM-Status-Code:

MM-Status-Code = "X-Mms-MM-Status-Code" ":" ( "Expired" | "Retrieved" | "Rejected" | "Deferred" | "Intermediate" | "Forwarded" | "Unrecognised" )

      - Request Status Codes Clarification

The table below dictates how the originator MMS Relay/Server should interpret the possible values of the X-Mms-Request-Status-Code header field.

Table XX: Clarification of the Request Status Codes

| X-Mms-Request-Status-Code | Meaning |
| --- | --- |
| Ok | The corresponding request and some or all of its contents were accepted without errors. |
| Error-unspecified | An unspecified error occurred during the processing or reception of the corresponding request. |
| Error-service-denied | The corresponding request was rejected due to failure of authentication or authorisation of the originating MMS Relay/Server. |
| Error-message-format-corrupt | An inconsistency with the message format was detected when the corresponding request was parsed. |
| Error-sending-address-unresolved | There were no MMS address (From:, To:, Cc:) in its proper format or none of the addresses belong to the recipient MMS Relay/Server. |
| Error-message-not-found | This status code is obsolete |
| Error-network-problem | The recipient MMS Relay/Server was not able to accept the corresponding request due to capacity overload. |
| Error-content-not-accepted | The MM content was not accepted due to size, media type, copyrights or some other reason. |
| Error-unsupported-message | The recipient MMS Relay/Server does not support the corresponding request abstract message. |