| 3GPP TSG-T2 SWG3#9 ad hoc Paris, France 26-30 January 2002 |  |  |  |  |  |  |  |  | T2M020097 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR-Form-v5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23.140 |  |  |  | CR | CRNum |  |  |  rev |  |  |  | - |  Current version: |  |  |  |  | 5.1.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  | X |  | Radio Access Network |  |  |  |  |  |  |  | Core Network |  |  |
| Title:  |  | Reject Duplicates |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Logica |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  | MESS5-MMS |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 22 January 2002 |  |  |  |  |  |
| Category:  |  | F |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-5 |  |  |  |  |  |
|  |  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | There is currently no standard method of detecting MMs that have been previously successfully submitted that are re-submitted by an MMS User Agent. The MMS User Agent may re-submit an MM because it falsely believes the original submission failed because it did not receive a positive acknowledgement from the MMS Relay/Server. (The reason it may not receive the acknowledgment may be because of delays in the network or temporary out of coverage etc.). Since MMs may be billed per submission an originator MMS User Agent may be billed twice (or more) for the submission of a single MM. In addition, since the MMs may also be billed on retrieval a recipient MMS User Agent may also be billed twice (or more) for the multiple copies of the same MM. This CR provides the functionality for stage 3 implementations to solve this problem. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Addition of the service offered to include the detection of duplicate MMs on the MM1 interface. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | Stage 3 definitions of the MM1 reference point may not implement functionality to detect duplicate MMs. Therefore the MMSE may not be capable of detecting duplicate MMs and originators and recipients may be billed more than once for the same MM. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 7.1.1, 8.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at: http://www.3gpp.org/3G_Specs/CRs.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://ftp.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2001-03 contains the specifications resulting from the March 2001 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

# 7	MMS Service Behaviour Description

## 7.1	MMS services offered

### 7.1.1	Submission of a Multimedia Message in the originator MMSE

When a user intends to send an MM to one or several destinations the MM shall be submitted to the originator MMS Relay/Server.

The support for submission of MMs is optional for MMS User Agents. The support for submission of MMs is mandatory for MMS Relay/Servers.

If an MMS User Agent supports submission of MMs the MMS User Agent shall be able to:

- Indicate the address of the MM recipient

- Identify the MIME content type of the message.

If a MMS User Agent supports submission of MMs the MMS User Agent may be able to:

- Request a delivery report for the message

- Request a read-reply report for the message

- Provide a time stamp for the time of submission of the message

- Set the earliest desired time of delivery for the message

- Set the desired time of expiry for the message

- Indicate the address of the MM originator

- Set further message qualifications (e.g. priority, message class, subject)

- Request the MM originator’s address being hidden from the recipient MMS User Agent.

- Upon reception of an MM from an originator MMS User Agent the originator  MMS Relay/Server

- shall assign a Message Identification to the MM and immediately provide the originator MMS User Agent with this Message Identification

- is responsible for retaining the MM until the earliest desired time of delivery, if the optional feature of earliest time of delivery is supported by the originator  MMS Relay/Server. If this feature is not supported then the MM is immediately routed forward.

- may provide a time stamp, i.e. it may also override the MMS User Agent’s time stamp,

- shall insert the originator’s address into the MM if not yet provided by the originator MMS User Agent

- shall pass the originator’s address to the peer entity if the peer entity is known to be a MMS Relay/Server

- shall route forward the request for address hiding unaltered to the recipient MMS Relay/Server if the peer entity is known to be an MMS Relay/Server .

- shall pass the originator’s address to the peer entity if the peer entity is not known to be an MMS Relay/Server and address hiding has not been requested by the originator MMS User Agent

- shall not pass the originator’s address to the peer entity and should override the address provided by the originator MMS User Agent in the MM to an "anonymous" address if the peer entity is not known to be an MMS Relay/Server and address hiding has been requested by the originator MMS User Agent

- may override the address provided by the originator MMS User Agent in the MM (subject to MMS service provider’s preferences)

- is responsible for resolving the MM recipient’s address(es),

- is responsible to route the MM towards the MM recipients.

- should pass the indication whether or not a delivery report is requested unaltered when routing the MM towards the MM recipient(s)

- shall pass the indication whether or not a read-reply report is requested unaltered when routing the MM towards the MM recipient(s)

- shall pass the indication about MIME content type of the message and message qualifications (e.g. priority, message class, subject) unaltered when routing the MM towards the MM recipient(s)

- shall generate a delivery report indicating “indeterminate” status of the MM’s delivery if a delivery report was requested by the originator MMS User Agent and if the peer entity the MM is routed forward to is not known by the originator MMS Relay/Server.

- may reject the MM submission if the MM is identified as a duplicate of an MM already stored.

A special case is where the recipient MMS Relay/Server is also the originator MMS Relay/Server. In this case the MM does not have to be routed forward.

………………

## 8.1	Technical realisation of MMS on reference point MM1

On the MM1 reference point an underlying authentication mechanism should be available.

The network-provided MMS User Agent’s ID (e.g. MSISDN or IMSI) should be made available to the MMS Relay/Server by the RADIUS mechanisms defined in [54]. This ID should be used to authenticate the MMS User Agent.

### 8.1.x	Detection of Duplicate MMs

On the MM1 reference point an underlying mechanism for detecting the submission of duplicate MMs should be available.

…