| 3GPP T3 Ad hoc #60, joint with T2 (MMS-USIM Aspects) Sophia Antipolis, France 11 January 2002 | T3xxxxxx |
| --- | --- |

| CR-Form-v3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 31.102 |  |  |  | CR | CR-Num |  |  |  rev |  |  |  | - |  |  Current version: |  |  |  |  | 4.2.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  | X | ME/UE |  |  |  | Radio Access Network |  |  |  |  |  |  |  |  | Core Network |  |  |
| Title:  |  | CR 31.102 Rel-5 Introduction of MMS files and procedures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Siemens AG |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 09-01-2002 |  |  |  |  |  |
| Category:  |  | B |  |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-5 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change: |  |  | Unlike SMS, for MMS-related information there is no storage possibilities on the (U)SIM. The MMS service has a couple of features which would benefit from related information being stored on the (U)SIM. Storing MMS parameters enables for automatic terminal configuration. Storing MMS notifications on the USIM allows a subscriber to remove the UICC from her ME and retain the ability to retrieve those messages using another ME. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | This contribution proposes to add the storage of MMS related information in several elementary files on the USIM. In detail this contribution proposes: - Changes in EFUST. - A new file EFMMSN , in which the MMS Notification can be stored in. - A new file EFMMSP , in which the MMS Parameters can be stored in. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | - A user, having been notified of pending MMs, will lose the ability to retrieve those MMs if she swaps MEs before they have been delivered. - Creation of mobile-originated MMs will be cumbersome, requiring more user interaction than otherwise necessary. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 2, 4.2, 4.7, 5.3, Annex A, Annex D, Annex E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## ...

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 21.111: "USIM and IC Card Requirements".

[2]	3GPP TS 22.011: "Service accessibility".

[3]	3GPP TS 22.024: "Description of Charge Advice Information (CAI)".

[4]	3GPP TS 22.030: "Man-Machine Interface (MMI) of the Mobile Station (MS)".

[5]	3GPP TS 23.038: "Alphabets and language".

[6]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS) Point-to-Point (PP)".

[7]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[8]	3GPP TS 22.067: "Enhanced Multi Level Precedence and Pre-emption service (eMLPP) - Stage 1".

[9]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification".

[10]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface".

[11]	3GPP TS 31.101: "UICC-Terminal Interface, Physical and Logical Characteristics".

[12]	3GPP TS 31.111: "USIM Application Toolkit (USAT)".

[13]	3GPP TS 33.102: "3G Security Architecture".

[14]	3GPP TS 33.103: "3G Security; Integration Guidelines".

[15]	3GPP TS 22.086: "Advice of charge (AoC) Supplementary Services - Stage 1".

[16]	3GPP TS 23.041: "Technical realization of Short Message Service Cell Broadcast (SMSCB)".

[17]	3GPP TS  02.07: "Mobile Stations (MS) features".

[18]	3GPP TS 11.11: "Specification of the Subscriber Identity Module – Mobile Equipment (SIM – ME) interface".

[19]	ISO 639 (1988): "Code for the representation of names of languages".

[20]	ISO/IEC 7816-4 (1995): "Identification cards - Integrated circuit(s) cards with contacts, Part 4: Interindustry commands for interchange".

[21]	ISO/IEC 7816-5 (1994): "Identification cards - Integrated circuit(s) cards with contacts, Part 5: Numbering system and registration procedure for application identifiers".

[22]	ITU-T Recommendation E.164: "Numbering plan for the ISDN era".

[23]	ITU-T  Recommendation T.50: "International Alphabet No. 5". (ISO 646 (1983): "Information processing - ISO 7-bits coded characters set for information interchange").

[24]	3GPP TS 22.101: "Service aspects; service principles".

[25]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[26]	ISO/IEC FCD 7816-9 (1999): "Identification cards - Integrated circuit(s) cards with contacts, Part 9: Additional Interindustry commands and security attributes".

[27]	3GPP TS 22.022: "Personalisation of GSM Mobile Equipment (ME); Mobile functionality specification".

[28]	3GPP TS 04.18 "Mobile Interface Layer3 Specification, Radio Resource control protocol"

[29]	3GPP TS 23.022: "Functions related to Mobile Station (MS) in idle mode and group receive mode".

[30]	3GPP TS 23.057: "Mobile Station Application Execution Environment (MExE);Functional description; Stage 2".

[31]	3GPP TS 23.122: "NAS Functions related to Mobile Station (MS) in idle mode"

[32]	ISO/IEC 7816-6 (1996): "Identification cards -- Integrated circuit(s) cards with contacts -- Part 6: Interindustry data elements".

[33]	3GPP TS 25.101: "UE Radio Transmission and Reception (FDD)"

[34]	3GPP TS 23.140: “Multimedia Messaging Service (MMS); Functional description; stage 2”.

[35]	WAP-209-MMSEncapsulation, June 2001. URL: http://www.wapforum.org.

### 4.2.8	EFUST (USIM Service Table)

This EF indicates which services are available. If a service is not indicated as available in the USIM, the ME shall not select this service.

| Identifier: '6F38' |  | Structure: transparent |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- |
| SFI: '04' |  |  |  |  |  |  |
| File size: X bytes, X >= 1 |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE ADM DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | Services n°1 to n°8 |  |  | M |  | 1 byte |
| 2 | Services n°9 to n°16 |  |  | O |  | 1 byte |
| 3 | Services n°17 to n°24 |  |  | O |  | 1 byte |
| 4 | Services n°25 to n°32 |  |  | O |  | 1 byte |
| Etc. |  |  |  |  |  |  |
| X | Services n°(8X-7) to n°(8X) |  |  | O |  | 1 byte |

| -Services |  |  |
| --- | --- | --- |
| Contents: | Service n°1: | Local Phone Book |
|  | Service n°2: | Fixed Dialling Numbers (FDN) |
|  | Service n°3: | Extension 2 |
|  | Service n°4: | Service Dialling Numbers (SDN) |
|  | Service n°5: | Extension3 |
|  | Service n°6: | Barred Dialling Numbers (BDN) |
|  | Service n°7: | Extension4 |
|  | Service n°8: | Outgoing Call Information (OCI and OCT) |
|  | Service n°9: | Incoming Call Information (ICI and ICT) |
|  | Service n°10: | Short Message Storage (SMS) |
|  | Service n°11: | Short Message Status Reports (SMSR) |
|  | Service n°12: | Short Message Service Parameters (SMSP) |
|  | Service n°13: | Advice of Charge (AoC) |
|  | Service n°14: | Capability Configuration Parameters (CCP) |
|  | Service n°15: | Cell Broadcast Message Identifier |
|  | Service n°16: | Cell Broadcast Message Identifier Ranges |
|  | Service n°17: | Group Identifier Level 1 |
|  | Service n°18: | Group Identifier Level 2 |
|  | Service n°19: | Service Provider Name |
|  | Service n°20: | User controlled PLMN selector with Access Technology |
|  | Service n°21: | MSISDN |
|  | Service n°22: | Image (IMG) |
|  | Service n°23: | Not used (reserved for SoLSA) |
|  | Service n°24: | Enhanced Multi-Level Precedence and Pre-emption Service |
|  | Service n°25: | Automatic Answer for eMLPP |
|  | Service n°26: | RFU |
|  | Service n°27: | GSM Access |
|  | Service n°28: | Data download via SMS-PP |
|  | Service n°29: | Data download via SMS-CB |
|  | Service n°30: | Call Control by USIM |
|  | Service n°31: | MO-SMS Control by USIM |
|  | Service n°32: | RUN AT COMMAND command |
|  | Service n°33: | shall be set to '1' |
|  | Service n°34: | Enabled Services Table |
|  | Service n°35: | APN Control List (ACL) |
|  | Service n°36: | Depersonalisation Control Keys |
|  | Service n°37: | Co-operative Network List |
|  | Service n°38: | GSM security context |
|  | Service n°39: | CPBCCH Information |
|  | Service n°40: | Investigation Scan |
|  | Service n°41: | MExE |
|  | Service n°42: | Operator controlled PLMN selector with Access Technology |
|  | Service n°43: | HPLMN selector with Access Technology |
|  | Service n°44: | Extension 5 |
|  | Service n°45: | PLMN Network Name |
|  | Service n°46: | Operator PLMN List |
|  | Service n°47: | Mailbox Dialling Numbers |
|  | Service n°48: | Message Waiting Indication Status |
|  | Service n°49: | Call Forwarding Indication Status |
|  | Service n°50: | RPLMN Last used Access Technology |
|  | Service n°51: | Service Provider Display Information |
|  | Service n°XX | Multimedia Message Service |

The EF shall contain at least one byte. Further bytes may be included, but if the EF includes an optional byte, then it is mandatory for the EF to also contain all bytes before that byte. Other services are possible in the future and will be coded on further bytes in the EF. The coding falls under the responsibility of the 3GPP.

Coding:

1 bit is used to code each service:
bit = 1: service available;
bit = 0: service not available.

-	Service available means that the USIM has the capability to support the service and that the service is available for the user of the USIM unless the service is identified as "disabled" in EFEST.
Service not available means that the service shall not be used by the USIM user, even if the USIM has the capability to support the service.

First byte:

|  |  | b8 |  | b7 |  | b6 |  | B5 |  | b4 |  | b3 |  | b2 |  | b1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°1 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°2 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°3 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°4 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°5 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°6 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°7 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°8 |

Second byte:

|  |  | B8 |  | b7 |  | b6 |  | B5 |  | b4 |  | b3 |  | b2 |  | b1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°9 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°10 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°11 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°12 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°13 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°14 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°15 |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Service n°16 |

...

4.2.XX	EFMMSN (MMS notification)

This EF contains information in accordance with 3G TS 23.140 [34] comprising MMS notifications (and associated parameters) which have been received by the UE from the network.

| Identifier: ’6FXX’ |  | Structure: Linear fixed |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| Record length: 3+X bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE PIN DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | MMS status |  |  | M |  | 1 byte |
| 2 | MMS Implementation |  |  | M |  | 1 byte |
| 3 – X+3 | MMS Notification |  |  | M |  | X bytes |
| X+4 – X+5 | Extension file record number |  |  | M |  | 1 byte |

-  Status

Contents:

The status will be updated when the UE receives an MMS notification.

Coding:

b1 indicates whether there is valid data or if the location is free. b2 indicates whether the message has been read or not. Bits b3-b5 indicates the retrieval status,

| b8 |  | B7 |  | b6 |  | b5 |  | B4 |  | b3 |  | b2 |  | b1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | X |  | X |  | X |  | X |  | X |  | 0 |  | Free space |
|  |  |  |  | X |  | X |  | X |  | X |  | X |  | 1 |  | Used space |
|  |  |  |  | X |  | X |  | X |  | X |  | 0 |  | 1 |  | Notification not read |
|  |  |  |  | X |  | X |  | X |  | X |  | 1 |  | 1 |  | Notification read |
|  |  |  |  | X |  | 0 |  | 0 |  | 0 |  | X |  | 1 |  | MM not retrieved |
|  |  |  |  | X |  | 0 |  | 0 |  | 1 |  | X |  | 1 |  | MM retrieved |
|  |  |  |  | X |  | 0 |  | 1 |  | 0 |  | X |  | 1 |  | MM rejected |
|  |  |  |  | X |  | 0 |  | 1 |  | 1 |  | X |  | 1 |  | MM forwarded |
|  |  |  |  | 0 |  | X |  | X |  | X |  | X |  | 1 |  | Creation of delivery report not allowed |
|  |  |  |  | 1 |  | X |  | X |  | X |  | X |  | 1 |  | Creation of delivery report allowed |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Reserved for future use |

-  MMS Implementation Information

Contents:

The MMS Implementation contains the used protocol type, e.g. WAP, IP, etc.

Coding:

Allocation of bits:

Bit number	Parameter indicated

1		WAP implementation of MMS according to [35]

2-8		Reserved for future use

Bit value	Meaning

0		Implementation not supported.

1		Implementation supported.

-  MMS notification

Contents:

The MMS notification contains the MMS notification.

Coding:

The MMS notification is coded according to 3G TS 23.140 [34].

-	extension file record number.

Contents:

-	extension file record number. This byte identifies the number of a record in the EFEXT1 containing an associated called party subaddress or additional data. The use of this byte is optional. If it is not used it shall be set to 'FF'.

-	if the ADN/SSC requires both additional data and called party subaddress, this byte identifies the additional record. A chaining mechanism inside EFEXT8 identifies the record of the appropriate called party subaddress (see clause 4.4.2.xx).

Coding:

-	binary.

Any bytes in the record following the TPDU shall be filled with 'FF'.

### 4.2.YY	EFEXT8 (Extension8)

- This EF contains extension data of a MMS Notification (Multimedia Messaging Service - see 4.2.XX).

| Identifier: '6FXX' |  | Structure: linear fixed |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| Record length: X+2 bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE PIN DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | Record type |  |  | M |  | 1 byte |
| 2 - X+2 | Extension data |  |  | M |  | X bytes |
| X+3 – X+4 | Identifier |  |  | M |  | 1 byte |

- For contents and coding see clause 4.4.2.4 (EFEXT1). The record length is to be defined by the application owner.

### 4.2.WW	EFMMSCP (MMS Connectivity Parameters)

This EF contains values for Multimedia Messaging Connectivity Parameters, which can be used by the ME for user assistance in preparation of connecting to the network for the MMS purpose.

| Identifier: '6FXX' |  | Structure: Transparent |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| File Size: X bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE ADM/PIN2 DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 to X | MMS Connectivity parameters TLV objects |  |  | M |  | X bytes |

- MMS Connectivity parameters tags

| Description | Tag Value |
| --- | --- |
| MMS Connectivity Parameters Tag | 'AX1' |
| MMS Implementation Tag | ‘80’ |
| MMS Relay/Server Tag | ‘81’ |
| Interface to Core Network and Bearer Tag | '82' |
| GatewayTag | '83' |

- 	MMS Connectivity Parameters contents

| Description | Value | M/O | Length (bytes) |
| --- | --- | --- | --- |
| MMS Implementation Tag | '80' | M | 1 |
| Length | - | M | Note |
| MMS Implementation Information |  | M | 1 |
| MMS Relay/Server Tag | '81' | M | 1 |
| Length | Y | M | Note |
| Address | -- | M | Y |
| Interface to Core Network and Bearer Tag | '82' | M | 1 |
| Length | -- | M | Note |
| Interface to Core Network and Bearer information | -- | M |  |
| GatewayTag | '83' | O | 1 |
| Length | -- | O | Note |
| Gateway Information | -- | O |  |
| Note : The length is coded according to ISO/IEC 8825 [35] |  |  |  |

MMS Implementation Tag ‘80’

See section 4.2.XX for contents and coding

-  MMS Relay/server Tag ‘81’

Contents:

The relay/server contains the address of the associated MMS relay/server.

Coding:

The MMS relay/server address is coded according to 3G TS 23.140 [34].

- Interface to Core Network and Bearer Tag ‘82’

Contents:

The Interface to Core Network and Bearer may contain following information to setup the bearer; Bearer, Address, Type of address, Speed, Call type, Authentication type, Authentication id, Authentication password

Coding:

The codis  according to 3G TS 23.140 [34].

-  Gateway Tag ‘83’

Contents: The following  gateway information is coded; Address , Type of address, Port, Service, Authentication type , Authentication id and Authentication password

Coding:

The  coding is according to 3G TS 23.140 [34].

### 4.2.ZZ	EFMMUP (MMS User Preferences)

This EF contains values for Multimedia Messaging Service header Parameters, which can be used by the ME for user assistance in preparation of mobile multimedia messages (e.g. default values for parameters that are often used).

| Identifier: '6FXX' |  | Structure: Linear Fixed |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| Record Length: x bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ PIN UPDATE PIN DEACTIVATE ADM ACTIVATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 to X | MMS User Preference TLV Objects |  |  | M |  | X bytes |

- MMS User Preference  tag

| Description | Tag Value |
| --- | --- |
| MMS User implementation Tag | '80' |
| MMS User Preference Alpha Tag | '81' |
| MMS User Preference information Tag | '82' |

MMS User preference information

| Description | Value | M/O | Length (bytes) |
| --- | --- | --- | --- |
| MMS Implementation Tag | '80' | M | 1 |
| Length |  | M | Note |
| MMS Implementation information |  | M | 1 |
| MMS User preference profile name Tag | '81' | M | 1 |
| Length | Y1 | M | Note |
| MMS User profile name |  | M | Y1 |
| MMS User Preference information Tag | '82' | M | 1 |
| Length | Y | M | Note |
| MMS User Preference information | -- | M | Y |
| Note : The length is coded according to ISO/IEC 8825 [35] |  |  |  |

-  MMS Implementation Tag ‘80’

For contents and coding see 4.2.XX

MMS  User preference profile name  Tag '81'

Contents

Alpha tagging of the MMS user preference profile

Coding

this alpha-tagging shall use either:

-	the SMS default 7-bit coded alphabet as defined in TS 23.038 [5] with bit 8 set to 0. The alpha identifier shall be left justified.

or:

-	one of the UCS2 coded options as defined in the annex of TS 31.101 [11].

MMS User Preference information Tag '82'   Contents:

The following information elements may be coded; sender visibility, Delivery Report, Read-Reply, Priority, Time of  delivery, Time of Expiry and Earliest Delivery Time.

Coding:

Depending upon the MMS implementation as indicated in Tag '80'

...

## 4.7	Files of USIM

This subclause contains two figures depicting the file structure of the UICC and the ADFUSIM. ADFUSIM shall be selected using the AID and information in EFDIR.

|  |  |  |  |  |  |  |  |  | MF |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | '3F00' |  |  |  |  |  |  |  |  |  |  |
| DFGSM |  |  | DFTELECOM |  |  |  |  |  | EFDIR |  |  | EFPL |  |  | EFARR |  |  | EFICCID |  |
| '7F20' |  |  | '7F10' |  |  |  |  |  | '2F00' |  |  | '2F05' |  |  | '2F06' |  |  | '2FE2' |  |
|  |  |  |  |  |  | EFARR |  |  | EFADN |  |  | EFFDN |  |  | EFSMS |  |  | EFCCP |  |
| see GSM |  |  |  |  |  | '6F06' |  |  | '6F3A' |  |  | '6F3B' |  |  | '6F3C' |  |  | '6F3D' |  |
| 11.11 [18] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | EFMSISDN |  |  | EFSMSP |  |  | EFSMSS |  |  | EFLND |  |  | EFSMSR |  |
|  |  |  |  |  |  | '6F40' |  |  | '6F42' |  |  | '6F43' |  |  | '6F44' |  |  | '6F47' |  |
|  |  |  |  |  |  | EFSDN |  |  | EFEXT1 |  |  | EFEXT2 |  |  | EFEXT3 |  |  | EFBDN |  |
|  |  |  |  |  |  | '6F49' |  |  | '6F4A' |  |  | '6F4B' |  |  | '6F4C' |  |  | '6F4D' |  |
|  |  |  |  |  |  | EFEXT4 |  |  | EFSUME |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | '6F4E' |  |  | '6F54' |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | DFGRAPHICS |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | '5F50' |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | EFIMG |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | '4F20' |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | DFPHONEBOOK |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | '5F3A' |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | EFPBR |  |  | EFIAP |  |  | EFADN |  |  | EFEXT1 |  |  | EFPBC |  |
|  |  |  |  |  |  | '4F30' |  |  | '4FXX' |  |  | '4FXX' |  |  | '4FXX' |  |  | '4FXX' |  |
|  |  |  |  |  |  | EFGRP |  |  | EFAAS |  |  | EFGAS |  |  | EFANR |  |  | EFSNE |  |
|  |  |  |  |  |  | '4FXX' |  |  | '4FXX' |  |  | '4FXX' |  |  | '4FXX' |  |  | '4FXX' |  |
|  |  |  |  |  |  | EFCCP1 |  |  | EFUID |  |  | EFPSC |  |  | EFCC |  |  | EFPUID |  |
|  |  |  |  |  |  | '4F3D' |  |  | '4F21' |  |  | '4F22' |  |  | '4F23' |  |  | '4F24' |  |
|  |  |  |  |  |  | EFEMAIL |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | '4FXX' |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 4.1: File identifiers and directory structures of UICC

| ADFUSIM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | DFPHONEBOOK |  |  | DFGSM |  |  | DFMExE |  |  |  |  |  |  |  |  | EFTKCDF |  |
|  |  |  | '5F3A' |  |  | '5F3B' |  |  | '5F3C' |  |  |  |  |  |  |  |  | '4FXX' |  |
|  |  |  |  |  |  |  |  |  | EFMExE-ST |  |  | EFORPK |  |  | EFARPK |  |  | EFTPRK |  |
|  |  |  |  |  |  |  |  |  | '4F40' |  |  | '4F41' |  |  | '4F42' |  |  | '4F43' |  |
|  |  |  |  |  |  | EFKc |  |  | EFKcGPRS |  |  | EFBCCH |  |  | EFCPBCCH |  |  | EFinvSCAN |  |
|  |  |  |  |  |  | '4F20' |  |  | '4F52' |  |  | '4F74' |  |  | '4F63' |  |  | '4F64' |  |
|  |  |  |  |  |  | EFUID |  |  | EFPSC |  |  | EFCC |  |  | EFPUID |  |  | EFPBR |  |
|  |  |  |  |  |  | '4F21' |  |  | '4F22' |  |  | '4F23' |  |  | '4F24' |  |  | '4F30' |  |
|  |  |  |  |  |  | EFCCP1 |  |  | EFIAP |  |  | EFADN |  |  | EFEXT1 |  |  | EFPBC |  |
|  |  |  |  |  |  | '4F3D' |  |  | '4FXX' |  |  | '4FXX' |  |  | '4FXX' |  |  | '4FXX' |  |
|  |  |  | EFGRP |  |  | EFAAS |  |  | EFGAS |  |  | EFANR |  |  | EFSNE |  |  | EFEMAIL |  |
|  |  |  | '4FXX' |  |  | '4FXX' |  |  | '4FXX' |  |  | '4FXX' |  |  | '4FXX' |  |  | '4FXX' |  |
|  |  |  | EFLI |  |  | EFARR |  |  | EFIMSI |  |  | EFKeys |  |  | EFKeysPS |  |  | EFDCK |  |
|  |  |  | '6F05' |  |  | '6F06' |  |  | '6F07' |  |  | '6F08' |  |  | '6F09' |  |  | '6F2C' |  |
|  |  |  | EFHPLMN |  |  | EFCNL |  |  | EFACMmax |  |  | EFUST |  |  | EFACM |  |  | EFFDN |  |
|  |  |  | '6F31' |  |  | '6F32' |  |  | '6F37' |  |  | '6F38' |  |  | '6F39' |  |  | '6F3B' |  |
|  |  |  | EFSMS |  |  | EFGID1 |  |  | EFGID2 |  |  | EFMSISDN |  |  | EFPUCT |  |  | EFSMSP |  |
|  |  |  | '6F3C' |  |  | '6F3E' |  |  | '6F3F' |  |  | '6F40' |  |  | '6F41' |  |  | '6F42' |  |
|  |  |  | EFSMSS |  |  | EFCBMI |  |  | EFSPN |  |  | EFSMSR |  |  | EFCBMID |  |  | EFSDN |  |
|  |  |  | '6F43' |  |  | '6F45' |  |  | '6F46' |  |  | '6F47' |  |  | '6F48' |  |  | '6F49' |  |
|  |  |  | EFEXT2 |  |  | EFEXT3 |  |  | EFBDN |  |  | EFEXT5 |  |  | EFCBMIR |  |  | EFEXT4 |  |
|  |  |  | '6F4B' |  |  | '6F4C' |  |  | '6F4D' |  |  | '6F4E' |  |  | '6F50' |  |  | '6F55' |  |
|  |  |  | EFEST |  |  | EFACL |  |  | EFCMI |  |  | EFSTART-HFN |  |  | EFTHRESHOLD |  |  | EFPLMNwAcT |  |
|  |  |  | '6F56' |  |  | '6F57' |  |  | '6F58' |  |  | '6F5B' |  |  | '6F5C' |  |  | '6F60' |  |
|  |  |  | EFOPLMNwAcT |  |  | EFHPLMNwAcT |  |  | EFRPLMNAcT |  |  | EFLOCIPS |  |  | EFACC |  |  | EFFPLMN |  |
|  |  |  | '6F61' |  |  | '6F62' |  |  | '6F65' |  |  | '6F73' |  |  | '6F78' |  |  | '6F7B' |  |
|  |  |  | EFLOCI |  |  | EFICI |  |  | EFOCI |  |  | EFICT |  |  | EFOCT |  |  | EFAD |  |
|  |  |  | '6F7E' |  |  | '6F80' |  |  | '6F81' |  |  | '6F82' |  |  | '6F83' |  |  | '6FAD' |  |
|  |  |  | EFeMLPP |  |  | EFAAeM |  |  | EFECC |  |  | EFHiddenkey |  |  | EFMMSN |  |  | EFMMSCP |  |
|  |  |  | '6FB5' |  |  | '6FB6' |  |  | '6FB7' |  |  | '6FC3' |  |  | '6FXX' |  |  | '6FXX' |  |
|  |  |  | EFMMUP |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | '6FXX' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- Figure 4.2: File identifiers and directory structures of USIM

DF 5F70 is reserved for SoLSA. EF 4F30 (EFSAL) and EF 4F31 (EFSLL) are reserved under DF 5F70 (SoLSA).

...

### 5.3.Z	MMS notifications

-	Requirement:	Service n°XX "available".

Update:	The ME analyses and assembles the MMS notification to be stored.

If the USIM has no available empty space to store the received notification, or if the procedure has been aborted, the ME advises the user.

For reasons of memory efficiency, the ME may analyse all Extension8 records to recognise if the additional or subaddress data to be stored is already existing in EFEXT8. In this case, the ME may use the existing chain or the last part of the existing chain from more than one notification. The ME is only allowed to store extension data in unused records. If existing records are used for multiple access, the ME shall not change any data in those records to prevent corruption of existing chains.

Erasure:	The ME sends the identification of the information to be erased. The content of the identified record in EFMMSN is marked as "free".

Request:	The ME sends the identification of the information to be read. The ME shall analyse the data of EFMMSN to ascertain, whether additional data is associated in EFEXT8. If necessary, then the ME performs the reading procedure on these EFs to assemble the complete notification.

Purge:	The ME shall access each EF which references EFEXT8 for storage and shall identify records in these files using extension data (additional data or called party subaddress). Note that existing chains have to be followed to the end. All referred Extension8 records are noted by the ME. All Extension8 records not noted are then marked by the ME as "free" by setting the whole record to 'FF'.

NOTE:	Dependent upon the implementation of the ME, and in particular the possibility of erasure of notification records by MEs, which have no knowledge of the EFEXT8, it is possible for Extension8 records to be marked as "used space" (not equal to 'FF'), although in fact they are no longer associated with an notification record.

...

Annex A (informative):
EF changes via Data Download or USAT applications

This annex defines if changing the content of an EF by the network (e.g. by sending an SMS), or by a USAT Application, is advisable. Updating of certain EFs "over the air" such as EFACC could result in unpredictable behaviour of the UE; these are marked "Caution" in the table below. Certain EFs are marked "No"; under no circumstances should "over the air" changes of these EFs be considered.

| File identification | Description | Change advised |
| --- | --- | --- |
| '2F00' | Application directory |  |
| '2F05' | Preferred languages | Yes |
| '2F06' | Access rule reference |  |
| '2FE2' | ICC identification | No |
| '4F20' | Image data | Yes |
| '4FXX' | Image Instance data Files | Yes |
| '4FXX' | Unique identifier | Yes |
| '4F22' | Phone book synchronisation counter | Yes |
| '4F23' | Change counter | Yes |
| '4F24' | Previous unique identifier | Yes |
| '4F30' | Phone book reference file | Yes |
| '4FXX' | Capability configuration parameters 1 | Yes |
| '4F75' | CPBCCH Information | No |
| '4F76 | Investigation Scan | Caution |
| '4FXX' | Additional number alpha string | Yes |
| '4FXX' | Additional number | Yes |
| '4FXX' | Second name entry | Yes |
| '4FXX' | Grouping information alpha string | Yes |
| '4FXX' | Phone book control | Yes |
| '4FXX' | E-mail addresses | Yes |
| '4FXX' | Index administration phone book | Yes |
| '4FXX' | Extension 1 | Yes |
| '4FXX' | Abbreviated dialling numbers | Yes |
| '4FXX' | Grouping file | Yes |
| '6F05' | Language indication | Yes |
| '6F07' | IMSI | Caution (Note 1) |
| '6F08' | Ciphering and integrity keys | No |
| '6F09' | Ciphering and integrity keys for packet switched domain | No |
| '6F20' | Ciphering key Kc | No |
| '6F2C' | De-personalization Control Keys | Caution |
| '6F31' | HPLMN search period | Caution |
| '6F32' | Co-operative network list | Caution |
| '6F37' | ACM maximum value | Yes |
| '6F38' | USIM service table | Caution |
| '6F39' | Accumulated call meter | Yes |
| '6F3B' | Fixed dialling numbers | Yes |
| '6F3C' | Short messages | Yes |
| '6F4F' | Extended Capability configuration parameters | Yes |
| '6F3E' | Group identifier level 1 | Yes |
| '6F3F' | Group identifier level 2 | Yes |
| Continued…. |  |  |

| File identification | Description | Change advised |
| --- | --- | --- |
| '6F40' | MSISDN storage | Yes |
| '6F41' | PUCT | Yes |
| '6F42' | SMS parameters | Yes |
| '6F43' | SMS status | Yes |
| '6F44' | Last number dialled | Yes |
| '6F45' | CBMI | Caution |
| '6F46' | Service provider name | Yes |
| '6F47' | Short message status reports | Yes |
| '6F48' | CBMID | Yes |
| '6F49' | Service Dialling Numbers | Yes |
| '6F4B' | Extension 2 | Yes |
| '6F4C' | Extension 3 | Yes |
| '6F4D' | Barred dialling numbers | Yes |
| '6F4E' | Extension 5 | Yes |
| '6F4F' | Capability configuration parameters 2 | Yes |
| '6F50' | CBMIR | Yes |
| '6F52' | GPRS Ciphering key KcGPRS | No |
| '6F54' | SetUp Menu Elements | Yes |
| '6F56' | Enabled services table |  |
| '6F57' | Access point name control list |  |
| '6F58' | Comparison method information |  |
| '6F5B' | Initialisation value for Hyperframe number | Caution |
| '6F5C' | Maximum value of START | Yes |
| '6F60' | User controlled PLMN selector with Access Technology | No |
| '6F61' | Operator controlled PLMN selector with Access Technology | Caution |
| '6F62' | HPLMN selector with Access Technology | Caution |
| '6F63' | RPLMN last used Access Technology | Caution |
| '6F73' | Packet switched location information | Caution |
| '6F78' | Access control class | Caution |
| '6F7B' | Forbidden PLMNs | Caution |
| '6F7E' | Location information | No (Note 1) |
| '6F80' | Incoming call information | Yes |
| '6F81' | Outgoing call information | Yes |
| '6F82' | Incoming call timer | Yes |
| '6F83' | Outgoing call timer | Yes |
| '6FAD' | Administrative data | Caution |
| '6FB5' | Enhanced Multi Level Pre-emption and Priority | Yes |
| '6FB6' | Automatic Answer for eMLPP Service | Yes |
| '6FB7' | Emergency Call Codes | Caution |
| '6FC2' | Group identity | No |
| '6FC3' | Key for hidden phone book entries |  |
| '6FC4' | Network Parameters | No |
| '6FC5' | PLMN Network Name | Yes |
| '6FC6' | Operator Network List | Yes |
| '6FC7' | Mailbox Dialling Numbers | Yes |
| '6FC8' | Extension 6 | Yes |
| '6FC9' | Mailbox Identifier | Caution |
| '6FCA' | Message Waiting Indication Status | Caution |
| '6FCB' | Call Forwarding Indication Status | Caution |
| '6FCC' | Extension 7 | Yes |
| '6FCD' | Service Provider Display Information |  |
| ‘6FXX’ | MMS Notification | Yes |
| ‘6FXX’ | Extension 8 | Yes |
| ‘6FXX’ | MMS Parameters | Yes |
| NOTE1: If EFIMSI is changed, the UICC should issue REFRESH as defined in TS 31.111 and update EFLOCI accordingly. |  |  |

Annex D (informative):
Tags defined in 31.102

| Tag | Name of Data Element | Usage |
| --- | --- | --- |
| 'A0' | GSM cell information The following tags are encapsulated within 'A0': '80' GSM Camping Frequency data object '81' GSM Neighbour Frequency Information data object | Network Parameters (EFNETPAR) |
| 'A1' | FDD cell information The following tags are encapsulated within 'A1': '80' FDD Intra Frequency data object '81' FDD Inter Frequency Information data object | Network Parameters (EFNETPAR) |
| 'A2' | TDD cell information The following tags are encapsulated within 'A2': '80' TDD Intra Frequency data object '81' TDD Inter Frequency Information data object | Network Parameters (EFNETPAR) |
| 'A3' | Service provider display information The following tags are encapsulated within 'A3': '80' Service provider PLMN list | Service Provider Display Information (EFSPDI) |
| 'A8' | Indicator for type 1 EFs (amount of records equal to master EF) The following tags are encapsulated within 'A8': 'C0' EFADN data object 'C1' EFIAP data object 'C3' EFSNE data object 'C4' EFANR data object 'C5' EFPBC data object 'C6' EFGRP data object 'C9' EFUID data object 'CA' EFEMAIL data object | Phone Book Reference File (EFPBR) |
| 'A9' | Indicator for type 2 EFs (EFs linked via the index administration file) The following tags are encapsulated within 'A9': 'C3' EFSNE data object 'C4' EFANR data object 'CA' EFEMAIL data object | Phone Book Reference File (EFPBR) |
| 'AA' | Indicator for type 3 EFs (EFs addressed inside an object using a record identifier as a pointer) The following tags are encapsulated within 'AA': 'C2' EFEXT1 data object 'C7' EFAAS data object 'C8' EFGAS data object 'CB' EFCCP1 data object | Phone Book Reference File (EFPBR) |
| 'DB' | Successful 3G authentication | Response to AUTHENTICATE |
| 'DC' | Synchronisation failure | Response to AUTHENTICATE |
| 'DD' | Access Point Name | APN Control List (EFACL) |
| ‘AX0’ | MMS Connectivity parameters: The following are encapsulated under ‘AX0’: ‘80’ MMS Implementation Tag ‘81’ MMS Relay/Server Tag ‘82’ Interface to corenetwork Tag ‘83’ Gateway Tag | MMS Connectivity Parameters (EFMMSCP) |
| ‘AX1’ | MMS User Preferences: The following are encapsulated under ‘AX1’: ‘80’ MMS Implementation Tag ‘81’ MMS User Preference Alpha Tag ‘82’ MMS User Preference Information Tag | MMS Parameters (EFMMUP) |

Annex E (informative):
Suggested contents of the EFs at pre-personalization

If EFs have an unassigned value, it may not be clear from the main text what this value should be. This annex suggests values in these cases.

| File Identification | Description | Value |
| --- | --- | --- |
| '2F00' | Application directory | Card issuer/operator dependant |
| '2F05' | Preferred languages | 'FF…FF' |
| '2F06' | Access rule reference | Card issuer/operator dependant |
| '2FE2' | ICC identification | operator dependant |
| '4F20' | Image data | '00FF...FF' |
| '4FXX' | Image instance data files | 'FF…FF' |
| '4FXX' | Unique identifier | '0000' |
| '4F22' | Phone book synchronisation counter | '00000000' |
| '4F23' | Change counter | '0000' |
| '4F24' | Previous unique identifier | '0000' |
| '4F30' | Phone book reference file | Operator dependant |
| '4FXX' | Capability configuration parameters 1 | 'FF…FF' |
| '4F63' | CPBCCH Information | 'FF..FF' |
| '4F64' | Investigation PLMN scan | '00' |
| '4FXX' | E-mail addresses | 'FF…FF' |
| '4FXX' | Additional number alpha string | 'FF…FF' |
| '4FXX' | Second name entry | 'FF…FF' |
| '4FXX' | Abbreviated dialling numbers | 'FF...FF' |
| '4FXX' | Grouping file | '00…00' |
| '4FXX' | Grouping information alpha string | 'FF…FF' |
| '4FXX' | Phone book control | '0000' |
| '4FXX' | Index administration phone book | 'FF…FF' |
| '4FXX' | Additional number | 'FF…FF' |
| '4FXX' | Extension 1 | '00FF...FF' |
| '6F05' | Language indication | 'FF…FF' |
| '6F07' | IMSI | Operator dependant |
| '6F08' | Ciphering and integrity keys | '07FF…FF' |
| '6F09' | Ciphering and integrity keys for packet switched domain | '07FF…FF' |
| '6F20' | Ciphering key Kc | 'FF...FF07' |
| '6F2C' | De-personalization control keys | 'FF…FF' |
| '6F31' | HPLMN search period | 'FF' |
| '6F32' | Co-operative network list | 'FF…FF' |
| '6F37' | ACM maximum value | '000000' (see note 1) |
| '6F38' | USIM service table | Operator dependant |
| '6F39' | Accumulated call meter | '000000' |
| '6F3B' | Fixed dialling numbers | 'FF...FF' |
| '6F3C' | Short messages | '00FF...FF' |
| '6F3E' | Group identifier level 1 | Operator dependant |
| '6F3F' | Group identifier level 2 | Operator dependant |
| '6F40' | MSISDN storage | 'FF...FF' |
| '6F41' | PUCT | 'FFFFFF0000' |
| '6F42' | SMS parameters | 'FF...FF' |
| '6F43' | SMS status | 'FF...FF' |
| '6F45' | CBMI | 'FF...FF' |
| '6F46' | Service provider name | Operator dependant |
| '6F47' | Short message status reports | '00FF…FF' |
| '6F48' | CBMID | 'FF...FF' |
| '6F49' | Service Dialling Numbers | 'FF...FF' |
| '6F4B' | Extension 2 | '00FF...FF' |
| '6F4C' | Extension 3 | '00FF...FF' |
|  | Continued.... |  |

| File Identification | Description | Value |
| --- | --- | --- |
| '6F4D' | Barred Dialling Numbers | 'FF...FF' |
| '6F4E' | Extension 5 | '00FF...FF' |
| '6F4F' | Capability configuration parameters 2 | 'FF...FF' |
| '6F50' | CBMIR | 'FF...FF' |
| '6F52' | GPRS Ciphering key KcGPRS | 'FF...FF07' |
| '6F54' | SetUp Menu Elements | Operator dependant |
| '6F55' | Extension 4 | 'FF...FF' |
| '6F56' | Enabled services table | Operator dependant |
| '6F57' | Access point name control list | '00FF…FF' |
| '6F58' | Comparison method information | 'FF…FF' |
| '6F5B' | Initialisation value for Hyperframe number | '00…00' |
| '6F5C' | Maximum value of START | Operator dependant |
| '6F60' | User controlled PLMN selector with Access Technology | 'FFFFFF0000..FFFFFF0000' |
| '6F61' | Operator controlled PLMN selector with Access Technology | 'FFFFFF0000..FFFFFF0000' |
| '6F62' | HPLMN selector with Access Technology | 'FFFFFF0000..FFFFFF0000' |
| '6F65' | RPLMN last used Access Technology | '0000' |
| '6F73' | Packet switched location information | 'FFFFFFFF FFFFFF xxxxxx 0000 FF 01' (see note 2) |
| '6F78' | Access control class | Operator dependant |
| '6F7B' | Forbidden PLMNs | 'FF...FF' |
| '6F7E | Location information | 'FFFFFFFF xxxxxx 0000 FF 01' (see note 2) |
| '6F80' | Incoming call information | 'FF…FF 000000 00 01FFFF' |
| '6F81' | Outgoing call information | 'FF…FF 000000 01FFFF' |
| '6F82' | Incoming call timer | '000000' |
| '6F83' | Outgoing call timer | '000000' |
| '6FAD' | Administrative data | Operator dependant |
| '6FB5' | EMLPP | Operator dependant |
| '6FB6' | AaeM | '00' |
| '6FB7' | Emergency call codes | Operator dependant |
| '6FC2' | Group identity | 'FFFFFFFF' |
| '6FC3' | Key for hidden phone book entries | 'FF…FF' |
| '6FC4' | Network Parameters | 'FF…FF' |
| '6FC5' | PLMN Network Name | Operator dependant |
| '6FC6' | Operator Network List | Operator dependant |
| '6FC7' | Mailbox Dialling Numbers | Operator dependant |
| '6FC8' | Extension 6 | '00 FF...FF' |
| '6FC9' | Mailbox Identifier | Operator dependant |
| '6FCA' | Message Waiting Indication Status | '00 00 00 00 00' |
| '6FCB' | Call Forwarding Indication Status | 'xx 00 FF...FF' |
| '6FCC' | Extension 7 | '00 FF...FF' |
| '6FCD' | Service Provider Display Information |  |
| ‘6FXX’ | MMS notification | ‘00FF…FF’ |
| ‘6FXX’ | Extension 8 | '00 FF...FF' |
| ‘6FXX’ | MMS connectivity parameters | ‘FF…FF’ |
| ‘6FXX’ | MMS user preferences | ‘FF…FF’ |

NOTE 1:	The value '000000' means that ACMmax is not valid, i.e. there is no restriction on the ACM. When assigning a value to ACMmax, care should be taken not to use values too close to the maximum possible value 'FFFFFF', because the INCREASE command does not update EFACM if the units to be added would exceed 'FFFFFF'. This could affect the call termination procedure of the Advice of Charge function.

NOTE 2:	xxxxxx stands for any valid MCC and MNC, coded according to 3G TS 24.008 [9].

# Annex X (informative): Example of MMS coding

0xAx2 MMS User Preference  Tag

0x34 Length

0x80 MMS  User preference profile name  Tag

0x1C Length

“Christmas Card”

0x81 MMS User Preference  tag

0x14 Length

0x14 0x80 (visibility: hide)

0x06 0x80 (delivery report: yes)

0x10 0x80 (Read-reply: yes)

0x0F 0x81 (Priority: Normal)

0x07  0x80 0x11 0x22 0x33 0x44   (Delivery time: Absolute-token Date -Value | value)

0x08  0x80 0x55 0x22 0x33 0x44   (Expiry: Absolute-token Date -Value | value)