| 3GPP TSG-T WG2 SWG3#10 Paris, France 21st – 25th January 2002 | T2M020109 |
| --- | --- |

Title:	Liaison Statement on MMS availability and USIM/UICC portability

Source:	T2-SWG3

To:	T2-SWG2, SA1

CC:	T3

Contact Person:

#### Name:	Ileana Leuca

Company:		AT&T Wireless Services

Tel. Number:	+ 1 425 580 7900

E-mail Address:	ileana.leuca@attws.com

Attachments:		None

1. Overall Description:

We would like to respectfully bring to the attention of your group(s) a brief summary of possible UE scenarios, related to USIM/UICC portability.

For Release 5, T2-SWG3 and T3, jointly are working in defining MMS-related information to be stored on the USIM application.  The MMS relevant information added to the 3GPP specifications (i.e. 3GPP TS 23.140 and 3GPP TS 31.102, includes

- MMS connectivity parameters, which can be used by an MMS User Agent to connect to the associated MMS Relay/Server and to specify the interface characteristics to the core network (e.g. user MMS Relay/Server IP address,  address type, call type)

- MMS user preferences, which can be used by the MMS User Agent in assisting the user in MM creation (e.g. sender visibility, priority, time of expiry).

- MMS notifications and the associated status of an MM, reflecting the MM message status (e.g. MM not retrieve, MM rejected, MM forwarded), and the MMS implementation (WAP etc.)

Storing MMS parameters on the USIM allows a subscriber to port the UICC from device-to-device and retain the ability to retrieve MMs in a consistent manner.

It should be noted that the portability of the USIM/UICC enables a user to use a diversity of devices, having different set of capabilities (e.g. MMS client present, MMS client absent).   Our group is concerned that the USIM/UICC portability and the dynamic relation between device capabilities/UISM capabilities is not studied enough and that the USIM/UICC portability may have an impact on the MMS service.

E.g. the MMS Relay/Server may continue to send MMS notifications while a user changes his device to a non-MMS capable one. In this scenario, it is suggested, that the MMS Relay/Server should be able to get the information from the GUP when the device’s MMS capabilities are changed.

Our group wishes to inform the GUP of the need for the 3GPP- GUP specifications to include the relation between the USIM/UICC portability and MMS availability.

2. Actions:

To GUP:

ACTION: 	T2 asks GUP to consider this scenario and the impact on MMS service, while defining the Generic User Profile. 
T2 asks GUP to define the requirements for this scenario and share them with us.

3. Date of Next T2 Meetings:

| T2#16 | 11-15 Feb 2002 | Sophia Antipolis |
| --- | --- | --- |
| T2#17 | 13-17 May 2002 | tbd |