| 3GPP TSG-T2 #13 EMS Ad-Hoc London, UK 3rd - 5th April 2001 |  |  |  |  |  |  |  |  |  | T2E-010006 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR-Form-v3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23.040 |  |  |  | CR | CR-Num |  |  |  rev |  |  |  |  | - |  |  Current version: |  |  |  |  | 4.1.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  |  | x |  | Radio Access Network |  |  |  |  |  |  |  |  | Core Network |  |  |
| Title:  |  | Default Text Format |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Ericsson |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  | Messaging (SMS) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 26-March-2001 |  |  |  |  |  |
| Category:  |  | B |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | Rel 4 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | There is no way to specify a default text format for an entire message |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Addition of an IE to allow a default text formatting to be set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | There will be no sensible method of specifying a default text format for the entire message, since at the moment any text format can apply only within a single SM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at: http://www.3gpp.org/3G_Specs/CRs.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://www.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2000-09 contains the specifications resulting from the September 2000 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

### 3.10.1	Text formatting

- Text formatting may be applied to a run of text or to the message as a whole. The following text formatting features are supported:

Alignment

-	Left

-	Centre

- Right

- Default (Language dependent)

Font size

-	Normal (default)

-	Large

-	Small

Style

-	Normal (default)

-	Bold

-	Italic

-	Underlined

-	Strikethrough

9.2.3.24

…

The Information Element Identifier octet shall be coded as follows:

| VALUE (hex) | MEANING |
| --- | --- |
| 00 | Concatenated short messages, 8-bit reference number |
| 01 | Special SMS Message Indication |
| 02 | Reserved |
| 03 | Value not used to avoid misinterpretation as <LF> character |
| 04 | Application port addressing scheme, 8 bit address |
| 05 | Application port addressing scheme, 16 bit address |
| 06 | SMSC Control Parameters |
| 07 | UDH Source Indicator |
| 08 | Concatenated short message, 16-bit reference number |
| 09 | Wireless Control Message Protocol |
| 0A | Text Formatting |
| 0B | Predefined Sound |
| 0C | User Defined Sound (iMelody max 128 bytes) |
| 0D | Predefined Animation |
| 0E | Large Animation (16*16 times 4 = 32*4 =128 bytes) |
| 0F | Small Animation (8*8 times 4 = 8*4 =32 bytes) |
| 10 | Large Picture (32*32 = 128 bytes) |
| 11 | Small Picture (16*16 = 32 bytes) |
| 12 | Variable Picture |
| 13 | User prompt indicator |
| 14 | Default Text Formatting |
| 15-1F | Reserved for future EMS features (see subclause 3.10) |
| 20 | RFC 822 E-Mail Header |
| 21-6F | Reserved for future use |
| 70 – 7F | (U)SIM Toolkit Security Headers |
| 80 – 9F | SME to SME specific use |
| A0 – BF | Reserved for future use |
| C0 – DF | SC specific use |
| E0 – FF | Reserved for future use |

A receiving entity shall ignore (i.e. skip over and commence processing at the next information element) any information element where the IEI is Reserved or not supported. The receiving entity calculates the start of the next information element by looking at the length of the current information element and skipping that number of octets.

The SM itself may be coded as 7, 8 or 16 bit data.

If 7 bit data is used and the TP-UD-Header does not finish on a septet boundary then fill bits are inserted after the last Information Element Data octet up to the next septet boundary so that there is an integral number of septets for the entire TP-UD header. This is to ensure that the SM itself starts on an septet boundary so that an earlier Phase mobile shall be capable of displaying the SM itself although the TP-UD Header in the TP-UD field may not be understood.

It is optional to make the first character of the SM itself a Carriage Return character encoded according to the default 7 bit alphabet so that earlier Phase mobiles, which do not understand the TP-UD-Header, shall over-write the displayed TP-UD-Header with the SM itself.

If 16 bit (USC2) data is used then padding octets are not necessary. The SM itself shall start on an octet boundary.

If 8 bit data is used then padding is not necessary. An earlier Phase mobile shall be able to display the SM itself although the TP-UD header may not be understood.

It is also possible for mobiles not wishing to support the TP-UD header to check the value of the TP-UDHI bit in the SMS-Deliver PDU and the first octet of the TP-UD field and skip to the start of the SM and ignore the TP-UD header.

9.2.3.24.10.1.11	Default Text Formatting

- The Information-Element-Data octet(s) shall be coded as follows.

Octet 1	Default Formatting mode value coded as following:

- Octet 3:	Bit 7	Bit 6	Bit 5	Bit 4	Bit 3	Bit 2	Bit 1	Bit 0

- Bit 1	Bit 0		*Alignment
0	0		Left
0	1		Center
1	0		Right
1	1		Language dependent

- *in case formatting text is inserted on the same line as previous non formatting text or with a different mode value, the alignment value shall be set to the same value as the previous formatted predefined object.

- Bit 3	Bit 2		Font Size
0	0		Normal 
0	1		Large
1	0		Small
1	1		reserved

- Bit 4			Style bold
1			Bold on
0			Bold off

- Bit 5			Style Italic
1			Italic on
0			Italic off

- Bit 6			Style Underlined
1			Underlined on
0			Underlined off

- Bit 7			Style Strikethrough
1			Strikethrough on
0			Strikethrough off

- If bit 4,5,6 and 7 are set to 0, it will mean normal style.

Octet 2		Text Colour

8-bit colour value from 0..215

A colour value is defined using the standard "websafe" colour palette. The colour value used corresponds to the ordinal value of the colour in the websafe range, assuming that the RGB hex triplets have been sorted in ascending order. *

Values outside the range 0..215 inclusive shall be treated as value 0.

Colour Value			RGB Hex Triplet				Websafe Name

0						000000						Black

1						000033						Obscure Weak Blue

2						000066						Obscure Dull Blue

3						000099						Dark Faded Blue

4						0000CC						Dark Hard Blue

5						0000FF						Blue

6						003300						Obscure Weak Green

7						003333						Obscure Weak Cyan

…						….								….

214					FFFFCC						Pale Weak Yellow

215					FFFFFF						White

216-255				000000						Black

*It is up to the manufacturer to provide a suitable mapping between the websafe colour value used and the actual colour displayed on the ME.

Octet 3		Text Background Colour

8-bit colour value from 0..215

(For definition of colour values see Octet 4 above)