| 3GPP TSG-T2 SWG3 SMS / EMS ad hoc #1 London, UK 3rd to 5th April 2001 | T2E-010008 |
| --- | --- |

Agenda Item:	Large Objects

Source:	Ericsson

Title:	Large Objects

Document for:	Discussion

___________________________________________________________________________

### Introduction

This CR to 3GPP TS 23.040 proposes to include two new Information Elements for EMS. These IEs allow to include large objects in EMS messages (GIF and JPEG images, vCard and vCalendar objects, etc.).

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.

[1]	3GPP TS 01.04: "Digital cellular telecommunication system (Phase 2+); Abbreviations and acronyms".

[2]	3GPP TS 02.03: "Digital cellular telecommunication system (Phase 2+); Teleservices supported by a GSM Public Land Mobile Network (PLMN)".

[3]	3GPP TS 22.004: "General on supplementary services".

[4]	3GPP TS 22.041: "Operator determined barring".

[5]	3GPP TS 43.002: "Digital cellular telecommunication system (Phase 2+); Network architecture".

[6]	3GPP TS 23.008: "Organization of subscriber data".

[7]	3GPP TS 23.011: "Technical realization of supplementary services - General Aspects".

[8]	3GPP TS 23.015: "Technical realization of Operator Determined Barring (ODB)".

[9]	3GPP TS 23.038: "Alphabets and language-specific information".

[10]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[11]	3GPP TS 43.047: "Digital cellular telecommunication system; Example protocol stacks for interconnecting Service Centre(s) (SC) and Mobile-services Switching Centre(s) (MSC)".

[12]	3GPP TS 44.008: "Digital cellular telecommunication system (Phase 2+); Mobile radio interface layer 3 specification".

[13]	3GPP TS 24.011: "Short Message Service (SMS) support on mobile radio interface".

[14]	3GPP TS 27.005: "Use of Data Terminal Equipment - Data Circuit terminating Equipment (DTE - DCE) interface for Short Message Service (SMS) and Cell Broadcast Service (CBS)".

[15]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[16]	3GPP TS 51.011: "Digital cellular telecommunication system (Phase 2+); Specification of the Subscriber Identity Module - Mobile Equipment (SIM- ME) interface".

[17]	CCITT Recommendation E.164 (Blue Book): "Numbering plan for the ISDN era".

[18]	CCITT Recommendation E.163 (Blue Book): "Numbering plan for the international telephone service".

[19]	CCITT Recommendation Q.771: "Specifications of Signalling System No.7; Functional description of transaction capabilities".

[20]	CCITT Recommendation T.100 (Blue Book): "International information exchange for interactive videotex".

[21]	CCITT Recommendation T.101 (Blue Book): "International interworking for videotex services".

[22]	CCITT Recommendation X.121 (Blue Book): "International numbering plan for public data networks".

[23]	CCITT Recommendation X.400 (Blue Book): "Message handling system and service overview".

[24]	ISO/IEC10646: "Universal Multiple-Octet Coded Character Set (USC); UCS2, 16 bit coding".

[25]	3GPP TS 22.022: "Personalization of GSM ME Mobile functionality specification - Stage 1".

[26]	3GPP TS 23.042: "Compression Algorithm for Text Messaging Services".

[27]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[28]	3GPP TS 43.048: "Digital cellular telecommunications system (Phase 2+); Security Mechanisms for the SIM application toolkit; Stage 2".

[29]	3GPP TR 21.905: "3G Vocabulary".

[30]	3GPP TS 31.102: "Characteristics of the USIM application".

[31]	3GPP TS 31.101: "UICC – Terminal interface; Physical and logical characteristics".

[32]	3GPP TS 22.105: "Services and Service Capabilites".

[33]	Infrared Data Association. Specifications for Ir Mobile Communications (IrMC).
iMelody.

[34]	IETF RFC 822: "Standard for the format of ARPA Internet text messages".

[35]	Graphics Interchange Format (Version 89a), Compuserve, Inc., Columbus, Ohio, 1990.

[36]	JPEG Draft Standard ISO 10918-1 CD.

...

      - TP-User Data (TP-UD)

...

The Information Element Identifier octet shall be coded as follows:

| VALUE (hex) | MEANING |
| --- | --- |
| 00 | Concatenated short messages, 8-bit reference number |
| 01 | Special SMS Message Indication |
| 02 | Reserved |
| 03 | Value not used to avoid misinterpretation as <LF> character |
| 04 | Application port addressing scheme, 8 bit address |
| 05 | Application port addressing scheme, 16 bit address |
| 06 | SMSC Control Parameters |
| 07 | UDH Source Indicator |
| 08 | Concatenated short message, 16-bit reference number |
| 09 | Wireless Control Message Protocol |
| 0A | Text Formatting |
| 0B | Predefined Sound |
| 0C | User Defined Sound (iMelody max 128 bytes) |
| 0D | Predefined Animation |
| 0E | Large Animation (16*16 times 4 = 32*4 =128 bytes) |
| 0F | Small Animation (8*8 times 4 = 8*4 =32 bytes) |
| 10 | Large Picture (32*32 = 128 bytes) |
| 11 | Small Picture (16*16 = 32 bytes) |
| 12 | Variable Picture |
| 13 | User prompt indicator |
| 14 | Large object |
| 15 | Additional large object segment |
| 16-1F | Reserved for future EMS features (see subclause 3.10) |
| 20 | RFC 822 E-Mail Header |
| 21-6F | Reserved for future use |
| 70 – 7F | (U)SIM Toolkit Security Headers |
| 80 – 9F | SME to SME specific use |
| A0 – BF | Reserved for future use |
| C0 – DF | SC specific use |
| E0 – FF | Reserved for future use |

A receiving entity shall ignore (i.e. skip over and commence processing at the next information element) any information element where the IEI is Reserved or not supported. The receiving entity calculates the start of the next information element by looking at the length of the current information element and skipping that number of octets.

The SM itself may be coded as 7, 8 or 16 bit data.

If 7 bit data is used and the TP-UD-Header does not finish on a septet boundary then fill bits are inserted after the last Information Element Data octet up to the next septet boundary so that there is an integral number of septets for the entire TP-UD header. This is to ensure that the SM itself starts on an septet boundary so that an earlier Phase mobile shall be capable of displaying the SM itself although the TP-UD Header in the TP-UD field may not be understood.

It is optional to make the first character of the SM itself a Carriage Return character encoded according to the default 7 bit alphabet so that earlier Phase mobiles, which do not understand the TP-UD-Header, shall over-write the displayed TP-UD-Header with the SM itself.

If 16 bit (USC2) data is used then padding octets are not necessary. The SM itself shall start on an octet boundary.

If 8 bit data is used then padding is not necessary. An earlier Phase mobile shall be able to display the SM itself although the TP-UD header may not be understood.

It is also possible for mobiles not wishing to support the TP-UD header to check the value of the TP-UDHI bit in the SMS-Deliver PDU and the first octet of the TP-UD field and skip to the start of the SM and ignore the TP-UD header.

...

9.2.3.24.10.1.11	Large Object

This facility is used to include the first part of a large object or the entire large object itself in a short message.

The IE length is variable.

Octet 1	position indicating in the SM data the instant the large object shall be displayed.

This octet shall be coded as an integer value in the range 0 (beginning of the SM data) to the maximum number of characters included in the SM data of one single SM or one segment of a concatenated SM.

Octet 2	large object reference number

This octet shall contain a modulo 256 counter indicating the reference number for a particular large object. This reference number shall remain constant for every additional large object segment which makes up a particular large object.

Octet 3	maximum number of segments in the large object

This octet shall contain a value in the range 0 to 255 indicating the total number of segments (first segment + additional segments) in the large object. The value shall start at 1 (no additional large object segment). If the value is zero then the receiving entity shall ignore the whole Information Element.

Octet 4	large object format

This octet informs on the format of the large object from the following table:

| VALUE (hex) | MEANING |
| --- | --- |
| 00 | User defined sound (iMelody) User defined sounds are coded according to the iMelody format [33]. |
| 01 | User defined black and white picture User defined black and white pictures are coded according to subclause 9.2.3.24.10.1.11 |
| 02 | User defined 2-bit greyscale picture User defined 2-bit greyscale pictures are coded according to subclause [tbd]. |
| 03 | User defined colour picture User defined colour pictures are coded according to subclause [tbd]. |
| 04 | VCalendar object – UCS2 (16 bits) encoding vCalendar objects are coded according to the vCalendar format [33]. |
| 05 | VCard object – UCS2 (16 bits) encoding vCard objects are coded according to the vCard format [33]. |
| 06 | VCalendar object – ASCII (8 bits) encoding vCalendar objects are coded according to the vCalendar format [33]. |
| 07 | VCard object – ASCII (8 bit) encoding vCard objects are coded according to the vCard format [33]. |
| 08 | GIF image GIF images are coded according to the GIF format [35]. |
| 09 | JPEG image JPEG images are coded according to the JPEG format [36]. |
| 0A...FF | Reserved |

If the value is reserved then the receiving entity shall ignore the whole Information Element and associated Information Elements (additional large object segments).

Octet 5..n

These octets shall contain the first segment of, or an entire, large object.

9.2.3.24.10.1.11.1 User defined black and white picture

Octet 5	Horizontal dimension of picture

This octet shall contain the horizontal number of 8 pixels i.e. this value shall be multiplied by 8 to get the whole number of horizontal pixels

Octet 6	Vertical dimension of picture

This octet shall contain the vertical number of pixels.

Octet 7..n

Picture data, line by line from top left to bottom right. Within each byte the MSB represents the leftmost pixel of  eight. The colour values are encoded as follows

Bit Value			Colour

0					White

- Black

Picture data can be streamed across messages using the Additional Large Object Segment IE as defined in subclause 9.2.3.24.10.1.12

9.2.3.24.10.1.11.2 User defined 2-bit Greyscale picture

Octet 5	Horizontal dimension of picture

This octet shall contain the horizontal number of 4 pixels i.e. this value shall be multiplied by 4 to get the whole number of horizontal pixels

Octet 6	Vertical dimension of picture

This octet shall contain the vertical number of pixels.

Octet 7..n

Picture data, line by line from top left to bottom right.  Within each byte four pixels are encoded. The pair of bits at the MSB represents the leftmost pixel of the four defined in the same byte.

The colour values are encoded as follows

Bit Value			Colour

00					White

- Light Grey

10					Dark Grey

11					Black

Picture data can be streamed across messages using the Additional Large Object Segment IE as defined in subclause 9.2.3.24.10.1.12

9.2.3.24.10.1.11.3 User defined Colour picture

Octet 5	Horizontal dimension of picture

This octet shall contain the horizontal number of  pixels

Octet 6	Vertical dimension of picture

This octet shall contain the vertical number of pixels.

Octet 7..n

Picture data, line by line from top left to bottom right.  Each byte represents an 8-bit colour value from 0..215

A colour value is defined using the standard "websafe" colour palette. The colour value used corresponds to the ordinal value of the colour in the websafe range, assuming that the RGB hex triplets have been sorted in ascending order. *

Values outside the range 0..215 inclusive shall be treated as value 0.

Colour Value			RGB Hex Triplet				Websafe Name

0						000000						Black

1						000033						Obscure Weak Blue

2						000066						Obscure Dull Blue

3						000099						Dark Faded Blue

4						0000CC						Dark Hard Blue

5						0000FF						Blue

6						003300						Obscure Weak Green

7						003333						Obscure Weak Cyan

…						….								….

214					FFFFCC						Pale Weak Yellow

215					FFFFFF						White

216-255				000000						Black

Picture data can be streamed across messages using the Additional Large Object Segment IE as defined in subclause 9.2.3.24.10.1.12

*It is up to the manufacturer to provide a suitable mapping between the websafe colour value used and the actual colour displayed on the ME.

9.2.3.24.10.1.12 	Additional Large Object Segment

This facility is used to include an additional large object segment for a large object.

The IE length is variable.

Octet 1	large object reference number

This octet shall contain a modulo 256 counter indicating the reference number for a particular large object. This reference number shall remain constant for every additional large object segment which makes up a particular large object.

Octet 2	sequence number of the additional large object segment

This octet shall contain a value in the range 0 to 255 indicating the sequence number of a particular additional large object segment complementing a large object. The value shall start at 1 and increment by one for every additional large object segment which makes up the large object. If the value is zero then the receving entity shall ignore the whole Information Element.

Octet 3..n

These octets shall contain a segment of a large object.

...

--