| 3GPP TSG-T2 #12 London Heathrow 3-5 April 2001 | T2E-010012 |
| --- | --- |

Agenda Item:		SMS

Source:		Magic4

Title:	Introduction of Extended  Format Tags

Document for:		Change Request 23.040

___________________________________________________________________________

#### Introduction

This CR proposes to include  new Information Elements for EMS. These IE allow to grouped format types which support and expandable series of  sub level information formats

| 3GPP TSG-T2 SWG3 Ad Hoc 1 London UK 3-5 April 2001 |  |  |  |  |  |  |  |  | T2E-010012 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR-Form-v3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Spec-Number |  |  |  | CR | CR-Num |  |  |  rev |  |  |  | - |  |  Current version: |  |  |  |  | x.y.z |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  | X |  | Radio Access Network |  |  |  |  |  |  |  |  | Core Network |  |  |
| Title:  |  | Ammendment of iei tags to support grouped expanded format types |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Magic4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  | Messaging (SMS) |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 05 February 2001 |  |  |  |  |  |
| Category:  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-4 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | There is a limit to the number of iei’s available for use with different format types and an increasing number of new format types which will need to be supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Five IE are defined for allowing the inclusion of categories for sounds, images , animations, Text and non-specific information types. The category types will support via a 2 byte identifier space for 255 sub data formats and recognise the existence of current data formats. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | There will be no standardised means of exchanging existing data formats or support for existing or evolving formats. The iei table will be filled with specific data formats leaving limited headroom for future inclusive evolution |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at: http://www.3gpp.org/3G_Specs/CRs.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://www.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2000-09 contains the specifications resulting from the September 2000 TSG meetings.,

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

The Information Element Identifier octet shall be coded as follows:

| VALUE (hex) | MEANING |
| --- | --- |
| 00 | Concatenated short messages, 8-bit reference number |
| 01 | Special SMS Message Indication |
| 02 | Reserved |
| 03 | Value not used to avoid misinterpretation as <LF> character |
| 04 | Application port addressing scheme, 8 bit address |
| 05 | Application port addressing scheme, 16 bit address |
| 06 | SMSC Control Parameters |
| 07 | UDH Source Indicator |
| 08 | Concatenated short message, 16-bit reference number |
| 09 | Wireless Control Message Protocol |
| 0A | Text Formatting |
| 0B | Predefined Sound |
| 0C | User Defined Sound (iMelody max 128 bytes) |
| 0D | Predefined Animation |
| 0E | Large Animation (16*16 times 4 = 32*4 =128 bytes) |
| 0F | Small Animation (8*8 times 4 = 8*4 =32 bytes) |
| 10 | Large Picture (32*32 = 128 bytes) |
| 11 | Small Picture (16*16 = 32 bytes) |
| 12 | Variable Picture |
| 13 | User prompt indicator |
| 14 | Large EMS Messages |
| 15-1F | Reserved for future EMS features (see subclause 3.10) |
| 20 | RFC 822 E-Mail Header |
| 21-6F | Reserved for future use |
| 70 – 7F | (U)SIM Toolkit Security Headers |
| 80 – 9F | SME to SME specific use |
| A0 – BF | Reserved for future use |
| C0 – DF | SC specific use |
| E0 – FF | Reserved for future use |

A receiving entity shall ignore (i.e. skip over and commence processing at the next information element) any information element where the IEI is Reserved or not supported. The receiving entity calculates the start of the next information element by looking at the length of the current information element and skipping that number of octets.

The SM itself may be coded as 7, 8 or 16 bit data.

If 7 bit data is used and the TP-UD-Header does not finish on a septet boundary then fill bits are inserted after the last Information Element Data octet up to the next septet boundary so that there is an integral number of septets for the entire TP-UD header. This is to ensure that the SM itself starts on an septet boundary so that an earlier Phase mobile shall be capable of displaying the SM itself although the TP-UD Header in the TP-UD field may not be understood.

It is optional to make the first character of the SM itself a Carriage Return character encoded according to the default 7 bit alphabet so that earlier Phase mobiles, which do not understand the TP-UD-Header, shall over-write the displayed TP-UD-Header with the SM itself.

If 16 bit (USC2) data is used then padding octets are not necessary. The SM itself shall start on an octet boundary.

If 8 bit data is used then padding is not necessary. An earlier Phase mobile shall be able to display the SM itself although the TP-UD header may not be understood.

It is also possible for mobiles not wishing to support the TP-UD header to check the value of the TP-UDHI bit in the SMS-Deliver PDU and the first octet of the TP-UD field and skip to the start of the SM and ignore the TP-UD header.

Information elements  14-18  additionally support a  subgroup of  Content format tags (outlined in Annex F )  which  identify specific content formats  which can may be recognised by the mobile. If a mobile does not support the content an error message  may be display to inform the user that this content format is not supported.

Annex F

Information Format Tags

This Annex identiies the proposed supplementary tags which support iei values 14-18 and include the facility for support of existing formats and potential information formats which may be support by a handset .

| Information Element Identifier | Content Format Tag | Description |
| --- | --- | --- |
| Sounds ( ) | 00 | Predefined Sound |
|  | 01 | User Defined Sound |
|  | 02 | Nokia format Ringtone |
|  | 03 | Bitstream Format Sounds |
|  | 04 | Jade Format Sounds |
|  | 05 | MIDI Sounds |
|  | 06 | Polyphonic Sounds |
|  | 07-2F | Reserved for additional Sound formats |
| Images ( ) | 30 | Large Picture |
|  | 31 | Small Picture |
|  | 32 | Variable Picture |
|  | 33 | Operator replacement logo |
|  | 34 | Nokia Picture Message |
|  | 35 | variable Bitstream Image format |
|  | 36 | GiF format |
|  | 37 | J-PEG format |
|  | 38-4F | Reserved for additional Image Formats |
| Animations | 50 | Large Animations |
|  | 51 | Small Animations |
|  | 52 | Predefined Animations |
|  | 53 | Freeform animations |
|  | 55 | Animated GIF |
|  | 55-6F | Reserved for Additional Animation Formats |
| Text Mesages | 70 | Text formatting |
|  | 71 | Text Positioning |
|  | 72 | Text Display Mode |
|  | 73 | Embedded Text Stream |
|  | 74-7F | Reserved for Additional Text Formats |
| Non Specific Information Types | 80 | Forms |
|  | 81 | Service ID |
|  | 82 | Non Specific Binary Data |
|  | 83 | VCard objects |
|  | 84 | VCal objects |
|  | 85 | Compressed byte stream |
|  | 86-FF | Reserved for Additional Formats |

Definitions of the content type

Predefined sounds

As defined  in TS 23.040  9.2.3.24.10.1.2

User Defined Sound

As defined in TS 23.040 9.2.3.24.10.1.3

Nokia Ringtone format

As defined in Nokia Smart Messaging Revision 3.0.0    18 –12-2000

Bitsteam Format Sound

Bitstream Sound uses the format  <MAIN TAG><CONTROL BYTES><LENGTH> <DATA>

<Main Tag>  is the Content Format Tag identified in table F

<Control Bytes>

The control bytes (CBx) include various bit fields and flags for controlling how the client deals with the data. The client software will be able to read and process any number of control bytes, even if it does not use the content of the control bytes.  As a minimum the client will be able to understand at least control bytes 1 and 2 (CB1 & CB2). If a second control byte (CB2) is not defined then it is assumed that no encryption is used and licensing allows data to be transmitted without restriction.

| Control Byte 1 Table |  |
| --- | --- |
| Bit(s) | Usage |
| 7 | Another control byte following flag: 0 No more control bytes follow (default) 1 At least 1 more control byte follows |
| 6 | ‘O’ bit – flag indicating the current operation |
| 5 | ‘D’ bit – flag indicating display |
| 4 | ‘P’ bit – flag indicating persistence |
| 3 | ‘R’ bit – flag indicating unencrypted reply |
| 2 | ‘RE’ bit -- flag indicating encrypted reply |
| 1-0 | The Length of the <LENGTH> field 00 1 byte 01 2 bytes 10 3 bytes 11 4 bytes |

| Control Byte 2 Table- Optional |  |
| --- | --- |
| Bit(s) | Meaning |
| 7 | Another control byte following flag: 0 No more control bytes follow (default) 1 At least 1 more control byte follows |
| 6-4 | Encryption Method 000 No Encryption (default) 001 DES 010 Triple DES 011 RSA 100+ Reserved |
| 3-2 | Licensing bits: 00 Media can be transmitted and persisted freely (default) 01 Media can be persisted, but not transmitted 10 Media can be transmitted but target cannot persist 11 Media cannot be persisted |
| 1-0 | Reserved – Assumed set to zero. |

<Length>

The <LENGTH> element describes the length of the <DATA> section in bytes.

<Data>

Data is represented in a single Notestream

A <NOTE STREAM> is a simple series of note tones (including the special pause note).  Each note is represented by 2 bytes (16 bits), as described in the table below:

| Bit(s) | Usage |
| --- | --- |
| 15 – 14 | Octave Number: 00 Octave 0 01 Octave 1 10 Octave 2 11 Octave 3 |
| 13 – 10 | Note Identifier – Specifies particular note: 0000 Pause 0001 C 0010 C# 0011 D 0100 E$ 0101 E 0110 F 0111 F# 1000 G 1001 A$ 1010 A 1011 B$ 1100 B 1101 – 1111 Reserved |
| 9 – 0 | Duration – Length of note to be played (time units are 1/100 s): 0000000000 1 time unit (0.01s) … … … … 1111111111 1024 time units (10.24s) |

Jade Sound Format

As defined in Motorola Document Appended

MIDI  Musical Information Digital Interface

Definition to be  Appended MIDI 1.0 standard document

Polyphonic Sounds

To be defined as per Ad Hoc Group

Large Picture Format

As defined in TS 23.040      9.2.3.24.10.1.7

Small Picture Format

As defined in TS 23.040      9.2.3.24.10.1.8

Variable Picture

As defined in TS 23.040      9.2.3.24.10.1.9

Operator Replacement Logo

TBN

Nokia Picture Message Format

As defined in Nokia Smart Messaging Revision 3.0.0    18 –12-2000

Variable Bitstream  Image Format

<ICON SUBTAG><IMAGE HEADER>	[ <TOP LEFT POSITION> ]	<SIZE>[ <EXT INFO>] <LENGTH><IMAGE DATA>

### <IMAGE HEADER> is 1 byte defined as follows:

| Bit(s) | Usage |
| --- | --- |
| 7 – 6 | Length of the <LENGTH> field: 00 1 byte 01 2 bytes 10 3 bytes 11 4 bytes |
| 5 – 3 | Compression Scheme: 000 No Compression 001 RLE Scheme 1 010 RLE Scheme 2 011-111 Reserved. |
| 2 | Reserved. |
| 1 | Reserved. |
| 0 | <EXT INFO> presence flag: 0 <EXT INFO> not present 1 <EXT INFO> present |

### <Position>

| Position Co-ordinate position in X, Y format. A <POSITION> consists of four bytes. | <POSITION>::= { X Position (2 bytes) } { Y Position (2 bytes) } |
| --- | --- |

### <SIZE>

The <SIZE> is a four-byte quantity.  The first pair of bytes hold the image width (in pixels), the second pair of bytes hold the image height (in pixels).  The values can range from 0 – 65535.

### <EXT INFO>

The <EXT INFO> byte is an optional byte that can be used to hold further extended information- at present unused.

### <LENGTH>

This field can range between 1 and four bytes inclusive.  Its value represents the amount of bytes comprising the < IMAGE DATA>.

### < IMAGE DATA>

The < IMAGE DATA> is the raw data for the image.  This is simply a bit-based bitmap for the whole of the image.  The final byte of the data may be partially used, depending upon the dimensions of the image.

Examples:

- An 8 by 8 bitmap would take 8 bytes, with one byte representing each row of bits within the bitmap.

- A 9 by 9 bitmap would take 11 bytes.  The highest bit of the 2nd byte would represent the last bit of the first row of the bitmap.  The second highest bit of the second byte would represent the first bit of the second row of the bitmap.  Continuing in this fashion wastes no bits in any of the bytes apart from the final byte where the highest bit represents the last bit of the ninth row of the bitmap.  The remaining bits within this byte are unused.

GIF Format

As defined in  attached Appendix

J-PEG

As defined in attached Appendix

Large Animations

As defined in TS 23.040  9.2.3.24.10.1.5

Small Animations

As defined in TS 23.040    9.2.3.24.10.1.6

Predefined Animations

As defined in TS 23.040  9.2.3.24.10.1.4

Freeform Animations

| Iconimation A <Freeform Animation> is an animated monochrome bitmap. | <ICONIMATION> ::= <Freeform Animation TAG> <Freeform Animation HEADER> <DELREP> [ <EXT INFO ] <Freeform IMAGE>+ |
| --- | --- |

### The <Freeform HEADER> is 1 byte defined as follows:

| Bit | Usage |
| --- | --- |
| 7 | Reserved. Assumed to be zero. |
| 6 – 3 | The number of images within the iconimation: 0000 2 images 1111 17 images |
| 2 | Iconimation reversing flag: 0 Play Iconimation normally 1 Once Iconimation has played normally, reverse the sequence |
| 1 | Reserved. Assumed to be zero. |
| 0 | <EXT INFO> presence flag: 0 <EXT INFO> not present 1 <EXT INFO> present |

### The <DELREP> byte always appears and is used as follows:

| Bits | Usage |
| --- | --- |
| 7 – 4 | Delay value. The value (in tenths of a second) that is requested between each frame: 0000 1 tenth (i.e. 0.1s) 1111 16 tenths (i.e. 1.6 s) |
| 3 – 0 | Repeat value. The requested number of repetitions of the iconimation: 0000 Unlimited repetition 0001 1 repetition 1111 15 repetitions |

### <EXT INFO>

The <EXT INFO> is presently unused.  It will be ignored.

<Image >

Animated GIF

As defined in the Outline specification appended

Text Formatting

As defined in 23.040

Text Positioning

Text Display Mode

Allows text to be  Scrolled  across the screen , flashing text to appear, text to be wrapped around objects

Embedded Text Stream

| Data Type/Description | Meta Language |
| --- | --- |
| Text Text is ASCII or UCS2 | <DATA>::= <TEXT HEADER> <POSITION>] <EXT INFO> [<WIDTH>]] <TEXT CHARACTERS> |

### The<TEXT HEADER> is 1 byte defined as follows:

| Bit(s) | Usage |
| --- | --- |
| 7 | Bold flag: 0 Bold off (default) 1 Bold on |
| 6 | Italic flag: 0 Italic off (default) 1 Italic on |
| 5 – 4 | Text size: 00 Medium / Normal (default) 01 Small 10 Large 11 Reserved |
| 3 – 2 | Justification: 00 Left Justify (default) 01 Right Justify 10 Centre 11 Reserved |
| 1 | <POSITION> flag: 0 <POSITION> does not appear (default) 1 <POSITION> appears |
| 0 | <EXT INFO> presence flag: 0 <EXT INFO> not present (default) 1 <EXT INFO> present |

### <POSITION>

A <POSITION> as previously defined defines the position where the top left part of the text will appear. If no <POSITION> details are specified then text will be displayed at the current cursor position.

### <EXT INFO>

The <EXT INFO> is 1 byte used as follows:

| Bit | Usage |
| --- | --- |
| 7 | <WIDTH> appears: 0 <WIDTH> does not appear (default) 1 <WIDTH> appears |
| 6 | Multiline Flag: 0 Multiline text (Text wrapping) (default) 1 Single line only (No text wrapping) |
| 5 | Indentation flag: 0 Position successive lines of text at the same horizontal position as the first line (default) 1 Position any successive lines of text at the leftmost position of the screen |
| 4 – 0 | Reserved. |

### <TEXT CHARACTERS>

<TEXT CHARACTERS> is simply a byte stream containing the text.