| 3GPP TSG-T2 #13 EMS Ad-Hoc London, UK 3rd - 5th April 2001 |  |  |  |  |  |  |  |  |  | T2E-010023 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR-Form-v3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23.040 |  |  |  | CR | CR-Num |  |  |  rev |  |  |  |  | - |  |  Current version: |  |  |  |  | 4.1.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  |  | X |  | Radio Access Network |  |  |  |  |  |  |  |  | Core Network |  |  |
| Title:  |  | Enhanced Text Format Parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Ericsson |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  | Messaging (SMS) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 26-March-2001 |  |  |  |  |  |
| Category:  |  | C |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | Rel 4 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | There is currently no way of specifying coloured text or coloured text backgrounds |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Addition of two new bytes of data to the existing ext Formatting IE allowing the specification of the text colour and the text background colour for the current formatting run. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | No means of specifying either Text or Text background colours. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 3.10.1 & 9.2.3.24.10.1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at: http://www.3gpp.org/3G_Specs/CRs.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://www.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2000-09 contains the specifications resulting from the September 2000 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

### 3.10.1	Text formatting

- The following text formatting features are supported:

Alignment

-	Left

-	Centre

- Right

- Default (Language dependent)

Font size

-	Normal (default)

-	Large

-	Small

Style

-	Normal (default)

-	Bold

-	Italic

-	Underlined

-	Strikethrough

Text Colour

Text Background Colour

9.2.3.24.10.1.1	Text Formatting

- The Information-Element-Data octet(s) shall be coded as follows.

Octet 1	Start position of the text formatting. Set to the number of characters after the formatting shall be applied from the beginning of the SM data.

This octet shall be coded as an integer value in the range 0 (beginning of the SM data) to the maximum number of characters included in the SM data of one single SM or one segment of a concatenated SM.

Octet 2	Text formatting length. Gives the number of formatted characters or sets a default text formatting

This octet shall be coded as an integer value in the range 1 to the maximum number of characters for which the formatting applies in one single SM or one segment of a concatenated SM.

A text formatting length value of  0 indicates that the text format shall be used as a default text format. The default text format shall be used for all text in a concatenated SM unless temporarily over-ridden by a text formatting IE with a non-zero text format length field.

It shall be possible to re-define the default text formatting to be applied to all subsequent text by sending a new Text Format IE with text format length zero.

Conflicting overlapping text formatting instructions shall be resolved by applying the formatting instructions in their sequential order.

Octet 3	formatting mode value coded as following:

- Octet 3:	Bit 7	Bit 6	Bit 5	Bit 4	Bit 3	Bit 2	Bit 1	Bit 0

- Bit 1	Bit 0		*Alignment
0	0		Left
0	1		Center
1	0		Right
1	1		Language dependent (default)

- *in case formatting text is inserted on the same line as previous non formatting text or with a different mode value, the alignment value shall be set to the same value as the previous formatted predefined object.

- Alignment may affect object placement.

- Bit 3	Bit 2		Font Size
0	0		Normal (default)
0	1		Large
1	0		Small
1	1		reserved

- Bit 4			Style bold
1			Bold on
0			Bold off

- Bit 5			Style Italic
1			Italic on
0			Italic off

- Bit 6			Style Underlined
1			Underlined on
0			Underlined off

- Bit 7			Style Strikethrough
1			Strikethrough on
0			Strikethrough off

- If bit 4,5,6 and 7 are set to 0, it will mean normal style (default).

Octet 4		Text Colour

Bits 0..3  Define the Text Foreground Colour

Bits 4..7 Define the Text Background Colour

Each colour is defined in a nibble according to the table below. These values are suggested, actual colours displayed may vary between ME’s depending on the display device used.

The colour values defined are simple primary and secondary colours plus four levels of grey. Bright colours have a higher intensity than dark colours.

Nibble Value			Colour

(msb…lsb)

0000					Black

0001					Dark Grey

0010					Dark Red

0011					Dark Yellow

0100					Dark Green

0101					Dark Cyan

0110					Dark Blue

0111					Dark Magenta

1000					Grey

1001					White

1010					Bright Red

1011					Bright Yellow

1100					Bright Green

1101					Bright Cyan

1110					Bright Blue

1111					Bright Magenta