ETSI SMG4	TSGT2#2(99)211

London, 15-19 March 1999

| CHANGE REQUEST No : |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 03.57 |  | Version |  | 1.5.0 |  |  |  |
| Submitted to SMG |  |  | for approval |  | X | without presentation ("non-strategic") |  |  |  |  |  |
| list plenary meeting or STC here |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | x | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | MExE 03.57 |
| --- | --- |

| Source: | Nokia | Date: | 11.03.1999 |
| --- | --- | --- | --- |

| Subject: | System Applications |
| --- | --- |

| Category: | F Correction | X | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 | x |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | There is no reason to disallow the manufacturer from certain operations, given that the manufacturer's system applications must be able to do anything, and that the operator and user must trust the manufacturer. |
| --- | --- |

| Clauses affected: | 8.1 |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  | ® List of CRs: |  |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: | The "HTTP cache manipulation" row in the security table should be "browser cache manipulation" in order to cover WAP's WSP cache as well. It does not make sense to allow the user to specify which directories they will allow applications access to. Typically users will not know which directories are safe and which are not. The Manufacturer must specify which directories are to be restricted. |
| --- | --- |

# 8	Security

Editor‘s Note: input awaited from SMG10

## 8.1		Generic security

In order to manage the MExE and prevent attack from unfriendly sources or transferred applications unintentionally damaging the MExE device a security system is required. This section defines the MExE security architecture.

The basis of MExE security is a framework of permissions which defines the permissions and privileges transferred content, applets or applications have within the MExE. The MExE framework is defined in TS GSM 02.57 and is as follows (there is no implied hierarchy):

MExE Security Level 1 (used by the HPLMN operator);

MExE Security Level 2 (system applications, applets and content by MS manufacturer);

MExE Security Level 3 (user trusted applications, applets and content);

MExE Security Level 4 (user untrusted applications, applets and content);

8.2		Application Permissions

The following table specifies the permissions of each security level.

User Permission is required for all actions by level 1 & 3 applications, applets and content.

|  | MExE Security Levels |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Actions |  | 1 Network Operator | 2 System Applications by MS Manufacturer |  | 3 User Trusted Applications | 4 User Untrusted Applications |
| - Read file - Write file - Get file information - Delete file | Yes 2 |  | Yes | Yes 2 | Yes 3, 4 |
| Load native code libraries | No | Yes |  | No |  |
| - Initiate a voice/data connection 5 - Accept a voice/data connection 5 - Intercept an incoming voice/data connection 5 | Yes |  |  |  | No |
| Call forward | Yes | Yes |  | Yes 6 | No |
| - Terminate an existing connection - Hold an existing connection - Resume an existing connection | Yes |  |  |  | No |
| Modification of user invoked connection | Yes |  |  |  | No |
| - Add a phonebook entry - Delete a phonebook entry - Get a phonebook entry - Modify a phonebook entry | Yes |  |  |  | No |
| Send an SMS | Yes |  |  |  | No |
| Add an application or applet | Yes |  |  |  | No |
| Delete a saved application or applet | Yes |  |  | Yes 7 | No |
| - Add a certificate for a given level - Delete a certificate for a given level - Replace a certificate for a given level | Yes 8 |  |  |  | No |
| Audio access (e.g. wake-up service) | Yes |  |  |  | No |
| Browser Cache manipulation | Yes |  |  |  | No |
| Get IMEI | Yes |  |  |  | No |
| Location Information | Yes | Yes |  | Yes | No |
| - Activate a user profile - Modify a user profile | Yes |  |  |  | No |
| Legend for above footnotes:- 2 - Except in manufacturer pre-specified directories 3 - Only in application/applet’s own directory 4 - Restriction may be required on space allocated to write 5 - A network connection may be via any supported bearer service 6 - To pre-defined numbers in the user profile 7 - Only in manufacturer pre-specified directory 8 - Only the organisation whose public key is certified (or the organisation that certified the public key) can add, delete or replace a particular certificate |  |  |  |  |  |

Table 3: Security levels and actions