ETSI SMG4	TSGT2#2(99)251

London, 15-19 March 1999

| CHANGE REQUEST No : |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 07.07 |  | Version |  | 7.2.0 |  |  |  |
| Submitted to SMG | #29 |  | for approval |  | X | without presentation ("non-strategic") |  |  |  | X |  |
| list plenary meeting or STC here |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | X | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | TEI |
| --- | --- |

| Source: | SMG4 | Date: | 15 March 1999 |
| --- | --- | --- | --- |

| Subject: | Additional result code for +CSSN |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature | X |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 | X |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | There is an omission in the CSSU: <code 2> codes of +CSSN. In the situation where a call is already in progress and a second incoming call is diverted resulting in a network notification, there will be no way to display the message. |
| --- | --- |

| Clauses affected: | 7.16 |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  | ® List of CRs: |  |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

## 7.16	Supplementary service notifications +CSSN

Table 1: +CSSN parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| +CSSN=[<n>[,<m>]] |  |
| +CSSN? | +CSSN: <n>,<m> |
| +CSSN=? | +CSSN: (list of supported <n>s),(list of supported <m>s) |

Description

This command refers to supplementary service related network initiated notifications. The set command enables/disables the presentation of notification result codes from TA to TE.

When <n>=1 and a supplementary service notification is received after a mobile originated call setup, intermediate result code +CSSI: <code1>[,<index>] is sent to TE before any other MO call setup result codes presented in this ETS or in V.25ter [14]. When several different <code1>s are received from the network, each of them shall have its own +CSSI result code.

When <m>=1 and a supplementary service notification is received during a mobile terminated call setup or during a call, or when a forward check supplementary service notification is received, unsolicited result code +CSSU: <code2>[,<index>[,<number>,<type>[,<subaddr>,<satype>]]] is sent to TE. In case of MT call setup, result code is sent after every +CLIP result code (refer command "Calling line identification presentation +CLIP") and when several different <code2>s are received from the network, each of them shall have its own +CSSU result code.

Test command returns values supported by the TA as a compound value.

Defined values

<n> (parameter sets/shows the +CSSI result code presentation status in the TA):

0	disable

1	enable

<m> (parameter sets/shows the +CSSU result code presentation status in the TA):

0	disable

1	enable

<code1> (it is manufacturer specific, which of these codes are supported):

0	unconditional call forwarding is active

1	some of the conditional call forwardings are active

2	call has been forwarded

3	call is waiting

4	this is a CUG call (also <index> present)

5	outgoing calls are barred

6	incoming calls are barred

7	CLIR suppression rejected

8	call has been deflected

<index>: refer "Closed user group +CCUG"

<code2> (it is manufacturer specific, which of these codes are supported):

0	this is a forwarded call (MT call setup)

1	this is a CUG call (also <index> present) (MT call setup)

2	call has been put on hold (during a voice call)

3	call has been retrieved (during a voice call)

4	multiparty call entered (during a voice call)

5	call on hold has been released (this is not a SS notification) (during a voice call)

6	forward check SS message received (can be received whenever)

7	call is being connected (alerting) with the remote party in alerting state in explicit call transfer operation (during a voice call)

8	call has been connected with the other remote party in explicit call transfer operation (also number and subaddress parameters may be present) (during a voice call or MT call setup)

9	this is a deflected call (MT call setup)

10    Additional incoming call forwarded

<number>: string type phone number of format specified by <type>

<type>: type of address octet in integer format (refer GSM 04.08 [8] subclause 10.5.4.7)

<subaddr>: string type subaddress of format specified by <satype>

<satype>: type of subaddress octet in integer format (refer GSM 04.08 [8] subclause 10.5.4.8)

Implementation

Optional.