SMG4/TSG-T2							TSGT2#2(99)205

London, UK

15th - 19th March 1999

Source: Vodafone

SMSC address checking in Status reports

A problem has been identified concerning the inability  for  Status Reports being received by certain MS’s to be processed correctly in the MS.

A change was recently introduced by a mobile phone manufacturer into some of their models which required the MS to check that the SMSC address contained in a status report was the same SMSC address to which the mobile originated SM was sent. Clearly, the actual physical SMSC must be the same but this does not necessarily require the SMSC address’s to be the same.

GSM 03.40 does not require the MS to check the SMSC address in the received status report and the implication is that it is unnecessary.

The problem became apparent when multiple SMSC’s were introduced into the PLMN for reasons of redundancy, capacity and load sharing. For a mobile originated SM, elements within the PLMN take responsibility for routing the SM to a particular SMSC so that the SMSC address used by the MS is always the same irrespective of which SMSC the SM finally gets submitted to.

When the SMSC wishes to send a status report to that MS, the address of the SMSC sent in the status report will not be the same as the one used by the MS  because the PLMN and the HLR require them to be different for each SMSC. The reason for this is that in the case where the delivery of the SM to the destination MS fails due to say Absent Subscriber the HLR Message Waiting Data must contain the address of the actual SMSC which tried to send the SM so that the correct SMSC can be alerted when the MS becomes available.

For any network operator who wishes to introduce multiple SMSC’s they are likely to encounter the same problem. Unfortunately, the mobile phone manufacturer has indicated their intention to continue to use the SMSC address checking mechanism in their current product software upgrades and their future products. The reason given by them for having to check that the SMSC addresses match is to allow the MS to send an SM with the same TP-MR to a different SMSC when the first SM using the same TP-MR value is still undelivered and therefore currently stored in a different SMSC.

The introduction of this feature has resulted in a considerable number of  complaints from mobile subscribers and in the case of corporate organisations, their applications have now ceased to function correctly.

In order to avoid a proliferation of this mechanism it may be necessary to amend 03.40 to say that the SMSC addresses in a Status report must not be checked in this way.

SMG4 are invited to discuss this problem.

SMG4 delegates are also urged to bring any aspect which could be argued as non compliant with GSM specifications to the ETSI GSM STC’s before they embark on an implementation into their products/ services.