| 3GPP TSG-T2#6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  |  |  |  | TSG-T2-99840 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Kyongju Korea. 4th – 6th October 99 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 3G 23.038 |  |  |  |  |  | CR |  |  |  |  |  |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  | 3.1.0 |  |  |  |  |
| Gsfor these languagesM (AA.BB) or 3G (AA.BBB) specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by MCC support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submision to: |  |  |  | T#5 |  |  |  |  | for approval |  |  |  |  |  |  | X |  |  |  |  | Strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | (for SMG |  |  |  |
| list approval meeting # here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | Use only) |  |  |  |
| Form: CR cover sheet, version 1.1 for 3GPP and SMG The latest version of this form is available from: ftp://ftp.3gpp.org/Information/3GCRF-11.rtf |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | (U)SIM |  |  |  |  | ME |  |  |  |  |  | X |  | UTRAN |  |  |  |  |  |  |  |  |  | Core Network |  |  |  |  |  |  |  |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | Vodafone |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  | 29-9-99 |  |  |  |  |  |
| Subject: |  |  | Language codes for Hebrew,Arabic and Russian |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item: |  |  | Technical improvements and enhancements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  |  |  |  | Phase 2 |  |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 99 |  |  |  | X |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (releases phase2, 96, 97 and 98 apply only to GSM specifications) |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change: |  |  | Partner Communications Company Israel network operator requires code points for Hebrew, Arabic and Russian languages to be added to 03.38 in order that their SIM LP file is able to work correctly for these languages. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  | Section 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other GSM core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  | The opportunity has been taken to carry out some minor editorial corrections to align entries in the table |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

.

# 5	Cell Broadcast Data Coding Scheme

The Cell Broadcast Data Coding Scheme indicates the intended handling of the message at the MS, the alphabet/coding, and the language (when applicable). Any reserved codings shall be assumed to be the GSM default alphabet (the same as codepoint 00001111) by a receiving entity. The octet is used according to a coding group which is indicated in bits 7..4. The octet is then coded as follows:

| Coding Group Bits 7..4 | Use of bits 3..0 |
| --- | --- |
| 0000 | Language using the default alphabet |
|  | Bits 3..0 indicate the language: |
|  | 0000 German |
|  | 0001 English |
|  | 0010 Italian |
|  | 0011 French |
|  | 0100 Spanish |
|  | 0101 Dutch |
|  | 0110 Swedish |
|  | 0111 Danish |
|  | 1000 Portuguese |
|  | 1001 Finnish |
|  | 1010 Norwegian |
|  | 1011 Greek |
|  | 1100 Turkish |
|  | 1101 Hungarian 1110 Polish |
|  | 1111 Language unspecified |
| 0001 | 0000 Default alphabet; message preceded by language indication. The first 3 characters of the message are a two-character representation of the language encoded according to ISO 639 [12], followed by a CR character. The CR character is then followed by 90 characters of text. A Pre-Phase 2+ MS will overwrite the start of the message up to the CR and present only the text. 0001 UCS2; message preceded by language indication The message starts with a two 7-bit default alphabet character representation of the language encoded according to ISO 639 [12]. This is padded to the octet boundary with two bits set to 0 and then followed by 40 characters of UCS2-encoded message. An MS not supporting UCS2 coding will present the two character language identifier followed by improperly interpreted user data. 0010..1111 Reserved for European languages |
| 0010.. | 0000 Czech 0001 Hebrew 0010 Arabic 0011 Russian.. 0100 - 1111 Reserved for other Languages , with unspecified handling at the MS |
| 0011 | 0000..1111 Reserved for other Languages , with unspecified handling at the MS |
| (continued) |  |

(concluded)

| 01xx | General Data Coding indication Bits 5..0 indicate the following: |
| --- | --- |
|  | Bit 5, if set to 0, indicates the text is uncompressed Bit 5, if set to 1, indicates the text is compressed using the GSM standard compressing algorithm. ( see GSM TS 03.42 ) |
|  | Bit 4, if set to 0, indicates that bits 1 to 0 are reserved and have no message class meaning Bit 4, if set to 1, indicates that bits 1 to 0 have a message class meaning: |
|  | Bit 1 Bit 0 Message Class: |
|  | 0 0 Class 0 |
|  | 0 1 Class 1 Default meaning: ME-specific. |
|  | 1 0 Class 2 SIM specific message. |
|  | 1 1 Class 3 Default meaning: TE-specific (see GSM TS 07.05 [8]) |
|  | Bits 3 and 2 indicate the alphabet being used, as follows: |
|  | Bit 3 Bit 2 Alphabet: |
|  | 0 0 Default alphabet |
|  | 0 1 8 bit data |
|  | 1 0 USC2 (16 bit) [10] |
|  | 1 1 Reserved |
| Coding Group Bits 7..4 | Use of bits 3..0 |
| 1000..1110 | Reserved coding groups |
| 1111 | Data coding / message handling |
|  | Bit 3 is reserved, set to 0. |
|  | Bit 2 Message coding: |
|  | 0 Default alphabet |
|  | 1 8 bit data |
|  | Bit 1 Bit 0 Message Class: |
|  | 0 0 No message class. |
|  | 0 1 Class 1 user defined. |
|  | 1 0 Class 2 user defined. |
|  | 1 1 Class 3 |
|  | default meaning: TE specific (see GSM TS 07.05 [8]) |