| 3GPP TSG-T3 (USIM) ad hoc meeting Paris, France, 1 - 3 December, 1999 | Tdoc T3z99047 |
| --- | --- |

# Call-related Toolkit TLVs

Source: Aspects

The following SIMPLE-TLVs used in USAT have been analysed for any changes needed in converting from GSM to 3G:

- Address

- CCP

- Called Party Subaddress

- Cause

- Transaction identifier

- BC Repeat Indicator

- DTMF

Transaction Identifier is defined in 3G TS 24.007, and is compatible with the definition in GSM. Therefore no change to the USAT TLV is required.

Address, CCP, Called Party Subaddress, Cause and BC Repeat Indicator are defined in 3G TS 24.008, and are all compatible with the definition in GSM. Likewise, no changes are required to the USAT TLVs.

- The definition of DTMF in Toolkit references the ADN coding in GSM 11.11 for digits and the pause character. We therefore need to ensure that the corresponding coding in 3G 31.102 is referenced. No material changes have been made to DTMF in the signalling specification (3G TS 24.008), so the coding should still be compatible.