| 3GPP TSG-T3 (USIM) ad hoc meeting #6 Madrid, Spain, 22 - 23 June, 2000 | Tdoc T3z000002 |
| --- | --- |

# Gemplus Proposition

# 1. Sim.Toolkit.ProactiveHandler

1.1. void Init(byte type, byte qualifier, byte dstDevice)

## 1.1.1. Normal Execution Test

1.1.1.1. Conformance requirement

The card is compliant to Java Card 2.1 VM, RE,& API and it has passed the SUN Java Card 2.1 test suite.

The SIM has to implement the API 03.19 as defined in the following technical specification :

GSM 03.19 version 8.x.x  Release 1999

Annex A (normative):Java Card SIM API / Class sim.toolkit.ProactiveHandler

1.1.1.2. Test Purpose

To verify that the method initialises correctly the next Proactive command with Command Details and Device Identities TLV. The source device is always the SIM card. The command number is generated by the method. The comprehension Required flag are set.

1.1.1.3. Prerequisites

The card is personalised with the standard profile.

### The applet corresponding to the test is loaded in the card.

### In the applet, this method will be called after getTheHandler( ) method.

1.1.1.4. Procedure

| Step | #### Action | #### Status | #### Comment |
| --- | --- | --- | --- |
| 1 | Reset the card |  |  |
| 2 | SELECT MF | 9F16h |  |
| 3 | VERIFY CHV1 | 9000h |  |
| 4 | TERMINAL PROFILE 1.1 | 9117h |  |
| 5 | FETCH SET UP MENU 1.1 | 9000h | Verify the received data |
| 6 | TERMINAL RESPONSE SET UP MENU 1.1 | 9000h |  |
| 7 | ENVELOPE MENU SELECTION 1.1 | 9109h |  |
| 8 | FETCH INIT 1.1 | 9000h | Verify the received data |
| 9 | SELECT EFtest | 9000h |  |
| 10 | READ BINARY EFtest | 9000h | Verify the applet status 9000h 0000h |

SELECT MF

Coding:

A0h A4h 00h 00h 02h

3Fh 00h

# VERIFY CHV1

Coding:

A0h 20h 00h 01h 08h

xxh xxh xxh xxh xxh xxh xxh xxh

TERMINAL PROFILE 1.1

Coding:

A0h 10h 00h 00h 09h

FFh FFh FFh FFh FFh FFh FFh FFh FFh

FETCH SET UP MENU 1.1

Logically:

Command Details TLV

Command Number:	01h

Command Type:	SetUpMenu

Command qualifier:	help information not available

Device Identities TLV

Source device:		SIM

Destination device:	ME

Alpha Identifier TLV

Alpha Id:		not present

Item TLV

Item Identifier:		01h

Item String		“applet1”

Coding:

A0h 12h 00h 00h 17h

[D0h 15h 81h 03h 01h 25h 00h

82h 02h 81h 82h

85h 00h

8Fh 08h 01h 61h 70h 70h 6Ch 65h 74h 31h]

TERMINAL RESPONSE SET UP MENU 1.1

Logically:

Command Details TLV

Command Number:	01h

Command Type:	SetUpMenu

Command qualifier:	help information not available

Device Identities TLV

Source device:		ME

Destination device:	SIM

Result TLV

General result:		Command perform successfully

Coding:

A0h 14h 00h 00h 0Ch

81h 03h 01h 25h 00h

82h 02h 82h 81h

83h 01h 00h

ENVELOPE MENU SELECTION 1.1

Coding:

A0h C2h 00h 00h 09h

D3h 07h 82h 02h 01h 81h 90h 01h 01h

FETCH INIT 1.1

Logically:

Command Details TLV

Command Number:	01h

Command Type:	FFh (test value)

Command qualifier:	FEh (test value)

Device Identities TLV

Source device:		SIM

Destination device:	FDh (test value)

Coding:

A0h 12h 00h 00h 0Bh

[D0h 09h 81h 03h 01h FFh FEh

82h 02h 81h FDh]

SELECT EFtest

Coding:

A0h A4h 00h 00h 02h

FFh FBh

READ BINARY EFtest

Coding:

A0h B0h 00h 00h 04h

[90h 00h 00h 00h]