3GPP TSG-CT Meeting #91e	CP-210186

# E-Meeting, 18th – 24th March 2021

# Source:	3GPP TSG CT WG3

# Title:	New WID on N7 Interfaces Enhancements to Support GERAN and UTRAN

# Document for:	Approval

Agenda Item:	17.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	N7 Interfaces Enhancements to Support GERAN and UTRAN

# Acronym: TEI17_NIESGU-CT

## Unique identifier: 	 910011

## Potential target Release: Rel-17

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a Building Block.

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| TEI17_NIESGU | SA2 | 870009 | N7/N40 Interfaces Enhancements to Support GERAN and UTRAN |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

## 3	Justification

To simplify the network architecture and network management when 5GS is deployed and 2G or 3G still exists in their network, operators would like to be able to deploy a combined node serving as both SMF+PGW-C and PGW-C, to simultaneously serve 2G/3G/4G/5G users.

To simplify the network deployments and reduce CAPEX and OPEX for operators, it is beneficial to enable the use of the N7 interface for policy control in 2/3/4/5G accesses.

This will beneficially allow PCF to handle UEs using 2G/3G and therefore separate PCRF is not required. This will also allow the PCF to be used to handle UEs roaming onto 2G/3G access in a different PLMN.

## 4	Objective

The objective is to study and then extend the interface on N7 reference point to support 2G/3G.

The expected work will include analysis and the related normative work on:

a)	Support of RAT Type for 2G/3G on interface extension for N7 reference point;

b)	Support of location change report of RAI, and SGSN level for 2G/3G on interface extension for N7 reference point.

NOTE:	Potential impact on CT4 related selection of PGW supporting N7 and create of PDU session ID will be further studied.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 29.512 | Extensions of N7 interface to support the feature | CT#93 (Sep. 2021) | CT3 responsibility |
| 29.513 | Potential impact on PCC signalling flows for supporting the feature. | CT#93 (Sep. 2021) | CT3 responsibility |

NOTE: 	Potential impact on TS 29.508 to support location change reporting of RAI provide by PGW supporting N7 will be further studied.

## 6	Work item Rapporteur(s)

Huang Zhenning (China Mobile)

huangzhenning (at) chinamobile (dot) com

## 7	Work item leadership

CT3

## 8	Aspects that involve other WGs

None.

## 9	Supporting Individual Members

Supporting IM name

China Mobile

Nokia

Nokia Shanghai Bell

Ericsson

CATT