3GPP TSG RAN Meeting #73	RP-161699

# New Orleans, September 19 - 22, 2016	revision of RP-152181

# Source:		Verizon, China Telecom, NTT DOCOMO Inc, ZTE, Alcatel-Lucen

# Title:	Revised WI proposal for Multicarrier Load Distribution of UEs in LTE

# Document for:	Approval

Agenda Item:	10.2.5

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Comprehensive instructions can be found at http://www.3gpp.org/Work-Items

Title:	Multicarrier Load Distribution of UEs in LTE

Acronym:	LTE_MC_Load

Unique identifier:	670056

NOTE:	If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI. Please tick (X) the applicable box(es) in the table below:

| This WID includes a Core part | x |
| --- | --- |
| This WID includes a Performance part | x |

1	3GPP Work Area

| x | Radio Access |
| --- | --- |
|  | Core Network |
|  | Services |

2	Classification of WI and linked work items

2.0	Primary classification

This work item is a …

|  | Study Item (go to 2.1) |
| --- | --- |
|  | Feature (go to 2.2) |
| x | Building Block (go to 2.3) |
|  | Work Task (go to 2.4) |

NOTE:	Core, Performance and Testing parts of RAN WIs are usually Building Blocks.
If you are in doubt, please contact MCC.

2.1	Study Item

| Related Work Item(s) (if any] |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Go to §3.

2.2	Feature

| Related Study Item or Feature (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Go to §3.

2.3	Building Block

| Parent Feature (or Study Item) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

This work item is …

|  | Stage 1 (go to 2.3.1) |
| --- | --- |
|  | Stage 2 (go to 2.3.2) |
| x | Stage 3 (go to 2.3.3) |
|  | Test spec (go to 2.3.4) |
|  | Other (go to 2.3.5) |

2.3.1	Stage 1

| Source of external requirements (if any) |  |  |
| --- | --- | --- |
| Organization | Document | Remarks |

Go to §3.

2.3.2	Stage 2

| Corresponding stage 1 work item |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Other source of stage 1 information |  |  |
| --- | --- | --- |
| TS or CR(s) | Clause | Remarks |

If no identified source of stage 1 information, justify:

Go to §3.

2.3.3	Stage 3

| Corresponding stage 2 work item (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Else, corresponding stage 1 work item |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Other justification |  |  |
| --- | --- | --- |
| TS or CR(s) or external document | Clause | Remarks |

If no identified source of stage 2 information, justify:

Go to §3.

2.3.4	Test spec

| Related Work Item(s) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

Go to §3.

2.3.5	Other

| Related Work Item(s) |  |  |  |
| --- | --- | --- | --- |
| Unique ID | Title | Nature of relationship | TS / TR |

Go to §3.

2.4	Work task

| Parent Building Block |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

3	Justification

Deploying multiple carriers is one of the most common solutions to address the ever increasing capacity needed in cellular networks, especially at traffic hotspots. This requires a balanced load among the multiple LTE carriers for efficient operation and optimal resources utilization. Load balancing across multiple carriers should consider a variety of deployment scenarios arising due to different capacities and the different numbers of the carriers available in a given area, especially when non-contiguous spectrum with multi-carriers of different bandwidths of different bands is involved, resulting in different number of carriers with different capacities in different areas.

For idle mode, it is observed that there is a strong correlation between the idle mode UE density and the active UE traffic load in a EUTRAN cell. Hence, control of the idle mode UE distribution is an essential element for traffic load balancing. However, it is difficult to conduct idle-mode load balancing by the current specified mechanisms such as adjusting the reselection measurement threshold, broadcast or dedicated priorities.  Furthermore, the deficiency in the current idle mode load balancing mechanisms has led to the partial reliance on redirection or handover (HO) after call establishment for load balancing. This has led to many more active redirections/HOs, increasing signaling load and HO failure rates. Additionally, even if load balance is achieved by HO and redirection, the situation will only last for a short period of time since UE will eventually follow idle mode cell reselection rules. The situation is worse in HetNet scenarios where the load in the different small cells at same frequency layers might be different leading to ping-pongs and uneven idle UEs distribution.

On the other hand, for connected mode, an ideal load balanced network should try to minimize active traffic overload probability while maximizing user throughput. However, current RSRQ based measurements as the HO and reselection metric may not be a good representation of the achievable throughput. Other measurements such as SINR may be more appropriate for load-balancing active traffic so as to achieve optimal throughput for the user, while simultaneously avoiding unnecessary HO or redirections.

In summary, load balancing should preferably be achieved already at RRC connection setup to minimize the need for load-triggered HO or redirection during connected mode, implying that pro-active control of the UE distribution in both idle and connected modes is of utmost importance. Additional efforts need to be made to addressing heterogeneous network scenarios.  Other measurement quantities could be considered for better representation of the achievable throughput.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objective of the WI is to look at solutions providing better distribution of idle UEs amongst multiple LTE carriers so as to minimize the need for load-triggered HO or redirection of UE during connected mode.  HetNet deployment scenarios should be considered.  Additional measurement quantities for evaluation of achievable cell throughput can be evaluated.

The WI should first have a study phase to look at:

- Limitations of the current mechanisms and measurement quantities for redistribution of UEs amongst multiple LTE carriers.

Based on the analysis of the study phase, the WI should provide solution(s) for:

- To redistribute RRC Idle UEs amongst LTE carriers that minimize the need for load triggered HO or redirection of UE during connected mode

- Carriers with different cell load, bandwidth and capabilities should be considered.

- Both homogeneous and heterogeneous deployment scenarios should be considered

- New measurement quantities, e.g. SINR, for better estimation of user throughput should be evaluated and introduced, if needed

The solutions should not adversely impact UE power consumption and should avoid increasing ping-pongs for high speed UEs between carriers.

Work plan of this WI:

Q2/2015:

RAN2#89bis and RAN2#90 meeting:

Study the limitations of the current mechanisms and measurement quantities, for redistribution of UEs amongst multiple LTE carriers

Provide input to RAN1/4 on the new measurement quantity.

Q3/2015:

RAN2#91:

Provide solution(s), based on the study phase, to redistribute RRC Idle UEs amongst LTE carriers that minimize the need for load triggered HO or redirection of UE during connected mode considering following aspects:

- Carriers with different cell load, bandwidth and capabilities

- Homogeneous and heterogeneous deployment scenarios

- New required measurement quantity as concluded in previous meetings, if any.

RAN4#76:

Based on input from RAN2, evaluate the details of core requirements for the new measurement quantity.

Q4/2015:

RAN2#91bis and RAN2#92:

Specify the concluded solution(s) and approve the corresponding CR(s).

RAN4#76bis:

Introduction of the core requirements for the new measurement quantity.

RAN1#83:

Capture the definition of the new measurement quantity in RAN1 specifications.

4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

Q4/2015:

RAN4#79:

Define the needed performance requirements for the new measurement quantity.

4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed leave the field empty otherwise enter a number in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each of them in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

5	Service Aspects

6	MMI-Aspects

7	Charging Aspects

8	Security Aspects

9	Impacts

| Affects: | UICC apps | ME | AN | CN | Others |
| --- | --- | --- | --- | --- | --- |
| Yes |  | x | x |  |  |
| No | x |  |  | x | x |
| Don't know |  |  |  |  |  |

10	Expected Output and Time scale

| New specifications [If Study Item, one TR is anticipated] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Spec No. | Title | 1st rsp. WG | 2nd rsp. WG(s) | Presented for information at plenary# | Approved at plenary # | Comments |

NOTE:	If this is a RAN WID including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Comments for each spec.
By default a new specs can only be new for one of both parts.

| Affected existing specifications [None in the case of Study Items] |  |  |  |  |
| --- | --- | --- | --- | --- |
| Spec No. | CR | Subject of the CR | Approved at plenary# | Comments |
| 36.300 |  | Overall description | RAN#70 | Core part |
| 36.133 |  | Requirements for support of radio resource management | RAN#70 | Core part |
| 36.214 |  | Physical layer – Measurements | RAN#70 | Core part |
| 36.304 |  | UE procedures in idle mode | RAN#70 | Core part |
| 36.331 |  | Radio Resource Control protocol specification | RAN#70 | Core part |
| 36.133 |  | Requirements for support of radio resource management | RAN#73 | Perf. part |

NOTE:	If this is a RAN WID including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Comments for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

11	Work item rapporteur(s)

DU, Zhongda

Company:	ZTE

Email: du.zhongda@zte.com.cn

12	Work item leadership

Primary: RAN WG2

Secondary: RAN WG4, RAN WG1

NOTE:	If this is a RAN WID including Core and Perf. part, then this WG specifies the WG leading the Core part.
RAN WG4 is by default leading the Perf. part.

13	Supporting Individual Members

Supporting IM name

Verizon

China Telecom

Alcatel Lucent

Alcatel Lucent Shanghai Bell

NTT DOCOMO, Inc.

CMCC

CHTTL

KDDI

HTC

MediaTek Inc.

Intel

T-Mobile USA

ZTE

CATT

III

US Cellular

ITRI

Acer

KT Corp.

Softbank Mobile

China Unicom

Motorola Mobility

CATR

Samsung

AsusTek

Shanghai Tejet Com Technology

NEC

Lenovo

LG Electronics

DISH Network

Telecom Italia

Potevio

Fujitsu

TELUS

Lightsquared

Kyocera

IAESI

Sequans

Qualcomm Incorporated

InterDigital

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"