3GPP TSG RAN Meeting #89e 	RP-201838

Electronic Meeting, September 14 - 18, 2020	(revision of RP-200902)

# Source:	Intel Corporation

# Title:	Revised SID: Study on supporting NR from 52.6GHz to 71 GHz

# Document for:	Approval

Agenda Item:	9.7.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Study on supporting NR from 52.6 GHz to 71 GHz

## Acronym: FS_NR_52_to_71GHz

## Unique identifier: 	#860037

Potential target Release: Rel-17

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  |  |  |
| No |  |  |  |  |  |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_NR_beyond_52GHz | RAN | 800090 | Study on NR beyond 52.6 GHz |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

## 3	Justification

In order to support wide range of services, 5G NR system aims to be flexible enough to meet the connectivity requirements of a range of existing and future (as yet unknown) services to be deployable in an efficient manner. In particular, NR considers supporting potential use of frequency range up to 100 GHz (ref. TR 38.913).

NR specifications that have been developed in Rel-15 and Rel-16 define operation for frequencies up to 52.6GHz, where all physical layer channels, signals, procedures, and protocols are designed to be optimized for uses under 52.6GHz.

However, frequencies above 52.6GHz are faced with more difficult challenges, such as higher phase noise, larger propagation loss due to high atmospheric absorption, lower power amplifier efficiency, and strong power spectral density regulatory requirements in unlicensed bands, compared to lower frequency bands. Additionally, the frequency ranges above 52.6 GHz potentially contain larger spectrum allocations and larger bandwidths that are not available for bands lower than 52.6 GHz.

As an initial effort to enable and optimize 3GPP NR system for operation in above 52.6GHz, 3GPP RAN has studied requirements for NR beyond 52.6GHz up to 114.25GHz including global spectrum availability and regulatory requirements (including channelization and licensing regimes), potential use cases and deployment scenarios, and NR system design requirements and considerations on top of regulatory requirements (ref. TR 38.807).  The potential use cases identified in the study include high data rate eMBB, mobile data offloading, short range high-data rate D2D communications, broadband distribution networks, integrated access backhaul (IAB), factory automation, industrial IoT (IIoT), wireless display transfer, augmented reality (AR)/virtual reality (VR) wearables, intelligent transport systems (ITS) and V2X, data center inter-rack connectivity, smart grid automation, private networks, and support of high positioning accuracy. The use cases span over several deployment scenarios identified in the study. The deployment scenarios include, but not limited to, indoor hotspot, dense urban, urban micro, urban macro, rural, factor hall, and indoor D2D scenarios. The study also identified several system design requirements around waveform, MIMO operation, device power consumption, channelization, bandwidth, range, availability, connectivity, spectrum regime considerations, and others.

Among the frequencies of interest, frequencies between 52.6 GHz and 71 GHz are especially interesting relatively in the short term because of their proximity to sub-52.6GHz for which the current NR system is optimized and the imminent commercial opportunities for high data rate communications, e.g., unlicensed spectrum but also licensed spectrum between 57GHz and 71GHz. Therefore, it would be beneficial to make a study focused on feasibility of using existing waveforms and required changes for frequencies between 52.6 GHz and 71 GHz, so as to take advantage of imminent commercial opportunities for the specific frequency regime by minimizing the specification burden and maximizing the leverage of FR2 based implementations.

## 4	Objective

This study item will include the following objectives:,

- Study of required changes to NR using existing DL/UL NR waveform to support operation between 52.6 GHz and 71 GHz

  - Study of applicable numerology including subcarrier spacing, channel BW (including maximum BW), and their impact to FR2 physical layer design to support system functionality considering practical RF impairments [RAN1, RAN4].

  - Identify potential critical problems to physical signal/channels, if any [RAN1].

- Study of channel access mechanism, considering potential interference to/from other nodes, assuming beam based operation, in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6 GHz and 71 GHz [RAN1].

  - Note: It is clarified that potential interference impact, if identified, may require interference mitigation solutions as part of channel access mechanism.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 38.808 | Study on supporting NR from 52.6 GHz to 71 GHz | 90-e | 91-e | Lee, Daewon, Intel Corp., daewon.lee@intel.com |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

## 6	Work item Rapporteur(s)

Lee, Daewon, Intel Corp., daewon.lee@intel.com (RAN1)

Coan, Phil, Qualcomm Inc., pcoan@qti.qualcomm.com (RAN4)

## 7	Work item leadership

RAN1

Secondary: RAN4

## 8	Aspects that involve other WGs

None identified yet

## 9	Supporting Individual Members

Supporting IM name

Apple

CATT

Charter Communications

Ericsson

ETRI

HiSilicon

Huawei

III

Intel Corp.

Interdigital

ITRI

Lenovo

LG Electronics

MediaTek

Motorola Mobility

NEC

Nokia

Nokia Shanghai Bell

OPPO

Qualcomm

Samsung

Sharp Corp.

vivo

WILUS

ZTE