3GPP TSG-CT WG1 Meeting #150	C1-244924

# Maastricht, Netherlands, 19-23 August 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.548 | CR | 0062 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SNRM-S sending SIP based MBS session announcement procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL_Ph3 |  |  |  |  |  | Date: |  |  | 2024-07-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To define SIP based MBS Session announcement and deannouncement procedure sent by SNRM-S |  |  |  |  |  |  |  |  |
| Summary of change: |  | SIP based MBS Session announcement and deannouncement procedures initiated by SNRM-S are defined re-using the SEAL MBS XML Info generation specified in clause 7.4.7. Backward compatibility analysis: This CR does not introduce any backward compatible issue. To address the editors note, the CR proposes new SIP procedures for sending the SIP based MBS Session announcement and deannouncement by SNRM-S, reusing the exsiting MBS XML information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedures to send the SIP based MBS Session announcement and deannouncement by SNRM-S will be missing in Stage-3 3GPP TS 24.548. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.10.2, 6.2.3.10.2.X(new), 6.2.3.10.2.Y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.2.3.10.2	SNRM server SIP and HTTP procedures

* * * Next Change * * * *

6.2.3.10.2.X	SIP based MBS session announcement procedure

If the VAL service supports SIP, the SNRM-S shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [6] and IETF RFC 3428 [17]. In the SIP MESSAGE request, the SNRM-S shall:

a)	set the Request-URI to the URI received in the To header field in a third-party SIP REGISTER request;

b)	include a Accept-Contact header field with the g.3gpp.icsi-ref media-feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.seal" along with parameters "require" and "explicit" according to IETF RFC 3841 [18];

c)	include a P-Asserted-Service header field with the value "urn:urn-7:3gpp-service.ims.icsi.seal";

d)	include the MBS public service identity of the SNRM-S in the P-Asserted-Identity header field;

e)	include a MIME body in the SIP MESSAGE request, with the MIME Content-Type header field set to "application/vnd.3gpp.seal-mbs-usage-info+xml"and MIME payload with the MBS session announcement XML generated as specified in clause 6.2.3.10.6;

f)	include a MIME body in the SIP MESSAGE request, with the MIME Content-Type header field set to "application/vnd.3gpp.seal-info+xml" and MIME payload with SEAL info XML as specified in clause 7.4.2 where the <seal-request-uri> element:

1)	shall include <VAL-user-id> element set to the VAL user ID of the user; and

2)	may include <VAL-group-id> element set to the VAL group identity that is served by this MBS session; and

g)	send the SIP MESSAGE request towards the SNRM-C according to 3GPP TS 24.229 [6].

NOTE 1:	The MBS session announcement procedure is used by the SNRM-S for announcement of both the pre-defined and on demand MBS session to the SNRM-C.

NOTE 2:	The MBS session announcement procedure can be used by the SNRM-S to share the associated information between a specific group communication and MBS session to the SNRM-C, rather than defining the MapGroupToSessionStream as specified in clause 14.3.4A.6.1 of 3GPP TS 24.434 [2].

* * * Next Change * * * *

6.2.3.10.2.Y	SIP based MBS session de-announcement procedure

To share the MBS session de-announcement with the SNRM-C, the SNRM-S shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [6] and IETF RFC 3428 [17], the SNRM-S shall:

a)	set the Request-URI to the URI received in the To header field in a third-party SIP REGISTER request;

b)	include an Accept-Contact header field with the g.3gpp.icsi-ref media-feature tag with the value of "urn:urn-7:3gpp-service.ims.icsi.seal" along with parameters "require" and "explicit" according to IETF RFC 3841 [18];

c)	include a P-Asserted-Service header field with the value "urn:urn-7:3gpp-service.ims.icsi.seal";

d)	include the MBS public service identity of the SNRM-S in the P-Asserted-Identity header field;

e)	include the mbs-usage-info XML payload in the SIP MESSAGE body carrying the MBS session de-announcement XML generated as below;

1)	an "application/vnd.3gpp.seal-mbs-usage-info+xml" with root element as <seal-mbs-usage-info>; and shall

i)	include <version> sub-element set to "1"; and

ii)	include one or more <mbs-announcement> elements, with each <mbs-announcement> element shall include only the <mbs-session-id> element set to the MBS session ID that will be released; and

f)	send the SIP MESSAGE request towards the SNRM-C according to 3GPP TS 24.229 [6].

* * * End of Changes * * * *